3GPP TSG-RAN WG4 Meeting#115		R4-2506229

Malta, MT, May 19th -23rd, 2025

Agenda item:	7.26.2

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on system parameters for ambient IoT

Document for:	Discussion

Introduction

In RAN4#114bis meeting, a WF [1] was agreed. This contribution continues to discuss the remaining open issues as follows.

- Topic 2-1: R2D bandwidth Issue 2-1-1: R2D transmission bandwidth Agreement: - For RAN4 requirements, only define R2D transmission bandwidth configuration as 1PRB, 2PRB, 3PRB, 4PRB with 15KHz SCS. Issue 2-1-2: R2D channel bandwidth Agreement: - Define R2D channel bandwidth with assumption of 90% spectrum utilization. - Topic 2-2: D2R bandwidth Issue 2-2-1: D2R bandwidth Agreement: - Define D2R channel bandwidth for A-IoT device: - Based on 2SB transmission, - Consider SFO impact - Consider small frequency shift defined in RAN1 - D2R transmission bandwidth (2 times of 1SB), consider bit rate defined in RAN1 - Topic 2-3: Channel raster Issue 2-3-1: R2D channel raster Agreement: - Consider following options for R2D channel raster - Option 1: Define 10KHz as R2D channel raster (i.e. same as enhanced channel raster) - Option 2: Define channel raster considering the RB orthogonality between macro BS and A-IoT BS - Option 3: Define 100KHz as R2D channel raster - Other options are not precluded.

Discussion

Values of R2D bandwidth

According to the last meeting agreements, the transmission bandwidth configuration is 1PRB, 2PRB, 3PRB, 4PRB with 15 kHz SCS and define R2D channel bandwidth with the assumption of 90% spectrum utilization. Therefore, we can define R2D channel bandwidth: 200 kHz, 400 kHz, 600 kHz and 800 kHz with 15 kHz SCS.

Proposal 1: Define R2D channel bandwidth: 200 kHz, 400 kHz, 600 kHz and 800 kHz with 15 kHz SCS.

R2D channel raster

The transmission bandwidth configuration of R2D is 1PRB, 2PRB, 3PRB and 4PRB with 15 kHz SCS. The channel raster to resource element mapping as follow. When they

|  | NRBmod2 = 0 | NRBmod2 = 1 |
| --- | --- | --- |
| Resource element index | 0 | 6 |
| Physical resource block number |  |  |

When the transmission bandwidth configuration of R2D is odd PRB. If we assume the channel raster is 100 kHz, then we can calculate the offset by this formula: m*180 kHz + 90 kHz+7.5kHz=n*100 kHz+ offset. We can get the values of offset are - 2.5 kHz, 22.5 kHz, 42.5 kHz. When the transmission bandwidth configuration of R2D is even PRB, If we assume the channel raster is 100 kHz, then we can calculate the offset by this formula: m*180 kHz + +7.5kHz=n*100 kHz +offset. We can get the values of offset are - 12.5 kHz, -32.5k Hz, 7.5 kHz, 27.5 kHz, 42.5 kHz. Therefore, we can get the offset value are respectively -2.5kHz, -7.5kHz, -12.5kHz, -32.5KHz, 7.5kHz, 22.5kHz, 27.5kHz and 42.5kHz.

When the transmission bandwidth configuration of R2D is odd PRB. If we assume the channel raster is 10 kHz, then we can calculate the offset by this formula: m*180 kHz + 90 kHz+7.5kHz=n*10 kHz+ offset. We can get the values of offset are - 2.5 kHz, 7.5 kHz. When the transmission bandwidth configuration of R2D is even PRB, If we assume the channel raster is 100 kHz, then we can calculate the offset by this formula: m*180 kHz + 7.5kHz=n*10 kHz +offset. We can get the values of offset is 7.5 kHz. Therefore, we can get the offset value are respectively - 2.5 kHz and 7.5 kHz.

Based on the above analysis, the quantity of the offset value for 10 kHz channel raster is less and the implementation is simpler compared to 100 kHz channel raster. We propose to define the channel raster for R2D as 10 kHz.

Proposal 2: Define R2D channel raster as 10 kHz, and the offset can be - 2.5 kHz or 7.5 kHz.

D2R bandwidth

In the last meeting, an agreement was agreed to define D2R channel bandwidth for A-IoT device considering the following aspects: 2SB, SFO, small frequency shift in RAN1 and transmission bandwidth (2SB). According to RAN1’s agreement as follow.

Agreement For D2R transmission with small frequency shift +/- R/Tb Hz, where time duration Tb corresponding to a bit (that is after FEC if applied) and R = Tb/(2 × D2R chip length): - the transmitted 2R chips for bit 1 and bit 0 are [0 1 0 1 …] and [1 0 1 0 …], respectively, for OOK modulation - the transmitted 2R chips for bit 1 and bit 0 are [-1 +1 -1 +1 …] and [+1 -1 +1 -1 …], respectively, for BPSK modulation. - Note: The case of R=1 is equivalent to using a Manchester line code without repeating each Manchester codeword.

Agreement The potential values of D2R chip duration Tchip, bit duration Tb, and SFS factor R, are shown in the following table: - FFS further down-selections of Tchip, Tb, and R to be supported - Note: Detailed indication signaling of D2R scheduling information is discussed in AI 9.4.4. Tchip (μs). Tb (μs). 133.33; 66.67; 33.33; 16.67; 11.11; 8.33; 5.56; 4.17; 2.78; 2.08; 1.39; 1.04; 0.69; 0.52. 266.67; R=1; R=2; R=4; R=8; R=12; R=16; R=24; R=32; R=48; R=64; R=96; R=128; R=256. 133.33; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32; R=48; R=64; R=96; R=128. 66.67; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32; R=48; R=64. 33.33; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32. 22.22; R=1; R=2; R=4; R=8; R=16. 16.67; R=1; R=2; R=4; R=6; R=8; R=12; R=16. 11.11; R=1; R=2; R=4; R=8. 8.33; R=1; R=2; R=4; R=6; R=8. 5.56; R=1; R=2; R=4. 4.17; R=1; R=2; R=4. 2.78; R=1; R=2. 2.08; R=1; R=2. 1.39; R=1. 1.04; R=1. Note: For further down-selection regarding the bit duration, - bit duration 266.67μs, 133.33μs and 66.67μs to be kept, which provides better multiplexing capability in the frequency domain. - Down-select to 1 between bit duration 1.39μs and 1.04μs, which achieves the competitive peak data rate larger than 640 kbps. - The remaining bit durations will be further discussed..

Based on the above agreement and information, transmission bandwidth (2SB) is equal to 4/Tb, small frequency shift is +/- R/Tb Hz. For SFO, the guard band for SFO is 10 %*( small frequency shift plus 1/2 transmission bandwidth). We can calculate the channel bandwidth for D2R by this formula.

When R=1, Channel bandwidth=transmission bandwidth +2*0.1(transmission bandwidth).

When R>1, Channel bandwidth = 2*(R/Tb+1/2 transmission bandwidth)+0.1(R/Tb+1/2 transmission bandwidth)) =2*1.1(R/Tb+1/2transmission bandwidth) =2*1.1*(2+R)/Tb.

The possible channel bandwidth values for D2R are calculated in Table 1.

Table 1. The channel bandwidth of D2R

| Tb (μs) | R value | CBW(kHz) | Tb (μs) | R value | CBW(kHz) | Tb (μs) | R value | CBW(kHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 266.67 | 1 | 18 | 133.33 | 1 | 36 | 66.67 | 1 | 72 |
| 266.67 | 2 | 33 | 133.33 | 2 | 66 | 66.67 | 2 | 132 |
| 266.67 | 4 | 49.5 | 133.33 | 4 | 99 | 66.67 | 4 | 198 |
| 266.67 | 8 | 82.5 | 133.33 | 6 | 132 | 66.67 | 6 | 264 |
| 266.67 | 12 | 115.5 | 133.33 | 8 | 165 | 66.67 | 8 | 330 |
| 266.67 | 16 | 148.5 | 133.33 | 12 | 231 | 66.67 | 12 | 462 |
| 266.67 | 24 | 214.5 | 133.33 | 16 | 297 | 66.67 | 16 | 594 |
| 266.67 | 32 | 280.5 | 133.33 | 24 | 429 | 66.67 | 24 | 858 |
| 266.67 | 48 | 412.5 | 133.33 | 32 | 561 | 66.67 | 32 | 1121.9 |
| 266.67 | 64 | 544.5 | 133.33 | 48 | 825 | 66.67 | 48 | 1649.9 |
| 266.67 | 96 | 808.5 | 133.33 | 64 | 1089 | 66.67 | 64 | 2177.9 |
| 266.67 | 128 | 1072.5 | 133.33 | 96 | 1617 | 16.67 | 1 | 287.9 |
| 266.67 | 256 | 2128.5 | 133.33 | 128 | 2145.1 | 16.67 | 2 | 527.9 |
| 33.33 | 1 | 144 | 22.22 | 1 | 216 | 16.67 | 6 | 791.8 |
| 33.33 | 2 | 264 | 22.22 | 2 | 396 | 16.67 | 4 | 1055.8 |
| 33.33 | 4 | 396 | 22.22 | 4 | 594.1 | 16.67 | 8 | 1319.7 |
| 33.33 | 6 | 528.1 | 22.22 | 8 | 990.1 | 16.67 | 12 | 1847.6 |
| 33.33 | 8 | 660.1 | 22.22 | 16 | 1782.2 | 16.67 | 16 | 2375.5 |
| 33.33 | 12 | 924.1 | 11.11 | 1 | 432 | 8.33 | 1 | 576.2 |
| 33.33 | 16 | 1188.1 | 11.11 | 2 | 792.1 | 8.33 | 2 | 1056.4 |
| 33.33 | 24 | 1716.2 | 11.11 | 4 | 1188.1 | 8.33 | 4 | 1584.6 |
| 33.33 | 32 | 2244.2 | 11.11 | 8 | 1980.2 | 8.33 | 6 | 2112.8 |
| 5.56 | 1 | 863.3 | 4.17 | 1 | 1151.1 | 8.33 | 8 | 2641.1 |
| 5.56 | 2 | 1582.7 | 4.17 | 2 | 2110.3 | 2.78 | 1 | 1726.6 |
| 5.56 | 4 | 2374.1 | 4.17 | 4 | 3165.5 | 2.78 | 2 | 3165.5 |
| 2.08 | 1 | 2307.7 | 1.39 | 1 | 3453.2 |  |  |  |
| 2.08 | 2 | 4230.8 | 1.04 | 1 | 4615.4 |  |  |  |

It is understood that RAN1 will down-select regarding the bit duration (Tb) for further. Even so, there are still so many kinds of CBW for D2R. From testing perspective or other RF requirements, we need to reduce the amount of CBWs. We can take several typical values from small to large for RF requirements.

Proposal 3: To define all kinds of CBW for D2R based on RAN1’s conclusion about Tb and R. We can take several typical values from small to large for RF requirements.

Conclusions

In this paper, we provide our views on system parameters for ambient IoT, we have made the following proposals.

Proposal 1: Define R2D channel bandwidth: 200 kHz, 400 kHz, 600 kHz and 800 kHz with 15 kHz SCS.

Proposal 2: Define R2D channel raster as 10 kHz, and the offset can be - 2.5 kHz or 7.5 kHz.

Proposal 3: To define all kinds of CBW for D2R based on RAN1’s conclusion about Tb and R. We can take several typical values from small to large for RF requirements.

Reference

- RP-2505230, WF on requirements for system parameters and A-IoT device, CMCC, TSG RAN4 meeting #114bis