# 3GPP TSG-RAN WG4 Meeting # 115		  R4-2506232

# Malta, MT, 19th – 23rd May, 2025

# Source:	ZTE Corporation, Sanechips

# Title:	Discussion on UE RF requirements for supporting new SDO band for LTE-based 5G Broadcast

# Agenda Item:	7.9.4

# Document for:	Approval

- Introduction

In RAN#107 meeting, a new Work Item [1] was approved on introduction of NTN broadcast band with the following RAN4 objective. In this contribution, we want to share some further views about NTN broadcast band from UE RF perspective.

Specify a new SDO band for LTE-based 5G Broadcast operating in 1467 to 1492 MHz in Region 3 for GSO: - Specify related UE RF core requirements for 10MHz PMCH - UE requirements specified in TS 36.101 for LTE-based 5G Broadcast shall be considered as baseline (reuse assumption of 10 MHz UE Rx filter) - Specify related SAN RF core requirements for 10 MHz PMCH considering also applicable regulatory requirements: - Take existing SAN RF requirements specified in TS 38.108 for 10MHz as baseline - Requirements based on Resolution 761 in WRC-19 - Only consider QPSK and 16QAM modulation Note: UE only needs to support 5G broadcast for this band Note: Release independence applicability will be further discussed

- Discussion

2.1 Out-of-band blocking

Issue 3-1-5: Out-of-band blocking - Agreement: - Add a new chapter 7.6C.3 for out-of-band blocking - To specify out-of-band blocking for 10MHz PMCH for LTE based 5G broadcast utilizing satellite in TS 36.102 as following tables. Table 7.6C.3-1: Out-of-band blocking parameters Rx Parameter; Units; PMCH bandwidth. 10 MHz. Power in Transmission Bandwidth Configuration; dBm; REFSENS + 6. NOTE: Reference measurement channel is specified in Annex A.1.3.. Table 7.6C.3-2: Out of band blocking Operating band; 1; Units; Frequency. Range 1; Range 2; Range 3. FInterferer (CW); MHz; FDL_low -40 to FDL_low -85; FDL_low -85 to FDL_low -110; FDL_low -110 to 1 MHz. FDL_high +30 to FDL_high + 75; FDL_high +75 to FDL_high +100; FDL_high +100 to +12750 MHz. 246; PInterferer; dBm; -44; -30; -15.

Regarding UE out-of-band blocking for the new NTN broadcast band, the agreement is to specify it based on RF filter sharing with LTE band 50. And the agreement in last meeting is shown in the above.While the frequency range of OOBB in spec is defined as 15 MHz below or above the UE receive band. So the start point of frequency range 1 should be FDL_low -15 or FDL_high +15. In addition, considering the offset of low and high frequency edge between band 50 and 246, OOBB requirements should be extended with 35MHz offset in the low frequency and 25MHz offset in the high frequency. The revised requirement is shown as below:

Table 7.6C.3-2: Out of band blocking

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pw | dBm | REFSENS + 6 dB |  |  |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 246 | Finterferer (CW) | MHz | -95 < f – FDL_low < -15 or 15 < f – FDL_high < 85 | -95 < f – FDL_low ≤ -120 or 85 ≤ f – FDL_high < 110 | 1 ≤ f ≤ FDL_low – 120 or FDL_high + 110 ≤ f ≤ 12750 |

A picture is provided to clarify the revised requirement more clearly in figure 1:

Figure 1 illustration of the frequency range of band 246 for OOBB requirement

Proposal 1: To revise the OOBB requirement as follows:

Table 7.6C.3-2: Out of band blocking

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pw | dBm | REFSENS + 6 dB |  |  |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 246 | Finterferer (CW) | MHz | -95 < f – FDL_low < -15 or 15 < f – FDL_high < 85 | -95 < f – FDL_low ≤ -120 or 85 ≤ f – FDL_high < 110 | 1 ≤ f ≤ FDL_low – 120 or FDL_high + 110 ≤ f ≤ 12750 |

2.2 Reference measurement channels

Issue 3-1-10: Reference measurement channels - Agreement: - To specify the RMC of 15kHz SCS for 10MHz PMCH in TS 36.102 as following tables as the baseline. FFS on other SCSs Table A.1.3.1-1 Fixed Reference Channel for PMCH Receiver Requirements (15 kHz SCS) Parameter; PMCH (15 kHz SCS). Unit; Value. Reference channel; R.PMCH.1; R.PMCH.2. PMCH bandwidth; MHz; 10; 10. Allocated resource blocks; 50; 50. Allocated subframes per Radio Frame(Note1); 10; 10. Modulation; QPSK; 16QAM. Target Coding Rate; 1/3; 3/4. Information Bit Payload (Note 2). For Sub-Frames 0,1,2,3,4,5,6,7,8,9; Bits; TBD; TBD. Number of Code Blocks per Sub-Frame (Note 3); 1; 3. Binary Channel Bits Per Subframe. For Sub-Frames 0,1,2,3,4,5,6,7,8,9; Bits; TBD; TBD. MBMS UE Category; ≥ 1; ≥ 2. Note 1: 2 OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit)..

It was agreed to specify the RMC of 15kHz SCS for 10MHz PMCH in TS 36.102, and FFS other SCS. A terminal which supports MBMS can use five SCSs (15k/7.5k/2.5k/1.25k/0.37k) optionally for LTE PMCH transmission in current RAN4 spec, and in the Rel-18 RAN4 WI “LTE based 5G terrestrial broadcast - part 2”, four SCSs(15k/2.5k/1.25k/0.37k) are considered for RMC parameters of PMCH, as smaller SCS is beneficial to broadcast coverage, so for LTE-based 5G Broadcast for GSO, the four SCSs(15k/2.5k/1.25k/0.37k) can still be considered for RMC.

Proposal 2: Four SCSs(15k/2.5k/1.25k/0.37k) are considered for RMC parameters for LTE-based 5G Broadcast for GSO.

Based on the RMC template in the WF[2], we further calculate the Information Bit Payload and Binary Channel Bits Per Subframe for 15 kHz SCS as follows:

Table A.1.3.1-1 Fixed Reference Channel for PMCH Receiver Requirements (15 kHz SCS)

| Parameter | PMCH (15 kHz SCS) |  |  |
| --- | --- | --- | --- |
|  | Unit | Value |  |
| Reference channel |  | R.PMCH.1 | R.PMCH.2 |
| PMCH bandwidth | MHz | 10 | 10 |
| Allocated resource blocks |  | 50 | 50 |
| Allocated subframes per Radio Frame(Note1) |  | 10 | 10 |
| Modulation |  | QPSK | 16QAM |
| Target Coding Rate |  | 1/3 | 3/4 |
| Information Bit Payload (Note 2) |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 3624 | 15264 |
| Number of Code Blocks per Sub-Frame (Note 3) |  | 1 | 3 |
| Binary Channel Bits Per Subframe |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 10200 | 20400 |
| MBMS UE Category |  | ≥ 1 | ≥ 2 |
| Note 1: 2 OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |

Proposal 3: To define Fixed Reference Channel for PMCH Receiver Requirements (15 kHz SCS) as Table A.1.3.1-1.

2.3 Spurious response

Issue 3-1-9: Spurious emissions - Agreement: - No spec impact.

After our review of the agreements, it seems the agreement for the spurious emission requirement made in last meeting is not appropriate, we need to specify it for LTE based 5G broadcast utilizing satellite. The 10 MHz LTE spurious emission can be reused for 10MHz PMCH for LTE based 5G broadcast utilizing satellite in TS 36.102. The requirement is proposed as below:

Table 7.7C-1: Spurious response parameters

| Rx parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 10 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |
|  |  | 6 |
| NOTE: Reference measurement channel is specified in Annex A.1.3. |  |  |

Table 7.7C-2: Spurious response

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

Proposal 4: To specify the spurious emission requirement for LTE based 5G broadcast utilizing satellite as table 7.7C-1 and table 7.7C-2.

- Conclusion

In this contribution, some views on UE RF requirements are provided and the proposals ares made as following:

Proposal 1: To revise the OOBB requirement as follows:

Table 7.6C.3-2: Out of band blocking

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pw | dBm | REFSENS + 6 dB |  |  |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 246 | Finterferer (CW) | MHz | -95 < f – FDL_low < -15 or 15 < f – FDL_high < 85 | -95 < f – FDL_low ≤ -120 or 85 ≤ f – FDL_high < 110 | 1 ≤ f ≤ FDL_low – 120 or FDL_high + 110 ≤ f ≤ 12750 |

Proposal 2: Four SCSs(15k/2.5k/1.25k/0.37k) are considered for RMC parameters for LTE-based 5G Broadcast for GSO.

Proposal 3: To define Fixed Reference Channel for PMCH Receiver Requirements (15 kHz SCS) as Table A.1.3.1-1:

Table A.1.3.1-1 Fixed Reference Channel for PMCH Receiver Requirements (15 kHz SCS)

| Parameter | PMCH (15 kHz SCS) |  |  |
| --- | --- | --- | --- |
|  | Unit | Value |  |
| Reference channel |  | R.PMCH.1 | R.PMCH.2 |
| PMCH bandwidth | MHz | 10 | 10 |
| Allocated resource blocks |  | 50 | 50 |
| Allocated subframes per Radio Frame(Note1) |  | 10 | 10 |
| Modulation |  | QPSK | 16QAM |
| Target Coding Rate |  | 1/3 | 3/4 |
| Information Bit Payload (Note 2) |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 3624 | 15264 |
| Number of Code Blocks per Sub-Frame (Note 3) |  | 1 | 3 |
| Binary Channel Bits Per Subframe |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 10200 | 20400 |
| MBMS UE Category |  | ≥ 1 | ≥ 2 |
| Note 1: 2 OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |

Proposal 4: To specify the spurious emission requirement for LTE based 5G broadcast utilizing satellite as table 7.7C-1 and table 7.7C-2.

Table 7.7C-1: Spurious response parameters

| Rx parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 10 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |
|  |  | 6 |
| NOTE: Reference measurement channel is specified in Annex A.1.3. |  |  |

Table 7.7C-2: Spurious response

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

- Reference

RP-250788, New band for 5G Broadcast Utilizing Geosynchronous Satellite.

R4-2504725, WF for 114bis318 5G_Broadcast_GSO_NTN_band, Astrum Mobile, Qualcomm, ZTE, Huawei

3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception"

3GPP TS 36.102: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception for satellite access"