3GPP TSG-RAN WG4 Meeting #115	R4-2506241

# Malta, MT, 19th – 23rd May, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for FR2-1 L3 measurement delay reduction

Agenda item:	7.18.3.1

Document for:	Discussion

Introduction

In this contritubtion, we provide our views on remaining open issue on L3 measurement delay reduction with multi-Rx simultaneous reception based on [1].

Discussion

Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”

For conditions to trigger UE to quit the FR2 FBS, there would be two kinds of cases; first, if UE cannot do perform FR2 FBS due to power saving or overheating issues; second, if UE completes the measurement based on FR2 FBS. For the first case, Rel-18 UAI ‘multiRx-PreferenceFR2’ can be reused, and for the second case, UE can deactivate FR2 FBS operation when UE reports valid measurement result after activating FR2 FBS for L3 measurement based on serving cell’s signal quality for scenario 1/2/3/4.

Therefore, RAN4 can consider two conditions to trigger UE to quit the FR2 FBS below;

- Reuse Rel-18 UAI ‘multiRx-PreferenceFR2’

- UE deactivates FR2 FBS based L3 measurements when UE has reported qualified measurement results

In the last meeting, re-activation condition was discussed. Re-activation of FR2 FBS should be occur when seving cell’s signal quality is lower then the threshold configured by network and after quit conditions being discussed have been met. It is better to leave it to UE implementation whether to keep or re-active FR2 FBS, and RAN4 needs to define only conditions for activation/deactivation for FR2 FBS.

- Proposal 1: Two condition for deactivating multi-Rx for scenario 1/2/3/4 could be considered as below:

  - Reuse Rel-18 UAI ‘multiRx-PreferenceFR2’

  - UE deactivates FR2 FBS based L3 measurements when UE has reported qualified measurement results

- Proposal 2: RAN4 to leave it to UE implementation whether to keep or re-active FR2 FBS when seving cell’s signal quality is lower then the threshold configured by network and after quit conditions being discussed have been met.

Conclusion

In this contribution, we provide views on remaining open issues for multi-Rx simultaneous reception, and we propose

- Proposal 1: Two condition for deactivating multi-Rx for scenario 1/2/3/4 could be considered as below:

  - Reuse Rel-18 UAI ‘multiRx-PreferenceFR2’

  - UE deactivates FR2 FBS based L3 measurements when UE has reported qualified measurement results

- Proposal 2: RAN4 to leave it to UE implementation whether to keep or re-active FR2 FBS when seving cell’s signal quality is lower then the threshold configured by network and after quit conditions being discussed have been met.

Reference

[1] R4-2504900, “WF on RRM requirements for NR_RRM_Ph5_Part2,” CATT.