3GPP TSG-RAN WG4 Meeting #115	R4-2506243

# Malta, MT, 19th – 23rd May, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for (e)RedCap in NTN

Agenda item:	7.31.5.1

Document for:	Discussion

Introduction

In this contritubtion, we provide our views on HD-FDD related impact for RedCap NTN RRM requirements based on the approved WF [1].

Discussion

General HD-FDD related impact

<RAN1#119 Agreements> Agreement: - The legacy requirements and applicable conditions can be reused as baseline: - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state - Handover interruption time - Random access - SA: RRC Connection Release with Redirection - [Minimum requirement for L1 indication for RLM and LR] - Scheduling availability of UE performing intra/inter measurements RAN4 to further discuss: - The legacy requirements and applicable conditions can be reused as baseline for defining requirements of RedCap UE in NTN scenario, and discuss whether there is spec impact for the following requirements by HD-FDD: - FFS: Scheduling availability of UE during L1-RSRP measurement - Other requirement is not precluded - Candidate options for the potential spec impact for each of the above requirements (if needed): - Option a: the requirement is extended, e.g., the requirement does not count resources, channels, SSB, etc if they overlap with the other direction as specified a valid case in RAN1 specification. - Option b: Clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period. - Option c: The requirement applies when there is at least one RS within each measuremen interval. - FFS the same or different option for DRX and non-DRX.

In the last meeting, RAN4 agreed that the legacy requirements and applicable conditions are reused as baseline for some requirements of HD-FDD operation, and one of those is scheduling availability of UE performing intra/inter-frequency measurements. From the RAN4 perspective, intra/inter-frequency measurement would be priorizted over reception/transmission of other signals within SMTC, so legacy scheduling availability could be reused. However, basically the collision handling between UL and DL signals based on RAN1 agreements are for UL/DL of serving cell, and in some collision handling cases, newtowrk can indicate UL overriding DL. To clarify the UE behavior for scheduling availability, RAN4 needs to add condition to reuse legacy scheduling availability such as “even if network indicates UL prioritization overriding DL”.

- Proposal: (To clarify previous agreements) For HD-FDD, legacy scheduling availbability requirements of intra/inter-frequency measurement within SMTC can be reused even if network indicates UL prioritization overriding DL.

Conclusion

In this contribution, we provide views on HD-FDD related impact for (e)RedCap NTN, and we propose

- Proposal: (To clarify previous agreements) For HD-FDD, legacy scheduling availbability requirements of intra/inter-frequency measurement within SMTC can be reused even if network indicates UL prioritization overriding DL.

Reference

[1] R4-2504912, “WF on RRM requirements for NR_NTN_Ph3_Part1,” Moderator (CATT)