3GPP TSG-RAN WG4 Meeting #115	R4-2506244

Malta, MT, 19th – 23th May, 2025

Agenda Item:	7.22.3

Source:	CAICT, Ericsson, Qualcomm

Title:	Alignment between RAN1 direction C and RAN4 options

Document for:	Discussion

1	Introduction

4 options of test decoder are proposed in RAN 4 for AI/ML-based CSI compression test:

- Option 1: DUT provides the decoder

- Option 2: Infra vendor provides the decoder

- Option 3: Full decoder specification in standard

- Option 4: TE vendor provides the decoder

  - Option 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors

  - Option 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors

The feasibility of Option 3 and 4 are further studied in R19. Some agreements and conclusions have been achieved on simulation parameters, basic step for feasibility study, dataset construction, encoder/decoder format, simulation results, further refinement of option 3 and 4, etc.

In last RAN4 meeting, we had an agreement to further discuss the relation between RAN4 test decoder options and RAN1 inter-vendor interoperability directions. In this contribution, we provide our initial understanding on this aspect.

Agreements in RAN4#114bis Difference of option 3 and RAN1 inter-vendor collaboration direction C Agreements - FFS on the relation between RAN4 test decoder options and RAN1 inter-vendor interoperability directions.

2	Discussion

2.1 Overview

Following are related definitions excerpted from RAN1 agreements:

Agreements in RAN1 #116/#116bis: Option 1: Fully standardized reference model (structure + parameters) Option 3: Standardized reference model structure + Parameter exchange between NW-side and UE-side - 3a: Parameters received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing. - 3a-1: Model/Parameters exchanged from the NW-side to UE-side is CSI generation part. Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side - 4-1: Dataset exchanged from the NW-side to UE-side consists of (target CSI, CSI feedback). Agreement in RAN1 #118: - Direction A: Sharing parameters/reference model/dataset that enables UE-side offline engineering (Inter vendor collaboration option 3a/5a/4) - Direction B: Sharing NW side encoder parameter to UE side for UE side inference directly with on-device operation (Inter vendor collaboration option 3b) - Direction C: Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (Inter vendor collaboration option 1)

Following are related conclusions in RAN1:

Conclusion in RAN1 #120 - Direction C with standardized model ensures a minimum performance requirement. - The so called fully specified reference model discussed in RAN1 for Direction C is assumed to be equivalent to the to-be-specified model, if specified, for performance requirements in RAN4. - Direction A with standardized model parameter / dataset exchange from NW-side to UE-side may achieves better performance. Conclusion in RAN1 #120bis - For Direction A Option 3a-1, based on evaluation results, RAN1 concludes that scalable model structure specification is feasible. - For Direction C, based on evaluation results, RAN1 concludes that scalable model structure specification and model parameters specification is feasible. - Note: RAN4 feasibility is a separate study to be confirmed by RAN4. Conclusion in RAN1 #120bis - RAN1 concludes that both Direction A and Direction C are feasible. - RAN1 concludes that both sub-option 4-1 and sub-option 3a-1 are feasible. This includes both 3a-1 with and without target CSI sharing.

Observations on progress in RAN1:

- Direction B and Direction A option 5a lacks sufficient progress in RAN1.

- For Direction A, the discussion and progress mainly focus on sub-option 3a-1 and 4-1.

Discrepancy on working target:

- In RAN1 direction A (option 3a-1/4-1), the model parameters/dataset is shared from NW-side to UE-side and UE-side use the shared model parameters/dataset to develop own encoder.

- In RAN1 direction C, though the reference model is fully standardized, UE vendor can still develop their own encoder, as long as compatibility is kept.

- In RAN4, option 3/4a/4b is defined to develop test decoder. In option 4a/4b, TE vendor can develop their own test decoder. In option 3, TE vendor is required to implement the fully standardized test decoder.

2.2 Relation between RAN4 test decoder options and RAN1 inter-vendor interoperability directions

- Analysis on relation between RAN4 test decoder options and RAN1 inter-vendor interoperability directions:

In RAN1 direction C, fully standardized model means both structure and parameters are specified. According to the definition of option 3 in RAN4, a fully developed decoder would be chosen as frozen decoder, of which the structure and parameters are both fixed. Thus, the output of option 3 on decoder in RAN4 aligns with that of direction C in RAN1. In RAN4 #114, RAN4 has also agreed to define reference encoder including encoder structure and parameters to verify the performance of test decoder. Consequently, we understand RAN4 would output a fully standardized decoder and capture a reference encoder under option 3. From this perspective, the output of option 3 in RAN4 aligns with direction C in RAN1. It is noted that this had also been taken as a conclusion in RAN1.

Observation 1: The output of option 3 in RAN4 aligns with direction C in RAN1.

According to current working procedure of option 4a, a mixed dataset is used for companies to develop own encoder-decoder and do cross-checking on performance alignment. The mixed dataset contains eigen-values of channel matrix, which we can consider as target CSI. Hitherto, the content of specified dataset and whether to introduce reference encoder for option 4a are still FFS. Thus, the relation between option 4a in RAN4 and direction A/C in RAN1 is not fully clear, however alignment can be reached if an encoder is captured by RAN4.

Observation 2: As RAN4 agreed that reference decoder will be captured for option4, the output of option 4a in RAN4 may align with direction C if reference encoder will be captured.

According to current working procedure of option 4b, a reference encoder would be defined for company to train intermediate decoder and then train own encoder with the intermediate decoder. Thus, if the feasibility of option 4b is confirmed in RAN4, the whole procedure can be abstracted as UE-vendor develops an own encoder based on a fully standardized reference encoder. From this perspective, we understand option 4b in RAN4 aligns with direction C in RAN1.

Observation 3: The output of Option 4b in RAN4, at least for reference encoder, aligns with direction C in RAN1.

- Next step:

RAN1 has concluded that both direction A and C are feasible, but the conclusion is drawn based on backbone structure in Transformer, not CNN currently being used in RAN4 discussion. Furthermore, the target of RAN1 is trying to specify mechanism that could achieve optimal performance, while RAN4 is trying to define the minimum performance requirement. From this perspective, there are still some gaps between RAN1 and RAN4 understanding, which requires further alignment and conclusions from RAN4 to support the further specification works.

The following aspects expected to be confirmed in next RAN4 meeting:

Aspect 1: the feasibility of option 3/4a/4b.

In last RAN4 meeting, preliminary observations have been endorsed for option 3 (track 1/2) and option 4a/4b. RAN4 had additionally concluded that no further simulation activity is needed for option 3 track 1.

  - Option 3: it is observed that “It is possible for UE vendors to train an encoder based on another companies’ frozen test decoder that is functional” for both track 1 and 2. For track 1, “careful attention would be needed to the alignment of datasets”. For track 2, further evaluation work is required for the case that company use specific dataset when training with frozen decoder, and the case with a lower complexity encoder.

  - Option 4: it is observed that “It is feasible to train a functional “own” decoder based on the frozen encoder, and then a function “own” encoder based on “own” decoder when the mixed dataset is used”. Similar observation can be expected for option 4a if with mixed dataset. Further evaluation work on option 4a/4b is to complete inter-vendor training.

Based on current progress, a positive conclusion on the feasibility of option 3/4a/4b could be expected in next meeting. It should be noted that mixed dataset is crucial for performance convergence for all 3 options.

Proposal 1: For option 3, it is feasible to derive a test decoder based on an agreed structure and mixed dataset. How to obtain a real dataset, agree on a useful model structure and how to represent the model in 3GPP specifications would need further attention in a WI. Study on Option 4 could continue till August.

- Depending on the results for 4a, RAN4 might say Option 4a is feasible in May meeting.

Aspect 2: The relationship between option 3 in RAN4 and Direction C in RAN1

Based on above analysis, the equivalence between option 3 in RAN4 and direction C in RAN1 is expected to be confirmed in RAN4. Even the model structure assumed by the two WGs are different at present, it does not affect the final conclusion reached. The feasibility study and conclusion in RAN4 is not bundled with specific model structure and this also holds for the conclusions in RAN1.

Regarding to which group should be responsible for defining the standardized model in normative phase, RAN1 as the leading group should decide whether Direction C is specified in RAN1 or not. If yes, RAN4 can take the model(s) standardized in RAN1 as starting point to specify performance requirements. If not, RAN4 could make independent decision on whether to specify option 3/4a/4b.

Proposal 2: If direction C is adopted in the WI, RAN4 should leverage RAN1’s work related to model structure during the SI.

Aspect 3: suggested options for normative work

Based on precedent working in RAN4,

  - For option 3, we need to define model structure of both reference encoder and testing decoder, and parameters of training dataset, and align on the dataset itself.

  - For option 4b, we need to define model structure of reference encoder, and it is highly probable that we also need to define the parameters of training dataset (to generate mixed dataset) or align on the dataset itself.

  - For option 4a, we need to define the parameters of training dataset and align on the dataset itself, and it is highly probable that we also need to define the model structure of reference encoder (to verify the test decoder).

  - For all options, we need to devise a means of representing models in 3GPP (similar to ONNX, but within 3GPP control)

From our perspective, the normative workload for the 3 options is commensurate. Thus, we suggest that all 3 options could be approved for normative phase, if the feasibility has been confirmed.

Proposal 3: Any option concluded as feasible during the SI could be pursued in the WI phase, although the workload for devising an appropriate dataset, agreeing the structure and formulating a means of capturing models in the specification should be taken into account when deciding which options to pursue.

Conclusion

In this contribution, relation between RAN4 test decoder options and RAN1 inter-vendor interoperability directions are discussed with following observations and proposals:

Observation 1: The output of option 3 in RAN4 aligns with direction C in RAN1.

Observation 2: As RAN4 agreed that reference decoder will be captured for option4, the output of option 4a in RAN4 may align with direction C if reference encoder will be captured.

Observation 3: The output of Option 4b in RAN4, at least for reference encoder, aligns with direction C in RAN1.

Proposal 1: For option 3, it is feasible to derive a test decoder based on an agreed structure and mixed dataset. How to obtain a real dataset, agree on a useful model structure and how to represent the model in 3GPP specifications would need further attention in a WI.

- Depending on the results for 4a, RAN4 might say Option 4a is feasible in May meeting.

Proposal 2: If direction C is adopted in the WI, RAN4 should leverage RAN1’s work related to model structure during the SI.

Proposal 3: Any option concluded as feasible during the SI could be pursued in the WI phase, although the workload for devising an appropriate dataset, agreeing the structure and formulating a means of capturing models in the specification should be taken into account when deciding which options to pursue.