3GPP TSG-RAN WG4 Meeting #115	R4-2506245

# Malta, MT, 19th – 23rd May, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for downlink coverage enhancement in NTN

Agenda item:	7.31.5.2

Document for:	Discussion

Introduction

In this contritubtion, we provide our views on downlink coverage enhancement for NR NTN RRM requirements based on the approved WF [1] and RAN2 agreements.

Discussion

Downlink coverage enhancement

<RAN2#129bis Agreements> 3. RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases). 4. We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking) 5. Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

For downlink coverage enhancmemt, RAN2 agreed to support configuraing two different SMTC periodicities with different offsets in one frequency layer, and more than 4 SMTCs would be configurable. Considering the agreements, from the requirements perspective, there would be no impact on existing requirements in IDLE/INACTIVE. Even if more than 4 SMTCs per frequency carrier are configured, the UE behaviour is not different since RAN2 assumed no enhancement for more SMTCs/Gaps in parallel measurement than in previous release. Also, RAN2 agreed that network can provide assistance information to the UE about which SMTC it can use to perform neighbor cell measurements, and detailed method is still discussing, so RAN4 needs to wait RAN2 final conculsion and whether/how to capture it in RAN4 specification.

- Proposal 1: For the configuration of different SMTC periodicities SMTC with different offsets in IDLE / INACTIVE mode, RAN4 does not need to update existing requirements for cell re-selection. FFS whether/how to capture measurement behavior using network assistance information after finalizing RAN2 discussion.

- Proposal 2: RAN4 to keep existing UE capability of parallelSMTC, i.e., optionally support 4 SMTCs on a single frequency carrier.

RAN2 sent the LS [2] for feasibility of above RAN2 agreements.

For downlink coverage enhancement, RAN2 assumes that it will be possible to have different SSB periodicities among neighbour cells in the same frequency layer, and for the case of intra-frequency neighbouring cells, the SSBs of surrounding neighbour cells may be transmitted at different times. Thus, RAN2 made the following agreements, given that at any time the UE will not use more SMTCs/Gaps in parallel than in previous releases: - RAN2 considers to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements. - RAN2 agrees to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs. RAN2 would like RAN4 to provide feedback on whether it is feasible to support these agreements in Rel-19 timeframe.

Based on views on above proposals, it is feasible to support different SMTC periodicities with different offsets in a single frequency layer and configure more than 4 SMTCs with existing UE capability for parralel measurement. However, network assistance information on which RAN2 is discussing should be provided.

- Proposal 3: RAN4 to send reply LS that it is feasible to support the RAN2 agreements

Conclusion

In this contribution, we provide views on downlink coverage enhancement for NR NTN, and we propose

- Proposal 1: For the configuration of different SMTC periodicities SMTC with different offsets in IDLE / INACTIVE mode, RAN4 does not need to update existing requirements for cell re-selection. FFS whether/how to capture measurement behavior using network assistance information after finalizing RAN2 discussion.

- Proposal 2: RAN4 to keep existing UE capability of parallelSMTC, i.e., optionally support 4 SMTCs on a single frequency carrier.

- Proposal 3: RAN4 to send reply LS that it is feasible to support the RAN2 agreements

Reference

[1] R4-2504913, “WF on RRM requirements for NR_NTN_Ph3_Part2,” Moderator (Qualcomm Incorporated)

[2] R2-2503068, “LS on SMTC enhancements,” RAN2