3GPP TSG-RAN WG4 Meeting #115	R4-2506256

St Julian’s, Malta, 19th – 23rd May 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on Rx requirements for IoT-NTN HPUE

# Agenda item:	7.10.3.3.2

# Document for:	Approval

# 1 Introduction

In RAN#106 meeting, a revised Work Item [1] was approved, and one of the work area is the introduction of HPUE for NTN with the following RAN4 objective. In this contribution, we want to share some further views about IoT-NTN HPUE from Rx perspective.

High power UE (HPUE) for NTN - Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives

# 2 Discussion

Issue 3-1-1: PC2 1Tx RSD (full duplex eMTC based) - Proposals - Reference sensitivity values - 256: - 0 dB (MTK, Samsung, Ericsson) - 1.1dB (Qualcomm) - 255: - 0.8dB (MTK) - 1.2dB (Qualcomm) - No differentiation of handheld and non-handheld for RSD - WF (Ad-hoc agreement): - Reference sensitivity values - 256: [0.5] dB - 255: 1 dB Current Status: (For Information) Table: Current status for requirements consideration and feasibility for Iot-NTN Power Class; Antenna Numbers; Handheld and/or non-handheld. NB-Iot based; PC1; 1Tx; Both* *Note: Tones limitation FFS. PC2; 1Tx; Both. eMTC based with half duplex; PC1; 1Tx; Non-handheld. PC2; 1Tx; Both. eMTC based with full duplex; PC1; 1Tx; Non-handheld. PC2; 1Tx; Both.

In last meeting, RSD requirement was discussed for full duplex eMTC based IoT-NTN. For PC2, RSD values were almost defined since handheld will be supported for PC2. The remaining issue is whether to specify RSD requirement for PC1. For eMTC based with half duplex, PC1 is only applicable to non-handheld UE. During TN HPUE discussion, PC1 and PC1.5 UE also only support non-handheld type, and there is no RSD defined for PC1 and PC1.5 since non-handheld form factor terminals can provide better isolation performance than handheld types. From this perspective, it is unnecessary to specify RSD for PC1 non-handheld UE for eMTC based with half duplex.

Proposal 1: To specify 0 dB RSD for PC1 non-handheld UE for eMTC based with half duplex.

# 3 Conclusion

In this contribution, we shared some views on Rx requirements for IoT-NTN HPUE and the proposals are made as follows:

Proposal 1: To specify 0 dB RSD for PC1 non-handheld UE for eMTC based with half duplex.

# 4 Reference

- R4-2505113, WF on HPUE RF requirements for NR-NTN, Samsung

- R4-2503842, Iot NTN HPUE Tx Requirements and feasibility analysis, vivo, SmarterMicro