3GPP TSG-RAN WG4 Meeting #115	R4-2506260

St Julian’s, Malta, 19th – 23rd May 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on UE RF requirements for NR NTN Ku band

# Agenda item:	7.11.5

# Document for:	Approval

# 1 Introduction

In RAN#107 meeting, a revised Work Item [1] was approved on introduction of Ku Band for NR NTN for NR NTN with the following RAN4 objective. In this contribution, we want to share some further views about NR NTN Ku band from UE RF perspective.

The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate Tx/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services. - Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements - Investigate and if possible specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN) and NTN VSAT types (excluding types 1 to 3) in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing. - Note: Same subcarrier spacing is assumed for SSB and data channels - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Note: Simultaneous operation of FR1-NTN and FR2-NTN numerology is not supported in the UE - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A. - Specify RRM requirements including timing requirements for mobile VSAT served by NGSO - NOTE: RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only.

# 2 Discussion

  - 2.1 Handling of FCC, ISED, CEPT regulation related requirement

- Issue 1-1-5: Handling of FCC, ISED, CEPT regulation related requirement - New NSs will be defined for the Ku band concerning different regulations - FFS on whether different NS numbers will be used for FR1-NTN Ku bands and FR2-NTN Ku bands - Different NS numbers will be used to handle FCC and CEPT regulation at least - FFS on how to handle FCC and ISED requirements in TS 38.101-5 - Option 1: capture the most stringent requirement with same NSs - Option 2: separate NSs

For Ku bands, we agreed to introduce both FR1-NTN Ku bands and FR2-NTN Ku bands and specify common FR1 and FR2 radiated RF requirements for NTN VSAT types. Take an example, it will specify same requirements for band n509 and n248 based on FCC regulations. On the other hand, NS_0X corresponds to the conducted requirements, while NS_20X represents the radiated requirements. From our understanding, it is unnecessary to define different NS numbers for FR1-NTN Ku bands and FR2-NTN Ku bands. It is proposed to use the same NS number for FR1-NTN Ku bands and FR2-NTN Ku bands with the same frequency range as follows.

- For bands n509 and n248:

  - NS_203N: to handle FCC Regulation.

  - NS_204N: to handle CEPT Regulation.

Proposal 1: It is proposed to use the same NS number for FR1-NTN Ku bands and FR2-NTN Ku bands with the same frequency range as follows.

- For bands n509 and n248:

  - NS_203N: to handle FCC Regulation for GSO.

  - NS_204N: to handle FCC Regulation for LEO.

  - NS_205N: to handle CEPT Regulation for GSO.

  - NS_206N: to handle CEPT Regulation for LEO.

  - 2.2 OTA reference sensitivity level

- Issue 1-3-1: EIS requirement Agreement: - FFS on the following two options - Option 1: - Defining EISREFSENS_10MHz for FR1-NTN Ku bands, and define EISREFSENS_50MHz for FR2-NTN Ku bands - Option 2: - Consider define the common EIS requirement for both FR1-NTN Ku bands and FR2-NTN Ku bands - FSS on the value of the EISREFSENS_10MHz and EISREFSENS_50MHz - The requirement for other channel bandwidth will be scaled based on PRB based

For OTA reference sensitivity level, it is channel bandwidth dependent. In current spec, it specifies the minimum EIS for 50MHz channel bandwidth, and for other channel bandwidths larger than 50MHz, the corresponding EIS requirement could be scaled with PRB based compared with 50MHz. For FR1-NTN Ku bands and FR2-NTN Ku bands with the same frequency range, they share the same requirements. For simplification, we prefer to define the common EIS requirement for both FR1-NTN Ku bands and FR2-NTN Ku bands.

Proposal 2: Define the common EIS requirement for both FR1-NTN Ku bands and FR2-NTN Ku bands

In addition, during Ka band discussion, it defines the minimum EIS based on the link budget [3][4][5]. For Ku band, we can use the same approach to define the minimum EIS. To guarantee the VSAT to access the cell with low data rate and narrow channel bandwidth, we can assume 10MHz channel bandwidth derive the requirement. The calculation can be found in the following table.

Table 2.2-1: The minimum EIS calculation

|  | GSO |  | LEO600 |  |
| --- | --- | --- | --- | --- |
| h | 35786.00 | 35786.00 | 600.00 | 600.00 |
| B | 6378.00 | 6378.00 | 6378.00 | 6378.00 |
| r | 42164.00 | 42164.00 | 6978.00 | 6978.00 |
| Elevation_degree | 35.00 | 90.00 | 35.00 | 90.00 |
| Elevation_radian | 0.61 | 1.57 | 0.61 | 1.57 |
| D2 | 38180.79 | 35786.00 | 967.37 | 600.00 |
| f_MHz | 11000.00 | 11000.00 | 11000.00 | 11000.00 |
| Pathloss_FreeSpace | 204.90 | 204.34 | 172.98 | 168.83 |
| Atmosphere_loss | 2.00 | 2.00 | 2.00 | 2.00 |
| NF [dB] | 3.00 | 3.00 | 3.00 | 3.00 |
| BW [MHz] | 50.00 | 50.00 | 50.00 | 50.00 |
| SNR [target SNR] | -1.00 | -1.00 | -1.00 | -1.00 |
| IM | 2.00 | 2.00 | 2.00 | 2.00 |
| SAN_TxPower [dBm] | 87.00 | 90.00 | 51.00 | 54.00 |
| EIS_min | -119.92 | -116.35 | -123.99 | -116.84 |

Based on the above analysis, the minimum EIS could be defines as -124 dBm for 50MHz, and for the other channel bandwidth the corresponding EIS requirement could be scaled with PRB based compared with 50MHz for both GSO and LEO.

Proposal 3: The minimum EIS could be defined as -124 dBm for 50MHz, and for the other channel bandwidth the corresponding EIS requirement could be scaled with PRB based compared with 50MHz for both GSO and LEO.

  - 2.3 Maximum input level

For the maximum input level, we also can use same way as Ka band to derive the requirement, which is calculated based on the link budget. The calculation for the maximum input level is shown in the following table. The parameters used in the table is based on the coexistence simulation assumptions.

Table 2.3-1: The maximum input level calculation

|  | GSO |  | LEO600 |  |
| --- | --- | --- | --- | --- |
| h | 35786.00 | 35786.00 | 600.00 | 600.00 |
| B | 6378.00 | 6378.00 | 6378.00 | 6378.00 |
| r | 42164.00 | 42164.00 | 6978.00 | 6978.00 |
| Elevation_degree | 35.00 | 90.00 | 35.00 | 90.00 |
| Elevation_radian | 0.61 | 1.57 | 0.61 | 1.57 |
| D2 | 38180.79 | 35786.00 | 967.37 | 600.00 |
| f_MHz | 11000.00 | 11000.00 | 11000.00 | 11000.00 |
| Pathloss_FreeSpace | 204.90 | 204.34 | 172.98 | 168.83 |
| Atmosphere_loss | 2.00 | 2.00 | 2.00 | 2.00 |
| NF [dB] | 3.00 | 3.00 | 3.00 | 3.00 |
| BW [MHz] | 50.00 | 50.00 | 50.00 | 50.00 |
| IM | 2.00 | 2.00 | 2.00 | 2.00 |
| SAN_TxPower [dBm] | 87.00 | 90.00 | 51.00 | 54.00 |
| MaxInputPower | -119.92 | -116.35 | -123.99 | -116.84 |

Proposal 4: To consider the maximum input level -116 dBm for all channel bandwidths for Ku bands.

# 3 Conclusion

In this contribution, we shared some views on UE RF requirements for NTN Ku band and the proposals are made as follows:

Proposal 1: It is proposed to use the same NS number for FR1-NTN Ku bands and FR2-NTN Ku bands with the same frequency range as follows.

- For bands n509 and n248:

  - NS_203N: to handle FCC Regulation for GSO.

  - NS_204N: to handle FCC Regulation for LEO.

  - NS_205N: to handle CEPT Regulation for GSO.

  - NS_206N: to handle CEPT Regulation for LEO.

Proposal 2: Define the common EIS requirement for both FR1-NTN Ku bands and FR2-NTN Ku bands

Proposal 3: The minimum EIS could be defined as -124 dBm for 50MHz, and for the other channel bandwidth the corresponding EIS requirement could be scaled with PRB based compared with 50MHz for both GSO and LEO.

Proposal 4: To consider the maximum input level -116 dBm for all channel bandwidths for Ku bands.

# 4 Reference

- RP-250799, Revised WID: Introduction of Ku bands for NR NTN, Intelsat et al.

- R4-2504720, Way Forward for [114bis][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL)

- R4-2402522, Further discussion on UE RF Rx requirements for NTN in Ka-band, ZTE Corporation

- R4-2405642, Further discussion on UE RF Rx requirements for NTN in Ka-band, ZTE Corporation

- R4-2402063, Discussion on Rx requirement for Ka band NTN UE, Huawei, HiSilicon