3GPP TSG-RAN WG4 Meeting #115	R4-2506263

St Julian’s, Malta, 19th – 23rd May 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on SAN RF requirements for NR NTN Ku band

# Agenda item:	7.11.6

# Document for:	Approval

# 1 Introduction

In RAN#107 meeting, a revised Work Item [1] was approved on introduction of Ku Band for NR NTN for NR NTN with the following RAN4 objective. In this contribution, we want to share some further views about NR NTN Ku band from SAN RF perspective.

The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate Tx/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services. - Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements - Investigate and if possible specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN) and NTN VSAT types (excluding types 1 to 3) in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing. - Note: Same subcarrier spacing is assumed for SSB and data channels - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Note: Simultaneous operation of FR1-NTN and FR2-NTN numerology is not supported in the UE - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A. - Specify RRM requirements including timing requirements for mobile VSAT served by NGSO - NOTE: RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only.

# 2 Discussion

  - 2.1 SAN radiated Tx requirements

- Issue 2-2-1: The tolerance for the radiated transmit power and SAN output power Agreement: - For the tolerance of the radiated transmit power and SAN output power for both the FR1-NTN and FR2-NTN Ku bands - [+-3.4] dB EIRP accuracy, [+-3] dB TRP accuracy (re-using FR2-NTN) as baseline - Other options are not precluded - Other NTN bands will not be impacted - Issue 2-2-2: OTA RE power control dynamic range for the Ku band SAN Agreement - The OTA RE power control dynamic range is not required for FR2-NTN Ku-band SAN. - Further discuss whether OTA RE power control dynamic range (i.e. RE power boosting) is needed for FR1-NTN Ku-band SAN for the smaller channel bandwidth. - Issue 2-2-3: OTA total power dynamic range for the Ku band SAN Agreements: - Agree on adding the following OTA SAN total power dynamic range requirement for new channel bandwidths of FR1-NTN Ku bands. Table 2-2-3.3 SAN channel; Total power dynamic range (dB). bandwidth (MHz); 15 kHz SCS; 30 kHz SCS. 25; 21.2; 18.1. 35; 22.7; 19.6. 50; 24.3; 21.2. 70; N/A; 22.7. 100; N/A; 24.3. - Issue 2-2-5: OTA sensitivity for the Ku band SAN - Proposals: - Proposal 1 (ZTE): It is proposed not to specify OTA sensitivity for NTN Ku bands. - Observation 1 (Nokia): OTA sensitivity and Rx dynamic range requirements can be omitted for Ku bands with FR1 numerologies. Agreement: - Proposal 1 (ZTE): It is proposed not to specify OTA SAN sensitivity for NTN Ku bands. - Issue 2-2-7: OTA dynamic range for the Ku band SAN - Proposals: - Proposal 1 (ZTE): Not to specify OTA dynamic range for NTN Ku bands. Agreement: - Proposal 1 (ZTE): Not to specify OTA SAN dynamic range for NTN Ku bands.

The above is the agreement and way forward we reached in last meeting [2]. There are some remaining issues need to be further discussed.

    - 2.1.1 OTA transmitter spurious emissions

For spurious frequency range, the upper limit is different between SAN type 1-O and 2-O in current spec. The upper limit of spurious frequency range for measurements is defined following the principle in ITU-R SM.329 as follows. For NTN Ku bands, we also propose to adhere the rule in ITU-R SM.329 to define the upper limit of spurious frequency range as 26 GHz due to Ku bands DL is 10.7-12.75 GHz.

Proposal 1: It is proposed to define the upper limit of spurious frequency range as 26 GHz for both FR1-NTN Ku bands and FR2-NTN Ku bands.

  - 2.2 SAN radiated Rx requirements

    - 2.2.1 OTA reference sensitivity level

For OTA REFSENS, it is channel BW and SCS specific. The requirement for SAN type 1-O is 2.4 dB lower than EISREFSENS in 38.104 with same reference measurement channel. For SAN type 2-O (also for Ka bands), it is a range and declared by the vendor, which is defined by the following equation with the assumption of the smallest supported bandwidth 50 MHz [3, 4].

We propose to use the same approach as Ka bands to define OTA REFSENS with the following assumptions:

PkT = -174 dBm/Hz.

IM = 2 dB.

BW for EISREFSENS_50M for Ku-band SAN is 270 RB, i.e. 270*12*15*1000Hz.

-1dB SNR can be reused for EIS for Ku-band SAN.

NF for GEO and LEO SAN: 3 dB.

G represents SAN Rx antenna gain. Since OTA REFSENS is range calculated based on antenna gain range, we can assume SAN antenna gain for Ku band: G = [36-44] dBi, and [30.5-38.5] dBi for GEO and LEO-600 respectively.

In addition, 3dB off peak margin need to be considered. Hence, EISREFSENS_50M can be calculated by the following for GEO class SAN.

So we propose to define OTA REFSENS for both FR1-NTN Ku bands and FR2-NTN Ku bands as follows:

For GEO class SAN, EISREFSENS_50M is an integer value in the range [-156] to [-164] dBm. The specific value is declared by the vendor.

For LEO class SAN, EISREFSENS_50M is an integer value in the range [-151] to [-159] dBm. The specific value is declared by the vendor.

Proposal 2: It is proposed to define OTA REFSENS for both FR1-NTN Ku bands and FR2-NTN Ku bands as follows:

For GEO class SAN, EISREFSENS_50M is an integer value in the range [-156] to [-164] dBm. The specific value is declared by the vendor.

For LEO class SAN, EISREFSENS_50M is an integer value in the range [-151] to [-159] dBm. The specific value is declared by the vendor.

    - 2.2.2 OTA out-of-band blocking

For OTA out-of-band blocking, we need to specify ∆fOOB requirement for Ku band. According to TR 38.922, it is agreed to adopt ΔfOOB = 100 MHz for AAS BS in 14800 - 15350 MHz frequency range. We propose to reuse ΔfOOB = 100 MHz for both FR1-NTN Ku bands and FR2-NTN Ku bands.

Proposal 3: It is proposed to reuse ΔfOOB = 100 MHz for both FR1-NTN Ku bands and FR2-NTN Ku bands.

    - 2.2.3 OTA in-channel selectivity

For Ka band, SAN OTA ICS requirement is defined through figuring out IOT levels in this system due to the co-channel interference from other cells or beams [5][6]. For Ku band, we can adopt the same approach to specify OTA ICS requirement. Based on the agreed coexistence simulation assumptions, we simulate IOT levels as shown in following figures. It can be observed that IOT @ 95% CDF is 25dB for VSAT parabolic and phased array antenna pattern for GEO scenario when FRF=1, which is similar as Ka band. If FRF=2 with dual polarization is considered, the expected IoT level would be close to 0dB.

Figure 1: IoT level for VSAT parabolic antenna pattern for GEO scenario [FRF=1]

Figure 2: IoT level for VSAT phased array antenna pattern for GEO scenario [FRF=1]

Observation 1: For FRF=1, IOT @ 95% CDF is 25dB for GEO scenario.

# 3 Conclusion

In this contribution, we share some views on SAN RF requirements for NTN Ku band and the proposals are made as follows:

Proposal 1: It is proposed to define the upper limit of spurious frequency range as 26 GHz for both FR1-NTN Ku bands and FR2-NTN Ku bands.

Proposal 2: It is proposed to define OTA REFSENS for both FR1-NTN Ku bands and FR2-NTN Ku bands as follows:

For GEO class SAN, EISREFSENS_50M is an integer value in the range [-156] to [-164] dBm. The specific value is declared by the vendor.

For LEO class SAN, EISREFSENS_50M is an integer value in the range [-151] to [-159] dBm. The specific value is declared by the vendor.

Proposal 3: It is proposed to reuse ΔfOOB = 100 MHz for both FR1-NTN Ku bands and FR2-NTN Ku bands.

Observation 1: For FRF=1, IOT @ 95% CDF is 25dB for GEO scenario.

# 4 Reference

- RP-250799, Revised WID: Introduction of Ku bands for NR NTN, Intelsat et al.

- R4-2504720, Way Forward for [114bis][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL)

- R4-2313996, WF on SAN RF, Ericsson, RAN4#108

- R4-2309770, WF on above 10 GHz SAN RF requirements, CATT, RAN4#107

- R4-2402520, Further discussion on SAN RF requirements for NTN in Ka-band, ZTE Corporation

- R4-2400041, Further discussion on SAN RF requirements for above 10GHz bands, CATT