3GPP TSG-RAN WG4 Meeting # 115	R4-2506268

# St Julian’s, Malta, 19th - 23th, May 2025

Source: 	Samsung

Title: 	Leftover issue for non-collocated

Agenda Item:	7.4.2.1

Document for:	Discussion

- Introduction

In RAN4#114bis, LS was sent out to ask RAN2 to devise the necessary new UE capability(s) and NW signalling(s) for Type-4, in which only the agreements reached in previous meetings have been captured. As said offline, we are happy to further discuss the leftover issues if any in future meetings.

This paper discusses one leftover issue.

- 2. Discussion

For Type-2 capability, the following description which is intended for “Type-2 supported UE shall also meet Type-1 requirements” was added following Rel-18 RAN-P LS for NR-CA, while there is no similar description for EN-DC which was introduced in Rel-16.

| intraBandNR-CA-non-collocated-r18 Indicates whether the UE supports TDD-TDD intra-band non-collocated NR-CA operation with MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [5] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [2], and TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in TS 38.133 [5] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [2]. If the capability is not reported, the UE only supports TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in TS 38.133 [5] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [2]. A UE supporting this feature shall also support network controlled indication of the MTTD/MRTD and RF requirements by nonCollocatedTypeNR-CA-r18 for intra-band non-collocated NR-CA, as defined in TS 38.331 [9]. | BC | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |

| interBandMRDC-WithOverlapDL-Bands-r16 Indicates whether the UE supports FDD-FDD or TDD-TDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [5] and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 [5] and inter-band RF requirements. If the capability is not reported, the UE supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially overlapping DL bands with (NG)EN-DC/NE-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [5] and intra-band RF requirements. | BC | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |

Similar point should be discussed for Type-4 as well.

Proposal 1: RAN4 to discuss if Type-4 supported UE shall also meet Type-1 requirements, and its applicability (i.e., for NR-CA and/or EN-DC?).

Any agreements reached in this meeting should be delivered to RAN2 in a proper way. Per RAN2 LS [2] on Rel-19 higher layer parameter and UE Feature, RAN2 prefer to receive the full UE feature list for the new UE capabilities rather than LS with no full information. RAN2 understands that RAN1 and RAN4 can still send early LSs to explain capabilities, but it is important that those capabilities are all captured in the feature list at the end.

With above, we think any agreement reached relevant to Proposal 1 can be included in the feature list RAN4 LS to RAN2, so RAN2 can update their CRs based on full UE feature list for this capability.

Proposal 2: The agreement reached on Proposal 1 can be included in RAN4 feature list LS to RAN2.

- 3. Conclusion

Proposal 1: RAN4 to discuss if Type-4 supported UE shall also meet Type-1 requirements, and its applicability(i.e., for NR-CA and/or EN-DC?).

Proposal 2: The agreement reached on Proposal 1 can be included in RAN4 feature list LS to RAN2.

- 4. Reference

[1] R4-2505161, LS to RAN2 on Type-4 for non-collocated deployment, Samsung, RAN4#114bis

[2] R2-2503180, LS on Rel-19 higher layer parameter and UE Feature, Xiaomi, RAN2#129bis