3GPP TSG-RAN WG4 Meeting # 115	R4-2506269

# Malta, 19th - 23th, May 2025

Source: 	Samsung

Title: 	Views on 3 switching solutions for n5-n8 2UL/2DL

Agenda Item:	6.11.2

Document for:	Discussion

- Introduction

In last meeting, companies had fruitful offline discussion for 2UL/2DL n5-n8 with non-simultaneous Tx/Rx between n5 DL and n8 UL. There was no determinist conclusion reached and discussion are expected to continue in this meeting based on offline outcome which can be generally categorized into 3 solutions:

- Solution 1: Switching by activation/deactivation (MAC-CE based) between case 1 and case 2

- Solution 2: Introduce a new UE capability of switching gap, Network accommodates the switching gap by scheduling (DCI-based), cross carrier scheduling might be needed (from case 2 to case 1)

- Solution 3: Further improvement, e.g., via L-L CA via switching is not precluded in Rel-20 or later (Semi-static pattern configured by NW)

Case 1 and Case 2 above refers to:

-  Case 1 (n5DL+n8DL; n5UL)

-  Case 2 (n5DL; n5UL+n8UL→UL only SCell)

Companies agree to further check the feasibility (including if there is RRM or other working group impact) of Solution 1 and 2 in May meeting in order to determine whether n5-n8 2UL/2DL with non-simultaneous Tx/Rx between n5 DL and n8 UL can be introduced in Rel-19.

We present our views for the 3 identified switching solutions in this paper.

- Discussion

The following agreements were made during online, it is worthy to make it clear that TS 38.133 is not listed as the impacted TS of Simultaneous RxTx WI, hence any solution leading to RRM impact are not expected to be adopted.

Chair: Continue the discussions of CA n5-n8 in the simultaneous Rx-Tx WI and no upscope of discussion is allowed.

2.1 Solution1 (Switching by MAC-CE based activation/deactivation)

Qualcomm’ paper [1] illustrates the typical RFFE to support 2UL/2DL n5-n8 with non-simultaneous Tx/Rx between n5 DL and n8 UL which is excerpted below, as can be found the architecture is more complex than the normal CA, and switching of RFFE is expected between case 1 and case 2.

(Copied from [1])

Solution 1 intends for NW only activates either n5DL or n8UL. Generally speaking RFFE switching period (35 us, 140us, 210us as typical values) can be covered by cell activation/deactivation time which is “ms” level and at least larger than 5ms, so most likely no extra interrupt/switch time is needed.

However, it doesnot mean there is no RRM impact at all, the following are the identified RRM requirements we should go through for solution 1.

- Interruption requirement

- Switching delay requirement

- Timing requirement

- SCell measurement requirement

We believe there is RRM impact and it is not easy to draw conclusion for all above identified impacts within 2 or 3 meetings even if this solution is posted to RRM room, given “UL only SCell” haven’t been supported yet by RAN4, for which“under what assumption”, the requirements or which requirements can be applied should be discussed further.

During offline discussion, company raised that whether the RRM impact could be smaller or marginal if the duration of Case 2 (n5DL; n5UL+n8UL→SCell UL only) is limited to certain value such as 160ms. In our understanding, 160ms is just for timing requirement. According to the TA mechanism, at UE side, the uplink transmission from the UE shall start TTA before the start of the downlink timing (according to the DL reference point). If the uplink only SCell would like to use the DL timing from the PCell, it shall ensure the performance by some assumption.

The following is defined for the initial transmission timing error requirements to make sure there is at least one SSB is available at the UE during the last 160ms for FR1.

When the UL SCS is 120 kHz or smaller, the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. When the UL SCS is 480 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available in the last 80 ms. When the UL SCS is 960 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available in the last 40 ms.

In our opinion, it might be not easy to conclude that timing requirement can be reused from legacy even the duration time for Uplink only SCell is within 160ms, let alone timing requirement is not the only impacted requirement.

Observation 1: For Solution 1, not only the interrupt requirements, but also the delay requirement, timing requirements, SCell measurement requirements need to be checked. It is not such easy to draw quick conclusion for these RRM requirements, especially considering UL only SCell has never been discussed before.

Proposal 1: Solution 1 (activation/deactivation-based switching) has RRM impact and should therefore not be considered in Rel-19.

  - Solution 2 (DCI-based switching)

Solution 2 in our understanding refers to DCI-based switching following NW scheduling. New UE capability is needed for switching gap (i.e., switching period and location) to cover necessary switching period including RFFE configuration switching, and Network needs to accommodate the switching gap by scheduling. For this solution, cross-carrier scheduling seems needed given in case 2 there is only 1DL with 2UL, while from what we heard cross-carrier scheduling has never been implemented by any infra vendor due to the complexity though it has been introduced since LTE period.

Similarly as Solution 1, from feature completeness perspective, the following RRM requirements have to be checked and it is not such easy to draw quick conclusion.

- Interruption requirement

- Switching delay requirement

- Timing requirement

- SCell measurement requirement

Observation 2: For Solution 2, not only the interrupt requirements, but also the delay requirement, timing requirements, SCell measurement requirements need to be checked, and it is not such easy to draw quick conclusion for these RRM requirements, especially considering UL only SCell has never been discussed before.

Observation 3: Solution 2 requires cross-carrier scheduling, which seems never been implemented by any infra vendor.

Proposal 2: Solution 2 (DCI-based switching) has RRM impact and should therefore not be considered in Rel-19.

2.3 Solution 3 (Semi-static switching pattern configured by NW)

Several companies including us commented that n5-n8 can be a potential suitable candidate to be considered in L-L CA via switching enh WI (if any) in future release, as it is not band switching but more like “UL-DL switching” which is not in Rel-19 L-L CA via switching WI scope thus not covered by RAN1 semi-static pattern design nor RAN4 RF/RRM discussion. During offline discussion of last meeting, companies seem all agreed that at least the discussion of this direction should be stopped in Rel-19, but not precluded in future release.

Proposal 3: Solution 3(semi-static switching pattern configured by NW) has RRM and RAN1 impact and should therefore not be considered in Rel-19, but not precluded in future release as potential new scenario for L-L CA via switching enh (if any).

- 3. Conclusion

-Solution1 (Switching by MAC-CE based activation/deactivation)

Observation 1: For Solution 1, not only the interrupt requirements, but also the delay requirement, timing requirements, SCell measurement requirements need to be checked. It is not such easy to draw quick conclusion for these RRM requirements, especially considering UL only SCell has never been discussed before.

Proposal 1: Solution 1 (activation/deactivation-based switching) has RRM impact and should therefore not be considered in Rel-19.

-Solution 2 (DCI-based switching)

Observation 2: For Solution 2, not only the interrupt requirements, but also the delay requirement, timing requirements, SCell measurement requirements need to be checked, and it is not such easy to draw quick conclusion for these RRM requirements, especially considering UL only SCell has never been discussed before.

Observation 3: Solution 2 requires cross-carrier scheduling, which seems never been implemented by any infra vendor.

Proposal 2: Solution 2 (DCI-based switching) has RRM impact and should therefore not be considered in Rel-19.

-Solution 3 (Semi-static switching pattern configured by NW)

Proposal 3: Solution 3(semi-static switching pattern configured by NW) has RRM and RAN1 impact and should therefore not be considered in Rel-19, but not precluded in future release as potential new scenario for L-L CA via switching enh (if any).

- 4. Reference

[1] R4-2504116, CA_n5A-n8A capabilities, Qualcomm France