3GPP TSG-RAN4 Meeting # 115	R4-2506270

Malta, May.19th – 23th, 2025

# Title: 	TP on RF requirements evaluation for fragmented carriers from company S

# Source: 	Samsung

# Agenda item:	7.5.4

# Document for:	Approval

- Introduction

This contribution provides a text proposal to TR38.755[1] on UE RF requirement evaluation for fragmented carriers based on our previous contributions.

- Reference

[1] TR38.755, Study on NR FR1 DL fragmented carriers, v0.2

- Text Proposal

----- Start of TP -----

## 7.x	RF requirements evaluation from company S

In the following content, we conduct link budget calculation for several intra-band non-contiguous combos. It is evident that the Tx leakage level of the shared Rx RF Chain at the RFIC output is unacceptable high. Though the AGC can be utilized to reduce the Tx leakage level in the RFIC block, it is important to note that the Tx leakage level is already quite strong in the analog block of the RFIC, which has the potential to reach the saturation limit. When the analog block saturates, it loses its ability to effectively amplify the received signal, resulting in distortion and degradation of the signal-to-noise ratio (SNR). The primary reason for the higher Tx leakage is the wider cut-off frequency of the analog filter. When using a partially shared Rx RF Chain, separate analog filters can be employed for the PCC and SCC, narrow filter response can be applied to both carrier components. However, the filter response becomes wider with the implementation of a shared Rx RF Chain, resulting in inadequate filter rejection at the Tx frequency. Additionally, we have taken into account other impairment such as “NF increase due to AGC adjustment” to determine the extent of extra relaxation needed to support the shared Rx RF chain architecture through AGC operation. These factors in the end leads to the REFSENS degradation for both PCC and SCC. Similarly for ACS and IBB, these factors also lead to the performance degradation to some extent.

Analysis on CA_n3(2A):

The link budget of Self-interference for 5+5MHz with WGAP=65MHz on band n3 can be found in table 7.x.1-1.

Table 7.x.1-1: Link budget of Self-interference for 5+5 MHz on band n3

|  | Tx leakage | Wanted Carrier | Wanted Carrier | Noise floor | Tx leakage mapping to -12dBFS (PAPR is considered) |
| --- | --- | --- | --- | --- | --- |
|  |  | (PCC) | (SCC) |  |  |
| PA out | 27 | / | / | / | / |
| Duplexer isolation | 50 | / | / | / | / |
| Rx Antenna input | / | -97 | -92.3 | / | / |
| FE loss | / | 4 | 4 | / | / |
| Duplexer output | -23 | -101 | -96.3 | / | / |
| LPF attenuation | 7.9 | 1 | 1 | / | / |
| LPF output (dBm) | -30.9 | -102 | -97.3 | / | / |
| Noise floor for basic REFSENS | / | / | / | -101 | / |
| NF increase due to AGC adjustment | / | / | / | 9.3 | / |
| Noise floor | / | / | / | -91.7 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -96.3 | / | / |
| ADC output (dBm) | / | / | / | / | -86.9 |
| SNR after ADC (dB) | / | / | / | / | -16.3(PCC) |
|  |  |  |  |  | -12 (SCC) |

The Link budget of ACS case 1 and case 2 with and without Rx image on band n3 can be found in table 7.x.1-2. 25dB rejection Rx image is used for the calculation.

ACS: with and without Rx image

Table 7.x.1-2. Link budget of ACS cases on band n3

| LPF attenuation (dB) | 7.9 |  |  |  |
| --- | --- | --- | --- | --- |
| NF increase due to AGC adjustment(dB) | 9.3 |  |  |  |
|  | Without Rx image |  | With Rx image |  |
|  | Case 1 | Case 2 | Case 1 | Case 2 |
| SNR after ADC (dB) for PCC | -4.7 | -1 | -4.7 | -1 |
| SNR after ADC (dB) for SCC | -2.6 | -1 | -5.3 | -7.6 |

The Link budget of IBB case 1 and case 2 with and without Rx image on band n3 can be found in table 7.x.1-3. 25dB rejection Rx image is used for the calculation.

IBB: with and without Rx image

Table 7.x.1-3. Link budget of IBB cases on band n3

| LPF attenuation (dB) | 7.9 |  |  |  |
| --- | --- | --- | --- | --- |
| NF increase due to AGC adjustment(dB) | 9.3 |  |  |  |
|  | Without Rx image |  | With Rx image (25dB rejection) |  |
|  | Case 1 | Case 2 | Case 1 | Case 2 |
| SNR after ADC (dB) for PCC | -10.8 | -10.8 | -10.8 | -10.8 |
| SNR after ADC (dB) for SCC | -6.9 | -6.9 | -9.2 | -17.7 |

Analysis on CA_n2(2A):

The link budget of Self-interference for 5+5MHz with WGAP=50MHz on band n2 can be found in table 7.x.1-4, respectively.

Table 7.x.1-4: Link budget of Self-interference for 5+5 MHz on band n2

|  | Tx leakage | Wanted Carrier | Wanted Carrier | Noise floor | Tx leakage mapping to -12dBFS (PAPR is considered) |
| --- | --- | --- | --- | --- | --- |
|  |  | (PCC) | (SCC) |  |  |
| PA out | 27 | / | / | / | / |
| Duplexer isolation | 50 | / | / | / | / |
| Rx Antenna input | / | -98 | -93 | / | / |
| FE loss | / | 4 | 4 | / | / |
| Duplexer output | -23 | -102 | -97 | / | / |
| LPF attenuation | 13 | 1 | 1 | / | / |
| LPF output (dBm) | -36 | -103 | -98 | / | / |
| Noise floor for basic REFSENS | / | / | / | -102 | / |
| NF increase due to AGC adjustment | / | / | / | 5 | / |
| Noise floor | / | / | / | -97 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -97 | / | / |
| ADC output (dBm) | / | / | / | / | -92 |
| SNR after ADC (dB) | / | / | / | / | -12.2 (PCC) |
|  |  |  |  |  | - 8.1 (SCC) |

The Link budget of ACS case 1 and case 2 with and without Rx image on band n2 can be found in table 7.x.1-5. 25dB rejection Rx image is used for the calculation.

ACS: with and without Rx image

Table 7.x.1-5. Link budget of ACS cases on band n2

| LPF attenuation (dB) | 13 |  |  |  |
| --- | --- | --- | --- | --- |
| NF increase due to AGC adjustment(dB) | 5 |  |  |  |
|  | Without Rx image |  | With Rx image |  |
|  | Case 1 | Case 2 | Case 1 | Case 2 |
| SNR after ADC (dB) for PCC | -2.8 | -1 | -2.8 | -1 |
| SNR after ADC (dB) for SCC | -1.7 | -1 | -4.6 | -7.6 |

The Link budget of IBB case 1 and case 2 with and without Rx image on band n2 can be found in table 7.x.1-6. 25dB rejection Rx image is used for the calculation.

IBB: with and without Rx image

Table 7.x.1-6. Link budget of IBB cases on band n2

| LPF attenuation (dB) | 13 |  |  |  |
| --- | --- | --- | --- | --- |
| NF increase due to AGC adjustment(dB) | 5 |  |  |  |
|  | Without Rx image |  | With Rx image (25dB rejection) |  |
|  | Case 1 | Case 2 | Case 1 | Case 2 |
| SNR after ADC (dB) for PCC | -7.3 | -7.3 | -7.3 | -7.3 |
| SNR after ADC (dB) for SCC | -4.1 | -4.1 | -8.2 | -18.2 |

Analysis on CA_n25(2A):

The link budget of Self-interference for 5+5MHz with WGAP=55MHz on band n25 can be found in table 7.x.1-7.

Table 7.x.1-7: Link budget of Self-interference for 5+5 MHz on band n25

|  | Tx leakage | Wanted Carrier | Wanted Carrier | Noise floor | Tx leakage mapping to -12dBFS (PAPR is considered) |
| --- | --- | --- | --- | --- | --- |
|  |  | (PCC) | (SCC) |  |  |
| PA out | 27 | / | / | / | / |
| Duplexer isolation | 50 | / | / | / | / |
| Rx Antenna input | / | -96.5 | -91.5 | / | / |
| FE loss | / | 4 | 4 | / | / |
| Duplexer output | -23 | -100.5 | -95.5 | / | / |
| LPF attenuation | 8.6 | 1 | 1 | / | / |
| LPF output (dBm) | -31.6 | -101.5 | -96.5 | / | / |
| Noise floor for basic REFSENS | / | / | / | -100.5 | / |
| NF increase due to AGC adjustment | / | / | / | 9.2 | / |
| Noise floor | / | / | / | -91.3 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -95.5 | / | / |
| ADC output (dBm) | / | / | / | / | -87.6 |
| SNR after ADC (dB) | / | / | / | / | -15.4 (PCC) |
|  |  |  |  |  | -10.9 (SCC) |

The Link budget of ACS case 1 and case 2 with and without Rx image on band n25 can be found in table 7.x.1-8. 25dB rejection Rx image is used for the calculation.

ACS: with and without Rx image

Table 7.x.1-8. Link budget of ACS cases on band n25

| LPF attenuation (dB) | 8.6 |  |  |  |
| --- | --- | --- | --- | --- |
| NF increase due to AGC adjustment(dB) | 9.2 |  |  |  |
|  | Without Rx image |  | With Rx image |  |
|  | Case 1 | Case 2 | Case 1 | Case 2 |
| SNR after ADC (dB) for PCC | -4.2 | -1 | -4.2 | -1 |
| SNR after ADC (dB) for SCC | -2.3 | -1 | -4.9 | -7.6 |

The Link budget of IBB case 1 and case 2 with and without Rx image on band n25 can be found in table 7.x.1-9. 25dB rejection Rx image is used for the calculation.

IBB: with and without Rx image

Table 7.x.1-9. Link budget of IBB cases on band n25

| LPF attenuation (dB) | 8.6 |  |  |  |
| --- | --- | --- | --- | --- |
| NF increase due to AGC adjustment(dB) | 9.2 |  |  |  |
|  | Without Rx image |  | With Rx image (25dB rejection) |  |
|  | Case 1 | Case 2 | Case 1 | Case 2 |
| SNR after ADC (dB) for PCC | -10 | -10 | -10 | -10 |
| SNR after ADC (dB) for SCC | -6.1 | -6.1 | -8.4 | -16.9 |

----- End of TP -----