3GPP TSG-RAN4 #115	R4-2506283

St. Julian’s, Malta, 19th May – 23rd May 2025

Title:	Demodulation test time issue with LEO-600

Source:			Anritsu Corporation

Agenda Item:			7.10.5.3

Document for:			Approval

# 1.	Introduction

At the RAN4 #114-bis meeting in Wuhan, we discussed on the NTN testing for NGSO UE under time varying doppler shift and propagation delay. And some open issues were captured in the way forward [1].

In this contribution we discuss the issue 1-2-5, 1-2-7 and 1-2-8, extracted below for reference.

Issue 1-2-5 Test cases for NR NTN with applying time varying Doppler Shift and propagation delay model - agreement - Apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 38.101-5 Clause 8.2.1.2.2.1 Test 1-1 and Test 1-2. - FFS on test 1-3 and 1-4 - Note: test 1-3 is related with UE capability to support 32 HARQ process, and test 1-4 is related with UE capability with HARQ disable Issue 1-2-7 Test case for NB-IoT NTN with applying the time-varying Doppler shift and propagation delay model - agreement - Apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 36.102 Clause 8.3.1.1 Test 1 - FFS If the LEO-600 model is not enough to cover the test time of Test 1, RAN4 should consider using the LEO-1200 model Issue 1-2-8 Test case for eMTC NTN with applying the time-varying Doppler shift and propagation delay model - agreement - FFS to consider Cat-M1 demodulation requirement with applying time varying Doppler shift and propagation delay model - If considering Cat-M1 demodulation requirement with applying time varying Doppler shift and propagation delay model - Cover test 1 and test 2 according to TS 36.102 Clause 8.2.1.1 - FFS to cover test 3

# 2.	Discussion

  - Identified test time issue with the demodulation PDSCH tests

As discussed until the last meeting, for NGSO (LEO-600) RRM/Demod test cases, the initial ephemeris information is assuming an elevation angle from 30 degrees and 150 degrees. And considering that angles, an available measurement period for LEO-600 scenario becomes 250 seconds maximum within that range [3].

Observation 1: Available measurement period for LEO-600 scenario is 250 seconds maximum under the assumption of the moving satellite with the elevation angle from 30 to 150 degrees.

Figure 2.2-1: Elevation angle for LEO-600

If we see the current test parameter for demodulation test for example in Table 8.2.1.2.1.2.1-2 of TS 38.101-5 [4], the parameter cellSpecificKoffset-r17 is defined as 8 slots for 15kHz SCS case. Therefore, considering the legacy k2 value (i.e. 2 slots) plus this cellSpecificKoffset-r17, 2 + 8 slots = 10 slots would be necessary for the HARQ processing, which means longer test time than a legacy test case is expected.

Now referring to the minimum test time definition table G.1.5-1 in TS 38.521-5 [2] for the PDSCH demodulation tests, NR FDD PDSCH test cases take from 32 to 85 seconds with reference channels R.PDSCH.1-1.1 FDD or R.PDSCH.1-2.1 FDD. (Table G.1.5-1 is extracted in Appendix A at the end of this contribution.)

Observation 2: NR FDD PDSCH demodulation test cases for LEO-600 are expected to take a test time from 32 to 85 seconds with reference channels R.PDSCH.1-1.1 FDD or R.PDSCH.1-2.1 FDD.

Though it looks the test time above can be fit within the available test time (i.e. 250 sec.), since it is known that it takes approximately two minutes to attach the UE to the network, care must be taken that we don’t exceed the total test time.

Observation 3: Care must be taken that the total test time including a time to attach the UE to the network won’t exceed the available test time (i.e. 250 sec.).

The issue becomes noticeable when we look at the NB-IoT-NTN cases. Since k2 is 13 slots for IoT-NTN cases, 13 + 8 = 21 slots are necessary for the HARQ processing. In addition to the longer total k-offset than NR, since the HARQ process number is 1 for NB-IoT-NTN, it means even longer test time than NR NTN case is expected. Then referring to the test time definition table G.3.5-1 and G.3.5-2 in TS 36.521-4 [5] for PDSCH demodulation tests, at least 243, 393 or 600 seconds are expected, which means the test time exceeds the available test time for LEO-600. (Table G.3.5-1 and G.3.5-2 are also extracted in Appendix A at the end of this contribution.)

Observation 4: NB-IoT-NTN PDSCH demodulation test cases will exceed the available test time for LEO-600 under the assumption of a moving satellite.

There is also a WI to introduce the IoT NTN TDD mode. So, it is anticipated that the test time becomes significant when it comes to TDD.

Observation 5: It is anticipated that the test time for IoT NTN TDD mode will also exceeds the available test time for LEO-600 under the assumption of a moving satellite.

Proposal 1: Companies are encouraged to bring a solution of the test time issue in a case of NGSO LEO-600 cases under the moving satellite assumption.

- Conclusion

In this contribution we shared our findings on the test time to conduct PDSCH demodulation tests with NR NTN and NB-IoT NTN UEs.

Observation 1: Available measurement period for LEO-600 scenario is 250 seconds maximum under the assumption of the moving satellite with the elevation angle from 30 to 150 degrees

Observation 2: NR FDD PDSCH demodulation test cases for LEO-600 are expected to take a test time from 32 to 85 seconds with reference channels R.PDSCH.1-1.1 FDD or R.PDSCH.1-2.1 FDD.

Observation 3: Care must be taken that the total test time including a time to attach the UE to the network won’t exceed the available test time (i.e. 250 sec.).

Observation 4: NB-IoT-NTN PDSCH demodulation test cases will exceed the available test time for LEO-600 under the assumption of a moving satellite.

Observation 5: It is anticipated that the test time for IoT NTN TDD mode will also exceeds the available test time for LEO-600 under the assumption of a moving satellite.

Proposal 1: Companies are encouraged to bring a solution of the test time issue in a case of NGSO LEO-600 cases under the moving satellite assumption.

# 4.	References

[1] R4-2504700, “Way Forward for [114bis][323] NTN_testing_NGSO_channel_model_and_demod”, Moderator (Samsung), RAN4 #114-bis, Wuhan, China

[2] TS 38.521-5, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance”, v18.5.0, 2025-03

[3] R4-2506282, “Views on TE emulated channel model for satellite mobility”, Anritsu, RAN4 #115, St. Julian’s, Malta

[4] TS 38.101-5, "User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance", v18.9.0, 2025-03

[5] TS 38.521-4, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Performance requirements”, v18.6.0, 2025-03

Appendix A: Minimum test time for PDSCH demodulation test

TS 38.521-5 [2] Table G.1.5-1: Minimum Test time for PDSCH demodulation

| Test number | Reference Channel | Propagation condition | Minimum number of active subframes (MNAS) | MNAS to MNS Scaling factor (Note 3) | Minimum Number of Subframes (MNS) after rounding up to nearest thousand MNS= |
| --- | --- | --- | --- | --- | --- |
| 1-1 | R.PDSCH.1-1.1 FDD | NTN-TDLA100-200 | 30000 (Note 1) | 1.0526 | 32000 |
| 1-2 | R.PDSCH.1-2.1 FDD | NTN-TDLC5-200 | 72000 (Note 1) | 1.0526 | 76000 |
| 1-3 | R.PDSCH.1-1.1 FDD | NTN-TDLC5-200 | 80000 (Note 1) | 1.0526 | 85000 |
| 1-4 | R.PDSCH.1-1.1 FDD | NTN-TDLA100-200 | 30000 (Note 1) | 1.0526 | 32000 |
| Note 1: MNAS determined by simulations. Note 2: For cases where MNAS is not determined by simulations, use same MNAS as the similar case simulated (same doppler speed) Note 3: MNS/MNAS ratio decided by scheduling pattern and is ratio of all slots to DL slots. |  |  |  |  |  |

TS 36.521-4 [5] Table G.3.5-1: Minimum Test time for PDSCH for UE Category NB1

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | Min Gap between consecutive NPDCCH taking into account RTT (ms) | Actual Gap between consecutive NPDCCH taking into account RTT (ms) | MNSF (Min No Sub frames) HD-FDD (Note2) |
| --- | --- | --- | --- | --- | --- |
| 8.3.1.1.1 Test 1 | R.NB.1 FDD (200kHz, QPSK,1/2) (1x1) NTN TDLC5-200 | 845 (Note 1) | 241 | 288 | 243360 |
| 8.3.1.1.1 Test 2 | R.NB.2 FDD (200kHz, QPSK,1/3) (1x1) NTN TDLA100-10 | 523 (Note 1) | 881 | 1152 | 602496 |
| 8.3.1.1.2 Test 1 | R.NB.6 FDD (200kHz, QPSK,1/2) (1x1) EPA5 | 683 (Note 1) | 489 | 576 | 393408 |
| 8.3.1.1.2 Test 2 | R.NB.6-1 FDD (200kHz, QPSK,1/2) (1x1) ETU1 | 440 (Note 1) | 1982 | 2304 | 1013760 |
| Note1: Minimum test time has been derived by simulating the test scenario taking into account the minimum scheduling gap needed between successive NPDCCH/NPDSCH Note2: For MNSF calculations, with the current assumption of SIB2/SIB31 scheduling every 640ms with max 8 subframes and max 2 repetition per SI window, 16 additional subframes are needed every 640ms to avoid collisions with NPDSCH. |  |  |  |  |  |

36.521-4 [5] Table G.3.5-2: Minimum Test time for PDSCH for UE Category NB2

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | Min Gap between consecutive NPDCCH taking into account RTT (ms) | Actual Gap between consecutive NPDCCH taking into account RTT (ms) | MNSF (Min No Sub frames) HD-FDD (Note2) |
| --- | --- | --- | --- | --- | --- |
| 8.3.1.1.3 Test 1 | R.NB.7 FDD (200kHz, QPSK,1/2) (1x1) EPA5 | 600 | 283 | 288 | 599904 |
| Note1: Minimum test time has been derived by simulating the test scenario taking into account the minimum scheduling gap needed between successive NPDCCH/NPDSCH Note2: For MNSF calculations, with the current assumption of SIB2/SIB31 scheduling every 640ms with max 8 subframes and max 2 repetition per SI window, 16 additional subframes are needed every 640ms to avoid collisions with NPDSCH. |  |  |  |  |  |