3GPP TSG-RAN WG4 Meeting #115	R4-2506284
Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.14.3.3

Source:	CAICT

Title:	On Rel-19 Dynamic MIMO OTA

Document for:	Approval

1	Introduction

This paper provides our views on some remaining open issues on Rel-19 dynamic MIMO OTA.

2	Discussion

2.1 Remaining open issues on Rel-19 dynamic MIMO OTA

The core part WI is expected to be completed at RAN#109 (Sep. 2025). According to the WID [1] and the latest SR [2], the following issues on Rel-19 FR1 dynamic MIMO OTA are still open:

- Dynamic channel model validation target values

- Dynamic channel model validation pass/fail limits

- UE performance metric

- MU assessment for FR1 dynamic MIMO OTA (RAN5)

2.2 Dynamic channel model validation pass/fail limits

At the last meeting, an initial agreement on dynamic channel model validation pass/fail limits is reached, as captured in the WF [3]:

Issue 3-1-3: Dynamic channel model validation method Agreements: - Reuse the principle of Rel-17 FR1 static channel model validation pass/fail limits in dynamic channel model validation for PDP, TCF, SCF, and XPO.

To make it clearer, we quote the Rel-17 FR1 static channel model validation pass/fail limits from Clause C.4 of TS 38.151 v18.3:

For PDP: The detailed pass/fail limits for each cluster of CDL-C UMa and CDL-C UMi are defined in Table C.4.2-1. Table C.4.2-1: PDP pass/fail limits for CDL-C UMa and CDL-C UMi channel model validation Power Tolerance; Delay Tolerance. Paths from 0dB to 10dB; ±1dB; ±6ns. Paths from 10dB to 20dB; ±2.5dB; ±6ns. Paths from 20dB to 30dB; ±5dB; ±6ns. Paths from 30dB to 40dB; ±10dB; ±6ns. For TCF: The pass/fail limits for theoretical temporal correlation defined in Clause C.3.3 above 0.3 are formed as bands of ±0.1 of correlation capped at 1 at the high end. Additionally, when the theoretical temporal correlation drops below 0.3, the limits are formed at bands of ±0.3 of correlation capped at 0 at the low end. For SCF: The pass/fail limits for spatial correlation are formed as bands of ±10% of correlation capped at 100% for the upper limit for target correlation defined in clause C.3.4 of 35% (for CDL-C UMa @3600MHz, this value is 65%) and above. For target correlations below 35% (for CDL-C UMa @3600MHz, this value is 65%), the band is widened to ±20% capped at 0%. For XPO: The cross-polarization ratio pass/fail limit is specified as ±1dB.

After reviewing some conducted and OTA channel model validation results based on CTIA dynamic MIMO OTA test method [4, 5], we found that the XPO pass/fail limit defined in 3GPP TS 38.151, i.e. ±1dB, seems too tight for the dynamic channel model validation. We suggest to further discuss the pass/fail limit for XPO and check if ±2dB is reasonable. For PDP, TCF, and SCF, the pass/fail limits defined in 3GPP TS 38.151 are feasible for the dynamic channel model validation.

Proposal 1: Reuse the principle of Rel-17 FR1 static channel model validation pass/fail limits in dynamic channel model validation for PDP, TCF, SCF, i.e.,

- For PDP: Define the detailed pass/fail limits for each cluster of CDL-C UMadyn and CDL-C UMidyn as below.

|  | Power Tolerance | Delay Tolerance |
| --- | --- | --- |
| Paths from 0dB to 10dB | ±1dB | ±6ns |
| Paths from 10dB to 20dB | ±2.5dB | ±6ns |
| Paths from 20dB to 30dB | ±5dB | ±6ns |
| Paths from 30dB to 40dB | ±10dB | ±6ns |

- For TCF: The pass/fail limits for theoretical temporal correlation above 0.3 are formed as bands of ±0.1 of correlation capped at 1 at the high end. Additionally, when the theoretical temporal correlation drops below 0.3, the limits are formed at bands of ±0.3 of correlation capped at 0 at the low end.

- For SCF: The pass/fail limits for spatial correlation are formed as bands of ±10% of correlation capped at 100% for the upper limit for target correlation of 35% (for CDL-C UMa @3600MHz, this value is 65%) and above. For target correlations below 35% (for CDL-C UMa @3600MHz, this value is 65%), the band is widened to ±20% capped at 0%.

Proposal 2: For XPO, FFS the pass/fail limit and check if ±2dB is reasonable.

2.3 UE performance metric

The agreements on UE performance metric are captured in the WF of RAN4#114 meeting [6] as below:

Issue 1-2: Dynamic MIMO OTA test and UE performance metric Agreement: - All measured throughput values for different device orientations and rotations shall be combined to derive a single CDF curve for the dynamic MIMO OTA test. FFS whether the performance metric should be based on three (10%/50%/90%) percentiles at the CDF. - Use the following as a starting point: - include both UMa and UMi channels via post measurement data processing. FFS how to do the post data processing.

The OTA performance metrics are always expected to be able to reflect an overall performance of a UE in different positions/orientations/scenarios, taking average of metrics of different states is a customary method for defining OTA performance metrics including TRP, TRS, TRMS, MASC. Considering the probability of occurrence with the three (10%/50%/90%) percentiles at the CDF, different weights can be assigned to CDF10%TMT, CDF50%TMT, and CDF90%TMT. The weights for the three percentiles should be 10%:50%:10%=1:5:1.

Thus, it is recommended to define the UE performance metric for dynamic MIMO OTA as  114×[CDF10%TMTUMi+5×CDF50%TMTUMi+CDF90%TMTUMi+CDF10%TMTUMa+5×CDF50%TMTUMa+CDF90%(TMTUMa)].

Proposal 3: Consider defining the UE performance metric for dynamic MIMO OTA as  114×[CDF10%TMTUMi+5×CDF50%TMTUMi+CDF90%TMTUMi+CDF10%TMTUMa+5×CDF50%TMTUMa+CDF90%(TMTUMa)].

3	Conclusion

This paper provides our views on some remaining open issues on Rel-19 MIMO OTA.

Proposal 1: Reuse the principle of Rel-17 FR1 static channel model validation pass/fail limits in dynamic channel model validation for PDP, TCF, SCF, i.e.,

- For PDP: Define the detailed pass/fail limits for each cluster of CDL-C UMadyn and CDL-C UMidyn as below.

|  | Power Tolerance | Delay Tolerance |
| --- | --- | --- |
| Paths from 0dB to 10dB | ±1dB | ±6ns |
| Paths from 10dB to 20dB | ±2.5dB | ±6ns |
| Paths from 20dB to 30dB | ±5dB | ±6ns |
| Paths from 30dB to 40dB | ±10dB | ±6ns |

- For TCF: The pass/fail limits for theoretical temporal correlation above 0.3 are formed as bands of ±0.1 of correlation capped at 1 at the high end. Additionally, when the theoretical temporal correlation drops below 0.3, the limits are formed at bands of ±0.3 of correlation capped at 0 at the low end.

- For SCF: The pass/fail limits for spatial correlation are formed as bands of ±10% of correlation capped at 100% for the upper limit for target correlation of 35% (for CDL-C UMa @3600MHz, this value is 65%) and above. For target correlations below 35% (for CDL-C UMa @3600MHz, this value is 65%), the band is widened to ±20% capped at 0%.

Proposal 2: For XPO, FFS the pass/fail limit and check if ±2dB is reasonable.

Proposal 3: Consider defining the UE performance metric for dynamic MIMO OTA as  114×[CDF10%TMTUMi+5×CDF50%TMTUMi+CDF90%TMTUMi+CDF10%TMTUMa+5×CDF50%TMTUMa+CDF90%(TMTUMa)].

References

- RP-250197, Revised WID: WI on FR1 TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3, vivo, CAICT, RAN meeting #107, Mar 2025.

- RP-250196, SR for TRP_TRS_MIMO_OTA_Ph3, vivo, CAICT, RAN meeting #107, Mar 2025.

- R4-2504763, WF for [114bis][328] TRP_TRS_MIMO_OTA_Ph3, vivo, RAN4#114bis, Apr 2025.

- CTIA OTAMIMO_2025_002_005, Conducted Validation Results for PDP, TCF, SIR, and XPO

- CTIA OTAMIMO_2023_010_008, Preliminary Dynamic Channel Model Validation Results

- R4-2502296, WF for [114][328] MIMO_OTA_Ph3, CAICT, RAN4#114, Feb 2025.