3GPP TSG-RAN WG4 Meeting # 115						    		R4-2506290

St Julian’s, Malta, May 19th – 23rd, 2025

Title: Consideration of beam angle on beam gain degradation for minimum EIRP discussion

Agenda Item: 7.11.5

Source: Sharp

Document for: Discussion

# 1	Introduction

At the last meeting, RAN4 made the following agreement as captured in the WF [1].

- Consider the following as the starting point Operating band; UE Type; Min peak EIRP (dBm). [When communicating with LEO]; [When communicating with GSO]. n248, n247, n509, n508; 4; [64]; [70]. 5; [64]; [70]. - FFS on whether further clarification is needed

In this contribution we present our views on the values of Min Peak EIRP for LEO.

# 2	Discussions

As identified above, 64 dBm was agreed as the starting point for Min peak EIRP for LEO. However, a 64 dBm requirement will exclude some devices currently available on the market. For example, one VSAT device provides an EIRP of 33.6 dBW (i.e., 63.6 dBm) [2]. Another VSAT device in [3] shows 36.6 dBW for Dual Carrier operation, which implies approximately 33.6 dBW (i.e., 63.6 dBm) per carrier. Therefore, Min Peak EIRP requirement should be at least 63.6 dBm or lower.

Proposal 1: Min Peak EIRP requirement should at least be 63.6 dBm or lower.

Furthermore, even with 63.6 dBm Min Peak EIRP, there remains a concern regarding implementation flexibility for electrically steered VSATs as vendors need to consider beam gain reduction as related to beam angle. For example, VSAT vendors need to implement power amplifiers so that the Min Peak EIRP requirement is met for all the satellite angles including 50 ~ 60 degrees down from the zenith in which beam gain reduction is observed.

Table 1 shows EIRP for each satellite angle in our measurement setup [4], which yields beam reduction gain at lower satellite angles.

Table 1: Measurement results in our measurement setup [4]

| Angle | 0 degree (Zenith) | 50 degrees | 60 degrees |
| --- | --- | --- | --- |
| EIRP | 66.14 dBm | 63.69 dBm | 62.31 dBm |

To meet the Min Peak EIRP requirement for satellite angles of 50 ~ 60 degrees, VSAT vendors need to implement power amplifiers which have around 3.5 dB higher power. However, power amplifiers with higher powers will lead to higher implementation burden, particularly for VSAT vendors which develop thinner VSAT panels targeting implementation on e.g., the rooftop of vehicles. Therefore, it is suggested to consider beam gain reduction of around 3.5 dB for Min Peak EIRP.

Proposal 2: Beam gain reduction of around 3.5 dB of electronically steered VSAT is taken into account for Min Peak EIRP.

As already analyzed in [5], for LEO1200, required EIRP is 58.2 dBmfor SNR=-1 dB requirement with 100 MHz bandwidth. Therefore, 63.6 – 3.5 = 60.1 dBm Min Peak EIRP has no issues. By rounding, a value of 60 dBm could be proposed.

Proposal 3: 60 dBm as min peak EIRP for electronically steered VSAT connected to LEO.

As a consideration for future work, we would like to highlight that a reduction in antenna size results in greater beam gain degradation. Therefore, although the target in the Rel-19 Ku-band work is a single antenna size equivalent to a 60 cm diameter, it should be noted that the beam gain degradation becomes significantly larger as the antenna size decreases.

Observation 1: The beam gain degradation becomes significantly larger as the antenna size decreases.

# 3	Summary

Observation 1: The beam gain degradation becomes significantly larger as the antenna size decreases.

Proposal 1: Min Peak EIRP requirement should at least be 63.6 dBm or lower.

Proposal 2: Beam gain reduction of 3.5 dB of electronically steered VSAT is taken into account for Min Peak EIRP.

Proposal 3: 60 dBm as min peak EIRP for electronically steered VSAT connected to LEO.

# 4	References

R4-2504720, Way Forward for [114bis][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL), April 2025

https://athenatelecom.ae/wp-content/uploads/2023/08/OneWeb-Data-Sheets-Intellian-Athena.pdf

https://www.hughes.com/sites/hughes.com/files/2025-04/Hughes_LEO_HL1100W_Terminal-datasheet.pdf

https://global.sharp/corporate/news/240911-a.html

R4-2501338, Discussion on the support VSAT types and EIRP, TRP, EIS requirement for the Rel.19 NTN Ku band, CHTTL, February 2025