3GPP TSG-RAN WG4 Meeting # 115	R4-2506294

Malta, MT, 19th – 23rd May, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements of on-demand SSB Scell operation for Rel-19 NES

Agenda item:	7.27.3.1

Document for:	Discussion

Introduction

In agreed WID for Rel-19 NES operation [1], following objective was captured for RAN4 RRM core requirements:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we provide our views on RRM core requirements for RRM requirements of on-demand SSB Scell operation for Rel-19 NES.

Discussion

General On-demand SSB requirements

Scheduling restriction for L1 measurement after SCell activation

In previous meeting, L1 measurement has been discussed as follows:

Issue 1-4-6: Scheduling restriction for L1 measurement after SCell activation Wayforward - FFS In Case 2 L1-RSRP measurement, no scheduling restriction and measurement restriction are expected for AO-SSB occasions if UE only supports OD-SSB based L1-RSRP measurement.

In non-periodic time domain pattern in the union of AO-SSB transmission and OD-SSB transmission, a UE may prioritizes OD-SSB measurement. In this case, AO-SSB can be ignored. It means no scheduling restriction and measurement restircion for AO-SSB occasion.

Proposal 1: In case 2, at least for same frequency, no scheduling restriction and measurement restriction are expected for AO-SSB occasions if UE only supports OD-SSB based L1-RSRP measurement

Additional processing time for OD-SSB reception

In the previous meeting, RAN4 achieved following agreement and wayforward.

Agreement UE applies the additional processing timing only in deactivated SCell measurement. - Note: The additional UE processing timing is not applied to SCell activation procedure. The additional processing timing is T_processing, the value of T_processing: - Introduce different values with UE capability: [2 or 3]ms and 5ms. Issue 1-4-3: Additional Processing time for OD-SSB deactivation Wayforward - FFS RAN4 to consider defining the same timeline for activating and deactivating OD-SSB

In the meanwhile, RAN1 sent following LS[4] for RAN4 confirmation.

RAN1 has made the following agreement for MAC-CE based deactivation of on-demand SSB (OD-SSB) transmission on SCell. Agreement - Upon the reception of MAC CE for deactivating on-demand SSB, UE expects that on-demand SSB is NOT transmitted from time instance B. - Time instance B’ is T slots after the slot where UE receives a signalling from gNB to deactivate on-demand SSB. If time instance B’ falls within an on-demand SSB burst, time instance B is the ending of the slot containing the last actually transmitted SSB index within the on-demand SSB burst, otherwise, time instance B = time instance B’. - As agreed in previous RAN1 meeting, the SSB time domain positions of on-demand SSB burst are configured by gNB. - As agreed in previous RAN1 meeting, the location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - (Working assumption) T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to deactivate on-demand SSB ending in slot n, and Nslotsubframe,μ is as defined in current specification. - (Working assumption) T=T_min - RAN4 to confirm the above two working assumptions - Nslotsubframe,μ is based on the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication RAN1 respectfully asks RAN4 if the two working assumptions can be confirmed to agreement.

In last meeting, a company raised an issue regarding the processing time for deactivating OD-SSB. Main argument is that if the timeline of processing time of deactivating OD-SSB is short, the UE may try to process OD-SSB that is not sent by the NW anymore. In our view, LS[4] from RAN1 can resolve the concern. In RAN1#120bis meeting, RAN1 introduced ‘Time instance B’, which is similar to ‘Time instance A’, but it indicates the ending slot for OD-SSB. Time instance B includes

MAC-CE processing time, e.g. T_min=m+3Nslotsubframe,μ+1. It is clear that the UE is not expected to measure OD-SSB after Time instance B. It can avoid the UE to measure the OD-SSB which is not presented.

Proposal 2: Confirm T_min=m+3Nslotsubframe,μ+1 for Time instance B

During the discussion for processing time for OD-SSB activation, RAN4 deceided to introduce additional processing time to reflecting the delay for searcher occupation or activating RF ciruit. On the other hand, regarding OD-SSB deactivation, additional processing time is not needed, because the UE can just stop measuring after Time instance B. So we don’t think additional processing time is needed for deactivating OD-SSB.

Proposal 3: Additional processing time for OD-SSB deactivation is not required

Figure 1 Illustration of Time instance B

Interruption

Agreement - New interruption is allowed for scenario 2 due to OD-SSB activation/deactivation. - New interruption is also allowed for scenario 2 at the end of the FMW if UE does not receive SCell activation command. - The interruption can be applied in both Case 1 and Case 2. - The detail condition, duration and position of the interruption is FFS. - The interruption is allowed for scenario 2a due to OD-SSB activation/deactivation. - No new interruption requirement is defined. - The interruption requirement follows the same principle of legacy SCell activation.

In the previous meeting, RAN4 agreed to introduce new interruption requirement for OD-SSB. Details will be disussed in the upcoming meeting. In general the principle of legacy requirement can be reused.

For example, the principle of SCell activation/deactivation can be reused. Regarding OD-SSB activation, the starting point of an interruption window on SpCell or any activated SCell shall not occur before slot n+1+HARQ/NR_slot_length and not accur after slot n+1+ (HARQ+3ms+Time_instance_A+additional_ processing_ time)/NR_slot_length. Regarding OD-SSB deactivation, the starting point of an interruption window on SpCell or any activated SCell shall not occur before slot n+1+HARQ/NR_slot_length and not accur after slot n+1+ (HARQ+3ms) /NR_slot_length.

Regarding interruption at the end of the FMW, interruption requirement can follow the interruptions during measurements on deactivated SCC. Interruption can be allowed with up to 0.5 % probability of missed ACK/NACK when the configured measCycleSCell is 640 ms or longer. The UE is allowed to cause interruptions on PCell or activated SCell(s) immediately after the end of FMW.

Propsoal 4: RAN4 to reuse the principle of interruption requirement for the new OD-SSB interruption as follows:

    - Interruption due to OD-SSB activation/deactivation: reuse the principle of interruption at SCell activation/deactivation

    - Interruption at the end of the FMW: reuse the principle of interruptions during measurements on deactivated SCC

RRM requirement impact for Scenario #3B

Agreement(RAN1#120bis) For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE. - Scenario #3B and Case #1 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. - Scenario #3B and Case #2 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

According to the agreement, RAN1 introduced OD-SSB activation in Scenario #3B, which is the time after SCell activation is completed. The MAC-CE for OD-SSB activation is used only for updating the transmission parameter of a transmitted OD-SSB for the cell. RAN4 need to discuss L1/L3 measurement requirement impact for Scenario#3B.

Proposal 5: RAN4 to identify L1/L3 measurement requirement impact for Scenario#3B

On-demand based deactivated SCell L3 requirements (Case 1)

In RAN4#112bis, RAN4 introduced the time window in which a UE is required to measure OD-SSB. The motivation of the time window is to reduce the UE power consumption, since NW may not guarantee OD-SSB deactivation. According to the agreement, the UE performs ‘fast L3 measurement’ during the time window, otherwise ‘slow L3 measurement’ is expected. In RAN4#113, slow L3 measurement was defined as using legacy deactivated SCell measurement, which means that the UE shall meet the requirements using measCycleSCell for measurement periodicity. In RAN4#114, the length of fast measurement window was defined. In RAN4#114bis, there was discussion regarding the starting point of T2, however, agreement was not achieved.

In this meeting, the measurement requirement based on interval between multiple FMW (T2), interval between for cell search, and the lower bound for fast measurement requirement are still open issue. We would like to touch upon one by one.

OD-SSB based deactivated SCell measurement

In previous meeting, following agreement regarding minimum interval between multiple fast measurement window has been achieved:

Sub-issue 1: The start point of T2 Wayforward The start point of T2 is - Option 1: - The time point of OD-SSB deactivation - Option 2: - The time point of Tend, i.e. the time point of OD-SSB activation plus Tuncertain +Tidentify. - Option 2a: - The time point of OD-SSB activation Sub-issue 2: The minimum interval(T2) between multiple FMWs Wayforward - Option 1: 5* measCycleSCell - Option 1a: 5*(max(measCycleSCell, DRX cycle) – OD-SSB periodicity) - Option 1b: 5*max (measCycleSCell, DRX cycles) - Option 2: Minimum measurement period defined in existing spec. - Option 3: The time period of known Scell condition but with OD-SSB periodicity for FR1 Sub-issue 3 –UE’s behaviour when OD-SSB further activation/reconfiguration within T2 Wayforward - Option 1: The UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement. - Option 2: The UE is assumed to perform fast mode measurement without cell identification period. - Option 3: No requirement for UE within T2.

Before going into sub issue, we would like to point out that introducing minimum interval between multiple FMW does not mean that minimum interval for cell search is precluded. We believe that restricting multiple FMWs is one approach, while cell search is another. That is because purpose of the timline is different. Suggest to discuss separately.

Proposal 6: Discuss minimum interval between FMW and minimum interval for cell search separately

Details of minimum interval between FMW is discussed as follows:

Sub-issue 1: The start point of T2

For the starting porint of minimum interval between multiple FMWs, there are three options are on the table. It can be analyzed as below:

- Option 1: the time point of OD-SSB deactivation

  - Pros: Clear starting point

  - Cons: UE behaviour is not clear before OD-SSB deactivation, restrictive scheduling

- Option 2: The time point of Tend, i.e. the time point of OD-SSB activation plus Tuncertain +Tidentify.

  - Pros: Clear starting point, aligned with motivation

  - Cons: UE behaviour is not clear before Tend, Tend should be calculated by UE

- Option 2a: The time point of OD-SSB activation

  - Pros: Clear starting point, and UE behaviour

  - Cons: Length of T2 should be longer than the length of FMW

From above analysis, it seems that the UE behaviour before T2 is not clear. Note that RAN4 has already agreed that the UE performs the OD-SSB based fast L3 measurement within FMW when NW re-activates/re-configures the OD-SSB transmission after T2. For simplicity there is no reason to differentiate the UE behaviour between before and after T2. Hence, we think same behaviour can be applied before T2.

Regarding the starting point of T2, we think all three options have its benefits. For Option 1, if NW wants to re-initiate FMW, OD-SSB deactivation followed by T2 and OD-SSB activation is required. The procedure is complicate than the other options. For Option 2, and 2a, NW can re-activate OD-SSB after T2, if NW wants to re-initate FMW. In the end, the only difference is procedure when it comes to re-initiating FMW. In that sense, Option2 is less complicated and reasonable choice for the starting point of T2.

Sub-issue 2: The minimum interval(T2) between multiple FMWs

For the minimum interval(T2) between multiple FMWs, we prefer to have similar time window as in Case 2 for simplicity.

Sub-issue 3 –UE’s behaviour when OD-SSB further activation/reconfiguration within T2

As we commented earlier, both Option 1 and Option 2 can be supported. Requirements can be defined separately.

Sub-issue 4 – Minimum interval for cell search

Unlike minimum interval for multiple FMWs, for minimum interval for cell search, we believe it is straight forward that the starting point of T2 for cell search begins at the OD-SSB deactivation. As PSS/SSS can be detected during OD-SSB activation, no cell search is required when OD-SSB is activated. However, after OD-SSB deactivation, if the time is long enough to lose the sync (e.g. 5s in the current spec), the UE may need to detect PSS/SSS and/or acquire the index of the SSB being measured.

Figure 3 OD-SSB transmission with T2 in Case1

Proposal 7: RAN4 to specify T2 as follows:

- Minimum interval between FMW

  - Starting point

    - The time point of Tend, i.e. the time point of OD-SSB activation plus Tuncertain +Tidentify

  - Minimum interval:

    - 5*max (measCycleSCell, DRX cycles)

  - UE Behaviour:

    - The UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement

- Minimum interval for cell search

  - Starting point:

    - The time point of OD-SSB deactivation

  - Minimum interval:

    - 5s

  - UE Behaviour:

    - The UE is assumed to perform fast mode measurement without cell identification period

Lower bound for measurement in FMW

In the pervious meeting, following wayforward was captured in [2].

Sub-issue 1: Lower bound for measurement in FMW Agreement Introduce UE capability on whether a lower bound is needed, i.e., - The lower bound is not needed (default value), or, - A lower bound is needed and based on OD-SSB periodicity of [10] ms - Further confirm whether [10] is feasible, and only one value

In practice, SSB periodicity with 20ms is used. In addition, 10ms may be useful for fast measurement. Therefore, 10ms is reasonable for the UE which requires lower bound for measurement in FMW.

Proposal 8: For lower bound for measurement in FMW, confirm 10ms

OD-SSB in FMW collided with MG

Sub-issue 3: OD-SSB in FMW collided with MG Wayforward - FFS UE is allowed to skip the OD-SSB measurement within FMW if the OD-SSB occasion in FMW collides with MG occasion.

In the current specification, Kp, which is the scaling factor for an SSB frequency layer to be measured without GAP, is defined. The principle is to skip the SSB measurement, if SMTC is partially overlapping with MG occasion. The principle can be reused for OD-SSB measurement within FMW.

Proposal 9: The principle of Kp (the scaling factor for an SSB frequency layer to be measured without GAP) can be reused

On-demand based deactivated SCell L3 requirements (Case 2)

Scenario for On-demand SSB requirements in Case 2

In the previous meeting, RAN4 introduced Scenario 4 for performing combined measurement of AO-SSB and OD-SSB. According to agreement, at least combined measurement can be performed based on the effective measurement periodicity within FMW (fast measurement window) in deactivated SCell L3 measurement. Meanwhile, RAN1 agreed to send reply LS[3] as follows:

Agreement(RAN1#114) Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, - For the case when the center frequency locations of always-on SSB and on-demand SSB are same, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - From RAN1 perspective, - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2 - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB - For the case when the center frequency locations of always-on SSB and on-demand SSB are different, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain. - AO-SSB and OD-SSB are located in the same BWP - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB - NOTE: AO-SSB periodicity is not adapted

In RAN4#114bis, following agreement has been achieved:

Issue 1-2-2: Requirement for RAN1 Alt Time-C1 Agreement: - RAN4 to update the agreement of EMP for deactivated SCell measurement and SCell activation to apply to RAN1 Alt Time-C1 scenario. - The EMP is the periodicity of the union of AO-SSB and OD-SSB and equals one of legacy SSB periodicities defined in the specification. - FFS for other procedures. Issue 1-2-3: Requirement for RAN1 Alt Time-C2 Agreement - Option 1: Define the requirement based on OD-SSB periodicity. - Option 2: Not to define requirement. Issue 1-2-4: Requirement for scenario 3 (OD-SSB and always-on SSB are within different frequencies) Agreement RAN4 to first discuss the RAN4 impact based on RAN2 LS R2-2503015. - Option 1: RAN4 to define requirement for different frequencies Option 2: RAN4 not to define requirements for different frequencies

In RAN2#129bis, folliwng LS[5] was sent.

In RAN2#129b, RAN2 discussed how to configure L3 measurement when always-on SSB (A-SSB) and on-demand SSB (OD-SSB) have different center frequency, and made the following Working Assumption: Working assumption: - When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. Note: The WA was made from RRC signaling perspective. RAN2 relies on RAN4/1 to decide whether to support the scenario when A-SSB and OD-SSB have different center frequencies or not.

Requirement for RAN1 Alt Time-C1

In our understanding, applying combination of AO-SSB and OD-SSB can be extend to L1 measurement after SCell activation. It would be helpful to reduce L1 measurement period in Case 2.

Proposal 10: Define L1 measurement in Sceanrio 4

Requirement for RAN1 Alt Time-C2

According to LS[3], RAN1 defined non-periodic time domain pattern which is Time-C2. In the last meeting, Time-C2 was discussed, and majority of company thinks no requirements or OD-SSB periodicity based requirements is needed. The argument is that non-periodic time domain pattern is complex to be captured in the requirement. We agree that it is difficult to capture every possible case available in Time-C2 for the requirement. However, we believe some limited cases can be specified with simplicity, e.g. when the period AO-SSB and OD-SSB are the same.

In Figure 5, example of Time-C2 when the periodicity of AO-SSB and OD-SSB is illustrated. Figure 5 shows that the requirement is based on OD-SSB periodicity assuming two measurement samples are included in a single period. The red box shows the measurement period based on OD-SSB periodicity. During the measurement period, 6 samples are located, and a UE can choose to measure 5 samples. Which one is to measure is based on UE implementation. The measurement period can be easily derived.

  - Ceil(5ⅹKp/2)ⅹOD-SSB-periodⅹCSSFintra

,which reflects that the UE can measure two measurement samples (AO-SSB+OD-SSB) during a one OD-SSB measurement period.

In conclusion, we think it is not complex to define measurements for non-periodic Time-C2. So, we suggest RAN4 to specify Time-C2 at least some limited cases.

Figure 5 Example of Time-C2 when the periodicity of AO-SSB and OD-SSB are the same

Proposal 11: RAN4 to specify requirements for non-periodic time domain pattern Time-C2 at least when periodicity of AO-SSB and OD-SSB are same.

  - For instance, measurement period can be[ Ceil(5ⅹKp/2)ⅹOD-SSB-periodⅹCSSFintra]

Requirement for scenario 3 (OD-SSB and always-on SSB are within different frequencies)

For the case when the center frequency loactions of AO-SSB and OD-SSB are different, RAN1 defined the case when AO-SSB is CD-SSB, and AO-SSB/OD-SSB are within the same BWP. Meanwhile, RAN2 agreed Working assumption[5] on introducing a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. In our understanding, combining the measurement of AO-SSB and OD-SSB with different frequency is not considered in RAN4. If the UE need to measure AO-SSB and OD-SSB separately, it is natural to have different MO, respectively. Thus, RAN4 can confirm the WA.

In case of activated SCell L3 measurement, since AO-SSB and OD-SSB are within the same BWP, a UE can measure both SSB even if they are overlapped. On the other hand, in case of deactivated SCell L3 measurement, BWP is not activated yet, and RF chain bandwidth is only available for SSB bandwidth, which lead the UE to measure AO-SSB or OD-SSB in some cases. According to LS[5] from RAN2, AO-SSB and OD-SSB can have different MO. In general, the UE is required to perform L3 measurement based on MO. Hence, certain condition should be defined to determine whether to peforme measurements for both AO-SSB and OD-SSB. In case when measuring both AO-SSB and OD-SSB is not possible, it is natural to prioritize OD-SSB measurements. The question is how do we determine the case? We think certain time offset between AO-SSB and OD-SSB for guaranteeing RF retuning time can resolve the issue. For instance, if the time offset between AO-SSB and OD-SSB is shorter than certain threshold, prioritize OD-SSB measurement, otherwise AO-SSB and OD-SSB can be measured independently.

Proposal 12: RAN4 to confirm WA from RAN2

Proposal 13: RAN4 to define requirement for different frequencies with OD-SSB measurements prioritization:

  - RAN4 to discuss how to prioritize OD-SSB measurements, e.g.

    - If the time offset between AO-SSB and OD-SSB is shorter than certain threshold, prioritize OD-SSB measurement, otherwise AO-SSB and OD-SSB can be measured independently

On-demand based SCell activation

Following Wayforward have been achieved:

Issue 1-3-2: The known cell condition – period of known condition - Wayforward Reuse the legacy known/unknown cell condition period for FR1 and FR2, i.e. max(5*measCycleSCell, 5*DRX cycles) in FR1 or 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 in FR2.

Known/unknown cell condition period

In the previous meeting, the principle of known cell and unkown cell using legacy principle has been agreed. In our understanding, as speficied in 38.133 - 8.3.2, Scell in FR1 is considered as ‘known’ when the UE has sent a valid measurement report for the SCell being activated, and the SSB measured remains detectable during the period max(5*measCycleSCell, 5*DRX cycles) before the reception of the SCell activation command. For SCell in FR2 is considered as ‘known’ when the UE has sent a valid L3-RSRP measurement report with SSB index, and SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation during the period equal to 4s or 3s before UE receives the last activation command, which depends on the UE power class. Otherwise, the SCell is considered to be unknown.

One issue to be resolved is whether to use legacy known/unknown cell condition period. As defined in the legacy requirement, measCycleSCell is applied in the legacy known/unknown condition. We think there is not much motivation to introduce new period for known/unown cell condition.

Proposal 14: RAN4 to specify known/unknown cell condition period as below:

- At least for FR1: max(5*( measCycleSCell), 5*DRX cycles)

Conclusion

In this contribution, we provide views on RRM requireiments for on-demand SSB SCell operation, and we propose

Proposal 1: In case 2, at least for same frequency, no scheduling restriction and measurement restriction are expected for AO-SSB occasions if UE only supports OD-SSB based L1-RSRP measurement

Proposal 2: Confirm T_min=m+3Nslotsubframe,μ+1 for Time instance B

Proposal 3: Additional processing time for OD-SSB deactivation is not required

Propsoal 4: RAN4 to reuse the principle of interruption requirement for the new OD-SSB interruption as follows:

    - Interruption due to OD-SSB activation/deactivation: reuse the principle of interruption at SCell activation/deactivation

    - Interruption at the end of the FMW: reuse the principle of interruptions during measurements on deactivated SCC

Proposal 5: RAN4 to identify L1/L3 measurement requirement impact for Scenario#3B

Proposal 6: Discuss minimum interval between FMW and minimum interval for cell search separately

Proposal 7: RAN4 to specify T2 as follows:

- Minimum interval between FMW

  - Starting point

    - The time point of Tend, i.e. the time point of OD-SSB activation plus Tuncertain +Tidentify

  - Minimum interval:

    - 5*max (measCycleSCell, DRX cycles)

  - UE Behaviour:

    - The UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement

- Minimum interval for cell search

  - Starting point:

    - The time point of OD-SSB deactivation

  - Minimum interval:

    - 5s

  - UE Behaviour:

    - The UE is assumed to perform fast mode measurement without cell identification period

Proposal 8: For lower bound for measurement in FMW, confirm 10ms

Proposal 9: The principle of Kp (the scaling factor for an SSB frequency layer to be measured without GAP) can be reused

Proposal 10: Define L1 measurement in Sceanrio 4

Proposal 11: RAN4 to specify requirements for non-periodic time domain pattern Time-C2 at least when periodicity of AO-SSB and OD-SSB are same.

  - For instance, measurement period can be[ Ceil(5ⅹKp/2)ⅹOD-SSB-periodⅹCSSFintra]

Proposal 12: RAN4 to confirm WA from RAN2

Proposal 13: RAN4 to define requirement for different frequencies with OD-SSB measurements prioritization:

  - RAN4 to discuss how to prioritize OD-SSB measurements, e.g.

    - If the time offset between AO-SSB and OD-SSB is shorter than certain threshold, prioritize OD-SSB measurement, otherwise AO-SSB and OD-SSB can be measured independently

Proposal 14: RAN4 to specify known/unknown cell condition period as below:

- At least for FR1: max(5*( measCycleSCell), 5*DRX cycles)

Reference

[1] RP-242354, “Revised WID: Enhancements of network energy savings for NR,” Ericsson

[2] R4-2504906, “WF on RRM requirements for Netw_Energy_NR_enh_Part1,” Ericsson

[3] R1-2501633,” LS on time location of on-demand SSB for SCell,” LG Electronics

[4] R1-2503108,” LS to RAN4 on time instance B for on-demand SSB SCell operation,” LG Electronics

[5] R2-2503019,” LS on on-demand SSB,” Apple