3GPP TSG-RAN WG4 Meeting # 115	R4-2506295

Malta, MT, 19th – 23rd May, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements of SSB adaptation for Rel-19 NES

Agenda item:	7.27.3.2

Document for:	Discussion

Introduction

In agreed WID for Rel-19 NES operation [1], following objective was captured for RAN4 RRM core requirements:

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we provide our views on RRM core requirements for RRM requirements of adaptation of SSB for Rel-19 NES.

Discussion

In the previous meeting, following agreement was captured in WF[2].

Sub-topic 1-1 SSB adaptation – Scenario Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement(RAN4#112bis): - Per RAN1 agreements, RAN4 to take following as starting point for SSB adaption discussion: - CONNECTED mode, and - SSB adaptation for SCell, and - SSB adaptation for NCD-SSB, and - SSB periodicity adaptation in time-domain Note: Above could be further updated based on further RAN1 progress (e.g. PCell, other adaptation mechanism) Agreement(RAN4#113): - No RRM impacts for SSB adaptation requirement for IDLE mode. - No RRM impacts for Cell DTX. Agreement(RAN4#114): - Further clarification on the scope of RRM impact for SSB adaption: - No RRM impacts for SSB adaptation requirement for PCell: - Frequency location is same before and after SSB adaptation. - No change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation - Note: Above could be further updated based on further RAN1 progress if any. Agreement(RAN4#114bis): - Further clarification on the scope of RRM impact for SSB adaption: - Existing SSB burst periodicity (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms). - SSB adaptation in SCell for NCD-SSB. - Time domain positions of SSBs within a burst is same before and after SSB adaptation - Note: Above could be further updated based on further RAN1 progress if any.

In previous meeting, RAN4 had further clarification on the scope of RRM impact for SSB adaptation. If there is no further progress in other WG, possible scenario can be as follows:

- CONNECTED mode

- SSB adaptation for SCell

- SSB adaptation for NCD-SSB

- SSB adaptation in time-domain only for SSB periodicity

- Frequency location is same before and after SSB adaptation

- SSB index is same before and after SSB adaptation

- Existing SSB burst periodicity (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms).

L1 requirements for SSB adaptation

Following is the agreement for L1 measurement achevied in last two meetings.

Issue 1-2-1: L1 requirements at transition Agreement - RAN4 FFS how to define Tfirst_SSB considering the processing/preparation time based on further RAN1/2 progress on SSB adaptation command timeline. - No impacts on section 9.5.3 of L1 reporting requirements due to SSB adaptation, unless identified by other WGs. Working Assumption(RAN1#120bis) When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first [actually] transmitted SSB within the first [possible] SSB burst according to the indicated SSB burst periodicity that is no earlier than the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell [in Table 11.5-1 of TS 38.213]. - FFS: how to determine the first [possible] SSB burst

Regarding the processing/preparation time for Tfirst_SSB, RAN1 achieved working assumption, which reuses cell DTX/DRX operation time, but FFS for the first possible SSB burst. In order to avoid duplicate discussion, we suggest to wait for more RAN1 progress.

Proposal 1: RAN4 to wait RAN1 progress for define Tfirst_SSB in L1 requirements at transition

L3 requirements for SSB adaptation

Issue 1-3-1: L3 requirements for SSB adaptation – general Agreement:(RAN4#114) Deactivated SCell measurement: - Option 1: For SSB adaption, reuse the same conclusion from OD-SSB case#2 (scenario#4) discussion to define RRM requirements. - At least for the case when the SSB periodicity is adapted to be shorter than before - Option 2: Same as legacy - Other options are not precluded. The requirement can be updated based on further progress in other WG or other aspects are identified that the conclusion can not be directly reused. In RAN2#129, reply LS[3] RAN2 thanks RAN4 for the LS in R4-2416911 on SSB adaptation. Regarding the question of SMTC impacts due to SSB adaptation, RAN2 made the following agreements: Agreements on SSB adaptation - RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation. - For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC. Issue 1-3-1: L3 requirements for SSB adaptation Agreement(RAN4#114bis): - RAN4 to define requirements based legacy L3 measurement framework for SSB adaptation. - For neighbour cell L3 measurement, legacy requirements apply. Open issue for further discuss: - For SCell with SSB adaptation (deactivated SCell or activated SCell), wait for more RAN2 progress on additional SMTC configuration, and discuss following aspects: - Whether to define L3 measurement requirement at transition. - Any other RRM requirements impacts due to additional SMTC configurations. In RAN2#129bis, Agreements on common channel adaptation - For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17. - For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17. - Paging clustering/bundling/adaptation is not supported/applied in RRC_CONNECTED. - Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1. - For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration. - Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

Deactivated SCell L3 requirements for SSB adaptation

In the RAN4#114, reusing the same requirements of OD-SSB Case#2 for deactivated SCell L3 requirements for SSB adaptation has been discussed. For SSB adaptation, RAN1 introduced DCI signaling to change SSB periodicity, which is different from OD-SSB operation. Furthermore, in our understanding, SSB dapatation includes both transition of shorter to longer, and longer to shorter. Hence, OD-SSB Case#2 cannot be directly used. Resusing the same requirement seems reasonable.

Proposal 2: For deactivated SCell L3 requirements for SSB adaptation, reuse legacy requirements.

Impacts due to additional SMTC/SMTC change

In reply LS[3], RAN2 decide to keep SMTC based L3 RRM framework, and to introduce additional SMTC configuration according to SSB adaptation. In RAN2#129bis, additional SMTC configurations are condifured and mapped to SSB burst periodicity in RRC. Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly. From the agreement, it can be concludes that SMTC periodicity can be switched dynamically. From RAN4 point of view, requirements apply based on the SMTC periodicity after transition for SMTC adaptation of periodicity. Therefore, Intra-freuqency without gap requriments can be reused.

Unlike intra-freuqency without gap requirements, particulary for intra-frequency without gap requirement, there can be an issue with measurement gap. In general, intra-frequency measurement with gap is applied to enable UE to perform intra-frequency measurement when the SSB to be measured is not within the active bandwidth part. To measure within measurement gap, current requirement take maximum of MGRP and SMTC periodicity. However, when SSB adaptation is indicated, the periodicity of SSB or SMTC can be reduced, which may lead to partially overlapped with measurement gap. In consequence, since a UE may not find SSB to be measured outside of measurement gap, the UE may not leverage the frequent adapted SSB in this situation. It is observed some modification regarding measurement gap is required, otherwise switching SMTC periodicity is not reflected in intra-frequency with gap requriements.

Figure 1 Intra-frequency measurement with gap for SSB adpatation

Regarding inter-frequency measurement, RAN2 concludes that the adapted SSB on neighbor cell is measured based on legacy SMTC, thus, reusing legacy requirement is enough.

Proposal 3:  For SCell with SSB adaptation L3 requirements,

- No RRM requirement impacts:

  - Intra-frequency without gap

  - Inter-frequency

- Define requirements if SSB adaptation happens:

  - Intra-frequency with gaps

L3 measurement requirement at transition

For the L3 transition requirements for SSB adaptation, we prefer to reuse legacy requirement, since there are no reason not to have different requirement. As captured in 9.1.6 as below, usually ‘requirements with the longer delay apply’ is used.

9.1.6 Minimum requirement at transitions When the measurement on one intra-frequency measurement object transitions from measurements performed outside gaps to measurements performed within gaps or vice versa during one measurement period, the cell identification and measurement period requirements with the longer delay apply. When the measurement on one intra-frequency measurement object transitions from measurements performed outside gaps to measurements performed within NCSG or vice versa during one measurement period, the cell identification and measurement period requirements with the longer delay apply.

Proposal 4: For L3 measurement period, requirements with longer delay is applied corresponding to the SSB before and after adaptation

Conclusion

In this contribution, we provide views on RRM requireiments for SSB adaptation, and we propose

Proposal 1: RAN4 to wait RAN1 progress for define Tfirst_SSB in L1 requirements at transition

Proposal 2: For deactivated SCell L3 requirements for SSB adaptation, reuse legacy requirements.

Proposal 3:  For SCell with SSB adaptation L3 requirements,

- No RRM requirement impacts:

  - Intra-frequency without gap

  - Inter-frequency

- Define requirements if SSB adaptation happens:

  - Intra-frequency with gaps

Proposal 4: For L3 measurement period, requirements with longer delay is applied corresponding to the SSB before and after adaptation

Reference

[1] RP-242354, “Revised WID: Enhancements of network energy savings for NR,” Ericsson

[2] R4-2502689, “WF on RRM requirements for Netw_Energy_NR_enh_Part2,” Huawei

[3] R2-2501484, “Reply LS on SSB adaptation”, Apple