3GPP TSG-RAN WG4 Meeting # 115	R4- 2506306

# Malta, MT, 19th – 23rd May, 2025

Source:	LG Electronics

Title:	Discussion of RAN4 impacts for RRM measurement prediction

Agenda item:	7.23.3.1

Document for:	Discussion

Introduction

In this contribution, we provide our views on issues for AI/ML mobility as below based on last RAN4 meeting’s moderator summary [2] and WF [1].

- Issue 2-1-1 Relative RSRP accuracy definition

- Issue 2-1-7 Impacts of measurement error on prediction accuracy

- Issue 3-1-2 Performance metric for event prediction use case

- Issue 4-1-3 Multiple cells in the testing setup

Discussion

- Topic #2: RAN4 requirements

Sub-topic 2-1: General and Scope

### Issue 2-1-1: Relative RSRP accuracy definition

During last RAN4#114bis meeting the relative RSRP prediction accuracy issue was discussed and made an agreement as below.

Agreement: - For intra-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell. - FFS if this definition will be needed. - For inter-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference. - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, [if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2]. - Depending on the agreement on ground truth definition, we may revisit relative RSRP accuracy definition. - FFS: whether the reported RSRP of cell2 can be predicted or measured or a combination of both predicted and measured samples. - FFS, if there is any impact on this definition for different scenarios/different frequency ranges.

Regarding intra-frequency L3-RSRP, the beam level predicted relative L3-RSRP is not necessary. Because, for mobility purpose, the required metric is cell level L3-RSRP. We think the beam level predicted relative L1-RSRP could be necessary to verify the performance of the AI/ML model, but it can be covered by cell level predicted relative L3-RSRP performance.

Proposal 1: The beam-level relative L3-RSRP accuracy requirement is not necessary for intra-frequency L3-RSRP.

Regaring inter-frequency L3-RSRP, our preference is predicted value for the reported RSRP of cell2 for predicted relative L3-RSRP accuracy requirement. Because the statistic performance can be different between predicted values and measured values. For example mean and variance can be different. So, to check the predicted relative RSRP accuracy, the reported RSRP of cell2 should be predicted value.

Proposal 2: Regaring inter-frequency L3-RSRP, for the predicted relative RSRP accuracy requirement, the reported RSRP of cell2 should be predicted value.

### Issue 2-1-5: Ground truth definition for RSRP Accuracy for FR1

Following agreement was reached in RAN4#114 and #114bis meeting.

RAN4#114 meeting - Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list RAN4#114bis meeting Agreement: During the SI, RAN4 to provide analysis on potential issue and possible solution to the two Alternatives. If RAN4 cannot make decision during the SI phase, RAN4 will discuss in the WI phase. - At least capture the definition of the two Alternatives in the TR.

During last RAN4#114bis meeting, many companies showed their views regarding two alternaitves. Among them, we have similar view with Nokia’s proposal as below:

|  | Issues | Impact to Alternative 1 | Impact to Alternative 2 |
| --- | --- | --- | --- |
| 1. | The information at TE side, on the timing UE performs and/ or finishes the measurement and/or prediction | TE may not be aware of the exact time period of the prediction performed at UE. This may lead to inaccuracies of ground truth derivation at TE due to dynamic channel conditions | NO |
| 2. | Impact of L3 filtering | L3 filtering methods, filtering co-efficient are configurable parameters, hence known to TE that can be utilized for deriving the ground truth. However, this may increase the complexity at TE when compared to legacy test setup. | NO |
| 3. | Number of samples of L1 filtering | The L1 filtering method, filtering co-efficient and the number of samples for filtering are not known by the TE since it is UE proprietary implementation, hence it will impact the derivation of ground truth at TE. | NO |
| 4. | Timing window, if it exists, where UE can’t measure the ground truth in Alt2 | NO | Depending upon the UE capabilities, some UEs may not be able to measure and predict at the same time. However, this can be handled during testing as described above under Alternative 2 description. |
| 5. | Channel condition | With fading channel, the dynamic channel variations in addition to L1/L3 filtering will have to be considered at TE for deriving the ground truth which may not be feasible due to proprietary algorithms used at UE. | NO |
| 6. | Avoid UE cheat in the test | NO | UE cheating can be an issue in this alternative |
| 7. | S(I)NR variations in the test at different time instances (for Alt2) | NO | One option is to measure at higher SINR level for more accurate ground truth and the SINR may be lower during the prediction with reference to the average SINR. |

Based on the analysis, our preference is alternative 2. Because the issues which impact to alternative 2 are can be overcome. But the issues which impact to alternative 1 are not easy to find the solution to overcome the impact.

Proposal 3: Alternative 2 is more reasonable and feasible method for ground truth definition for RSRP accuracy for FR1.

### Issue 2-1-7: Impacts of measurement error on prediction accuracy

During last RAN4#114bis meeting, RAN4 could not come to any conclusion for this issue. We think that the error model incorporates RF error and BB error, so that simply considering it as a Gaussian does not lose its generality. Legacy link-level simulator also do not consider the complicated error model. They just have used Gaussian error model, and it has already been proven that it reflects the actual situation well. We don’t think AI/ML case are much different.

Proposal 4: RAN4 to consider only Gaussian error model regardless what RF error model or BB error model is.

- Conclusion

In this contribution, we provide our views on issues for AI/ML mobility based on last RAN4 meeting’s moderator summary [2] and WF [1], and we propose

Proposal 1: The beam-level relative L3-RSRP accuracy requirement is not necessary for intra-frequency L3-RSRP.

Proposal 2: Regaring inter-frequency L3-RSRP, for the predicted relative RSRP accuracy requirement, the reported RSRP of cell2 should be predicted value.

Proposal 3: Alternative 2 is more reasonable and feasible method for ground truth definition for RSRP accuracy for FR1.

Proposal 4: RAN4 to consider only Gaussian error model regardless what RF error model or BB error model is.

Reference

[1] R4-2504901, “WF on AI/ML Mobility,” Moderater (Nokia)

[2] R4-2503620, “Topic summary for [114bis][210] FS_NR_AIML_Mob,” Moderator (Nokia)