TSG-RAN Working Group 4 (Radio) meeting #115	R4-2506307

St. Julian’s, Malta, 19th to 23rd May 2025

Source:	Ericsson

Title:	Overview of BS-to-BS coupling loss simulation results

Agenda item:	7.13.3.2

Document for:	Approval

# Introduction

In Rel-19 BS RF evolution WI a work package to further study possibilities to evolve the requirement concept for co-location requirements is included in the Work Item Description (WID) [1].

From previous RAN4 meetings we have (in contributions [2, 3, 4] submitted to RAN4#113 in Orlando until RAN4#114-bis in Wuhan) presented a new Near-Field coupling loss simulation methodology and a corresponding set of simulation results to indicate that the BS-to-BS coupling loss for AAS BS at different frequencies are very different to current assumption used for requirement derivation. The Near-Field coupling loss simulation methodology is suitable for simulating BS-to-BS coupling loss for different co-location situations where the base stations are located such that they experience each other in the Near-Field region, as the case for the co-location scenario adopted for AAS BS in Rel-15.

The simulation results presented in this contribution indicate that the BS-to-BS coupling loss is very different for different coupling cases (Array-to-Array or Array-to-Sub-Array) and common band or cross bands. It is also worth noting that results deviate significantly from the current assumption of 30 dB originally defined for 2 GHz and passive antennas in the early days of UTRA.

In this contribution we provide an overview of the simulation methodology and a collection of all simulation results. Based on the results we also provide some observations and proposals to further stimulate discussion. In a companion contribution [7], a CR to TR 37.941 is provided with the intention aligned with guidance from last meeting way-forward [5, 6] to collect simulation results as background for future work.

# Discussion

In this contribution we present a more detailed overview the simulation methodology used to evaluate BS-to-BS coupling loss between co-located Active Antenna System (AAS) Base Stations (BS). For completeness all simulation results have been collected to present a complete picture of BS-to-BS coupling for cases relevant for bands in the upper region of FR1.

By tradition the BS-to-BS coupling loss have been established as a constant value and applied to several requirements. As pointed out in previous contribution [2], the BS-to-BS coupling loss to consider for AAS base stations depends on requirement in mind.

- Co-location transmitter spurious emissions: For this requirement it is more suitable to consider Array-to-Array (A-to-A) coupling loss. This will include effects due to power spread over the whole antenna aperture and beamforming effect on both victim and aggressor side. The A-to-A coupling loss is defined as the total conducted power fed to the aggressor array antenna to the total conducted received power at the victim array antenna.

- Co-location receiver out-of-band blocking: For this requirement it is more suitable to consider Array-to-Sub-Array (A-to-SA) coupling loss. This is since the receiver front-end will only be exposed by power received by a single sub-array. The composite signal will be created in the digital domain in baseband. The A-to-SA coupling loss is defined as the total conducted power fed to the aggressor array antenna to the conducted received power per sub-array at the victim array antenna.

For other requirements used the BS-to-BS coupling loss (e.g., TDD OFF power requirement and Transmitter Intermodulation requirement) different definitions and co-location scenarios may be relevant. Hence, there is not one coupling loss definition that can be used for requirement derivation of all co-location requirements.

It also worth to point out that the Element-to-Element (elem-to-elem) coupling loss is not relevant for requirement derivation.

Figure 2-1 illustrates the different coupling references as described above. A dual-polarized system is depicted where each element has two polarization ports (red and blue) with corresponding sub-array ports (green and purple). Array ports (A) are formed within the same polarization (brown and orange) using uniform-magnitude and a linear phase gradient for beam steering.

Figure 2-1: A single column illustration of different coupling references

The coupling loss can either refer to antenna elements (elem), physical sub-array ports (SA), or logical/physical array ports (A).

  - Methodology

Performing a co-location coupling analysis of two side-by-side large arrays is a formidable task that easily leads to memory overflow and long simulation times, especially if a complete coupling model is used in an Electromagnetic (EM) simulation tool. To overcome this, simplifications have been used such as using a simplified EM model of the antenna element and analytical circuit theory models of subarray networks. The overall idea is to choose these simplifications in a conservative way, i.e., to minimize the coupling loss. For example, there are no isolating structures on the sides of the arrays and no losses in the subarray models. Hence, in a real case the coupling loss should be larger compared to these simulations.

Subarray beamforming networks are represented by S-matrices modelled analytically using circuit theory, see Section 2.1.4. Finally, array ports are obtained by applying beamforming weights to the subarray ports (and positions), see Section 2.2.

For the analysis summarized in this paper, we have split the simulations in two parts:

- In the first stage, the elem-to-elem coupling is represented by a scattering matrix simulated in an EM simulator, see Section 2.1.1. The victim and aggressor array antennas are placed according to decided co-location scenario side-by-side separated 0.1 m, as illustrated in Figure 2.1.2-1. The scattering matrix (S-matrix) includes all elements (both polarizations) of both arrays and contains elem-to-elem coupling loss and associated phase information. If needed, the S-matrix is calculated for several frequencies within the frequency range that is considered.

- In the second stage sub-arrays and array ports are created. Subarray beamforming networks are represented by S-matrices modelled analytically using circuit theory, see Section 2.1.4, and array ports are obtained by applying beamforming weights to the subarray ports (and positions), see Section 2.2. For this stage a general-purpose linear algebra computation tool, e.g., MathWorks MATLAB or Python can be used.

A block diagram of the simulation methodology is shown in Figure 2.1-1.

Figure 2.1-1: Overview of the simulation methodology

    - S-matrix simulation

The antenna element scattering matrix is simulated using a commercial 3D EM field simulation software, in this case CST Studio Suite. The S-matrix contains data for all elements and polarizations in both arrays.

As an example, in the case of 3.5 GHz arrays, each of the arrays has 192 ports (12 rows * 8 columns * 2 polarization). This results in a 384x384 scattering matrix for each frequency point in the defined frequency range (here 3.1 to 3.9 GHz). For more details on geometry and element model, see below.

    - Geometry

For simulating the coupling between two arrays, two antenna arrays are placed next to each other (see Figure 2.1.2-1).

Figure 2.1.2-1: Two antenna arrays in an edge-to-edge co-location scenario

Each array comprises 12x8 dual-polarized radiating elements and a metallic reflector. The vertical element separation is 0.7λ @ 3.5 GHz and the horizontal element separation 0.5λ @ 3.5 GHz.

The distance between the two antennas (edge-to-edge) has been increased from 10 cm to 16 cm to take account of the mechanical support structures, which are present in a real product, and of space for the enclosure, which is also not included in the simulation.

    - Antenna element model

A simplified antenna element was used to get reasonable simulation time for the elem-to-elem S-matrix. Mechanical details, not relevant for the EM simulation was removed to speed up the simulation stage. These removed details include, e.g., as corners, mechanical support structures, logotypes, etc. As example of such the antenna element model used for simulation is shown in Figure 2.1.3-1. It consists of a thin crossed half-wavelength dipole which is fed in the centre by a coaxial port for each polarization (+/-45°) and a metallic reflector at quarter-wave distance below the dipoles. There is no connection between the reflector and the dipole.

Figure 2.1.3-1: Perspective, top and side view of a simplified antenna element model

    - Sub-array modelling

The antenna elements are grouped in 3-element single column sub-arrays, as shown in Figure 2.1.4-1. A sub-array RDN, having 3 element ports and one feed port, is modelled with lossless ideal transmission lines and T-splitters, see Figure 2.1.5-1. To achieve equal power on the three elements, one of the T-splitters, S1, has an uneven power split ratio of 2:1, and the other, S2, has an even power split. Impedances of the quarter wave transformers on the output ports are chosen so that the desired power split ratio and impedance matching is achieved. The lengths of the lines L12, L13, L22 and L23 are chosen to achieve the desired fixed subarray tilt and long enough for the physical element spacing. The propagation constant of the transmission lines is chosen to equal that of free space for simplicity.

# Figure 2.1.4-1: Array and subarray layout

    - Sub-array ports S-matrix (cascading)

The S-matrices of the components in the antenna RF model are combined to an S-matrix describing the entire antenna including RDNs using well known circuit simulation theory, described, e.g., in [9], as visualised in Figure 2.1.5-1.

Z0, L13

Z0, L12

Z0, L23

Z0, L22

S1

S2

Figure 2.1.5-1: A 1:3 RDN is simulated by standard RF circuit techniques using S-matrix cascading

  - Deployment and beamforming weights

In the simulation it is assumed that two AAS base stations are co-located at the same site according to the enclosure edge-to-edge scenario defined in Rel-15 for BS type 1-O co-location requirements in TS 38.104, subclause 4.9. In this scenario it is assumed that both the aggressor AAS base station and the victim AAS base station are mounted side-by-side separated 0.1 m. The aggressor network and the victim network are configured in a hexagonal layout with 0% grid shift as visualised in Figure 2.2-1.

# Figure 2.2-1: Network layout

The victim and aggressor networks are co-sited. Narrow beams (similar to layer 1 codebook generated beams) are formed from the aggressor AAS to the UEs of the aggressor network and the corresponding power is received on the subarray ports of the co-sited AAS of the victim network. Only one cell is considered in the analysis.

Details related to deployment and BS assumptions is listed in Table 2.2-1, and Table 2.2-2.

Table 2.2-1: Network assumptions

| Parameter | Assumption |
| --- | --- |
| Network layout | Hexagon |
| ISD | 500 m |
| Network grid shift | 0 % |
| BS height | 25 m |
| UE height | 1.5 m |
| Number of UEs per sector | 1500 |
| Minimum 2D distance between BS and UE | 35 m |

Table 2.2-2: BS assumptions

| Parameter | 3500 MHz | 7000 MHz |
| --- | --- | --- |
| Sub-array configuration | 3x1 | 3x1 |
| Array configuration | 4x8 | 8x16 |
| Pre-set sub-array down-tilt | 3 degrees | 3 degrees |
| Mechanical down-tilt | 3 degrees | 3 degrees |
| Vertical element separation | 0.7l | 0.7l |
| Horizontal element separation | 0.5l | 0.5l |

For the evaluation in this contribution the physical size of the AAS BS antennas considered for 3500 MHz and 7000 MHz is equal. Considering other band combinations, such as lower bands, it is expected that the physical size will differ between aggressor and victim.

In the case of using randomly positioned UEs and A-to-A coupling loss, 1500 randomly distributed UEs per system are used. Then, at each time instance a pair of users are chosen, which are used to generate one sample of coupling loss, i.e., there will be a total of 1500 coupling loss samples. In Figure 2.2-2, an example of UE locations is visualised for the aggressor network and victim network.

In the case of A-to-SA coupling, only one system uses the above beamforming, and coupling is calculated between the array and the sub-array ports.

Figure 2.2-2: UE distribution

For an AAS BS array antenna operating outside the design frequency it is expected that the directivity will drop due to decorrelation effects due to out-of-band RF filter characteristics, transmission effects in the feeder network part of RDN and unwanted emission signal correlation.

For out-of-band beam weights a correlation model is used, see [4], corresponding to adding white noise to the antenna element weights. Here, 100% correlation corresponds to clean beam weights with no noise and 0% correlation corresponds to fully noisy beam weights. The effect of the beam pattern is that the directivity will decrease when the correlation decreases. In turn this leads to relatively more power being radiated sideways, which seems to minimize the coupling loss. Hence, 0% correlation could be used to derive conservative co-location requirements. The actual value of the correlation is unknown and is dependent on the specific implementation. Specifically, the digital predistortion will influence the out-of-band beam weights significantly.

Assuming a polynomial model, the beam weights for even harmonic frequencies equals the square of the beam weights of the carrier frequency, wc, while for odd harmonics the weights are equal to the beam weights of the carrier.

wnom=wcfor odd harmonicswc2/wc for even harmonics

where wnom, are the nominal beam weights.

All out-of-band beam weights, including the harmonics, are modelled as partly correlated with the correlation as a parameter. Partly correlated beam weights are modelled as:

w=r wnom+1-r wuncorr

where wuncorr are random weights:

wuncorr=x+jy2N

Here, x and y are normally distributed random variables with expected value 0, and N is the number of subarray ports.

  - Result

We have considered two co-location situations where aggressor and victim are operating within the same band and where the aggressor and victim are operating in different bands. In general, co-location spurious emission and co-location receiver blocking requirement is considering coupling between different bands, while TDD OFF power requirement and transmitter intermodulation requirement consider coupling loss within the same band.

In this contribution we looked at BS-to-BS coupling loss for a situation where both victim and the aggressor have the same physical size and operating at the same frequency and different frequencies.

Simulation results related to 3 different situations have been studied for 3500 MHz, 7000 MHz and when the victim and aggressor is operating at different frequencies, according to Table 2.3-1.

Table 2.3-1: Simulation results

| Case | Aggressor BS frequency (MHz) | Victim BS frequency (MHz) | Coupling loss definition |
| --- | --- | --- | --- |
| A | 3500 | 3500 | A-to-SA |
| B | 7000 | 7000 | A-to-A, A-to-SA |
| C | 3500 | 7000 | A-to-A, A-to-SA |

    - Case A

Using the simulation assumptions and methodology described in previous sections results for A-to-SA coupling loss have been produced.

A snapshot of the UE distribution and associated coupling loss is visualised in Figure 2.3.1-1.

Figure 2.3.1-1: A-to-SA coupling loss level versus beam direction for the aggressor AAS BS

Observation 1: Our results indicate angular regions of beam pointing directions with higher and lower coupling loss reflecting variations in the nearfield antenna pattern.

The minimum coupling loss value in this scenario is observed around phi = 20 degrees.

The empirical CDF of A-to-SA coupling loss is plotted for three frequencies in Figure 2.3.1-2. The CDF includes coupling loss for all sub-arrays in the victim base station.

Figure 2.3.1-2: A-to-SA coupling loss levels as an empirical CDF

It can be noticed that the mean A-to-SA coupling loss is 55 dB with a variation of ±15 dB. Some minor variation due to frequency can be seen.

The coupling loss variation comes from the fact that the aggressor BS is pointing the beam to UEs in the aggressor network, meaning that the coupling towards the victim BS will vary.

Observation 2: For 3500 MHz to 3500 MHz, the A-to-SA coupling loss varies between 55 ± 15 dB.

For AAS BS the beamforming will add a spread to the statistics not considered for non-AAS BS with passive antenna. This is an aspect to further consider to properly establish relevant foundation for requirement derivation. For the requirement level derivation instead of minimum coupling a statistical approach should be considered including aspects related to the larger variation of coupling loss in networks with AAS BSs.

The legacy assumption of 30 dB coupling loss was derived based on passive BS antennas and a specific scenario considering aggressor BS and victim BS being located in the same geographical area at different towers. Detailed information for the scenario can be found in TR 25.942, subclause 10.2. For AAS BS with beamforming a statical approach should be adopted when coupling loss for requirement derivation is considered.

In current specifications, a fixed value for BS-to-BS coupling loss have been derived from in-band assumptions and applied for requirement derivation for cases with large frequency offset between victim and aggressor.

Observation 3: In this contribution identically sized AAS base stations have been studied. Previous studies [8] indicate such size matching is the worst-case coupling loss.

In the co-location scenario introduced for AAS BS in Rel-15 a co-location scenario between an AAS BS and a non-AAS BS is considered, see description in TS 38.104, subclause 4.9. This scenario is relevant for non-AAS BS and AAS BS operating below 3 GHz. To further explore the co-location coupling loss above 3 GHz we consider two AAS BS to be co-located at the same site.

The co-location scenario analysed in this contribution is based on two AAS BS located side-by-side with separation of 0.1 m. This co-location scenario represents a situation rarely seen at real sites for wide area base stations. Often the separation of base stations operating in different networks operating in different bands are larger than 0.1 m in real situations. Moreover, the antennas interfere with each other’s reactive nearfield regions. This will complicate standardization of measurement with standard test antennas and simulations.

In reality, for a real AAS BS implementation the spacing between enclosures is specified to be 0.1 m, this means that the total spacing between the arrays will be larger than 0.1 m.

    - Case B

The empirical CDF of the A-to-A coupling loss is depicted in Figure 2.3.2-1.

Figure 2.3.2-1: A-to-A coupling loss levels as an empirical CDF

It can be noticed that the mean A-to-A coupling loss is 77 dB with a variation of ±15 dB. Some minor variation due to frequency can be seen.

Observation 4: For 7000 MHz to 7000 MHz, the A-to-A coupling loss varies between 77 ± 15 dB.

For each beam direction of the left (or right) array, the coupling loss depends on frequency. In Figure 2.3.2-2 the minimum coupling loss over frequency is depicted versus beam pointing direction and for the two arrays.

Figure 2.3.2-2: Minimum A-to-A coupling loss versus beam direction

In the A-to-SA coupling loss case, beams are directed to the users of the left system as depicted below. The coupling loss is then calculated from the corresponding beam port (including beamforming weights applied to all sub-arrays of the left array which have a fixed subarray down tilt) to all the subarray ports of the right system.

The empirical CDF of the A-to-SA coupling loss is depicted in Figure 2.3.2-3.

Figure 2.3.2-3: A-to-SA coupling loss levels as an empirical CDF

It can be noticed that the mean A-to-SA coupling loss is 70 dB with a variation of ±15 dB. Some minor variation due to frequency can be seen.

Observation 5: For 7000 MHz to 7000 MHz, the A-to-SA coupling loss varies between 70 ± 15 dB.

Observation 6: As expected, A-to-A and A-to-SA coupling losses are different.

Minimum coupling loss for A-to-SA case is depicted in Figure 2.3.2-4. For the beam directions of the left array, the minimum coupling loss over frequency and subarrays of the right array, is used. For the subarrays of the right array, the minimum coupling loss over frequency and beam directions of the left array, are presented.

Figure 2.3.2-4: Minimum A-to-SA coupling loss

It can be noticed that the coupling experiences between the array and different sub-arrays varies. The impact on requirement derivation should be further studied separately for each co-location requirement.

Observation 7: The non-uniform coupling loss with respect to subarray position in the Array-to-subarray and the impact for requirements derivation needs further study.

    - Case C

The empirical CDFs of the A-to-A coupling results are shown in Figure 2.3.3-1 for different levels of antenna correlation as explained in Section 2.2 and at different frequencies. Note that only the isolation between the beam (= array) ports are shown.

| - 6425 MHz | - 6775 MHz | - 7125 MHz |
| --- | --- | --- |

Figure 2.3.3-1: A-to-A coupling loss, aggressor at 3500 MHz at different correlation levels and victim at three frequencies

It can be noticed that the mean A-to-A coupling loss is 54 dB with a variation of ±10 dB. Some minor variation due to frequency can be seen.

Observation 8: For 3500 MHz to 7000 MHz, the A-to-A coupling loss varies between 54 ± 10 dB.

Observation 9: The A-to-A coupling loss is lower for higher correlation.

Observation 10: When considering AAS BS coupling loss characteristics for deriving requirement levels, choosing the minimum coupling loss represents an outlier case which would risk driving design complexity and cost. A more suitable percentile would be in the range 95 to 99%.

For each beam direction of the left (or right) array, the coupling loss depends on frequency. In Figure 2.3.3-2 the minimum coupling loss over frequency is depicted versus beam pointing direction for the left array (#1) and for 0 and 100% correlation levels (r = 0 or 1).

Figure 2.3.3-2: Minimum A-to-A coupling loss versus beam direction for 0% and 100% correlation

In the A-to-SA coupling case, beams are directed to the users of the left system as depicted below. The coupling loss is then calculated from the corresponding beam port (including beamforming weights applied to all sub-arrays of the left array which have a fixed subarray down tilt) to all the subarray ports of the right system. Note that only the isolation between the beam (= array) and subarray ports are shown.

The empirical CDFs of the A-to-SA coupling results are shown in Figure 2.3.3-3 for different levels of antenna correlation and at different frequencies.

| - 6425 MHz | - 6775 MHz | - 7125 MHz |
| --- | --- | --- |

Figure 2.3.3-3: A-to-SA coupling loss, aggressor at 3500 MHz at different correlation levels and victim at three frequencies

Compared to the A-to-A case the spread is significantly lower, but again the trend that the lower correlation gives the highest coupling is still clear.

It can be noticed that the mean A-to-SA coupling loss is 57 dB with a variation of ± 10 dB. Some minor variation due to frequency can be seen.

Observation 11: For 3500 MHz to 7000 MHz, the A-to-SA coupling loss varies between 57 ± 10 dB.

Observation 12: The spread in the coupling is lower in the A-to-SA compared to the A-to-A case.

The simulation results presented in this contribution indicate that the BS-to-BS coupling loss is very different for different coupling cases (Array-to-Array or Array-to-Sub-Array) and common band or cross bands. It is also worth noticing that results deviate significantly from the current assumption of 30 dB originally defined for 2 GHz and passive antennas in the early days of UTRA.

Observation 13: The simulation results indicates that the BS-to-BS coupling loss relevant for AAS BS operating at and above 3500 MHz is larger than current assumption of 30 dB.

# Conclusion

In this contribution we provide technical background related to simulation methodology required to model Near-Field coupling effects for the purpose to evaluate BS-to-BS coupling loss. The model was used to evaluate coupling loss considering the situation where two AAS BS are co-located at the same site. The BS-to-BS coupling loss is the core parameter used to derive requirement levels for co-location requirements. Depending on requirement, different isolation types (A-to-A or A-to-SA) is essential for proper requirement level derivation. We have studied the BS-to-BS coupling for 3 different cases including coupling within the same band and the coupling between bands. To be able to compare the results the mean value at 50% is evaluated together with ± spread approximated within the range of 5% to 95% on the CDFs. All the results are collected in Table 3-1.

Table 3-1: Overview of simulation results

| Case | A-to-A coupling loss (dB) | A-to-SA coupling loss (dB) |
| --- | --- | --- |
| A | No result | 55 ± 15 |
| B | 77 ± 15 | 70 ± 15 |
| C | 54 ± 10 | 57 ± 10 |

The results indicates that the BS-to-BS coupling loss is very different for different coupling cases (Array-to-Array or Array-to-Sub-Array) and common band or cross bands. It is also worth noticing that results deviate significantly from the current assumption of 30 dB originally defined for 2 GHz and passive antennas in the early days of UTRA.

Based on the experience from the simulation it can be concluded that the coupling loss associated with 100% on the CDF also referred to as Minimum Coupling Loss (MCL) is very sensitivity to number of UEs considered. A more stable value can be found in the range 95% to 99% on the CDF.

For non-AAS BS the coupling loss was evaluated as MCL derived based on a link budget, since the BS radiation patterns was fixed. With AAS BS it would be more reasonable to use a point on the CDF in the range 95% to 99% due to the dynamic behaviour of the AAS BS beamforming due to UE location and channel condition.

From the results we observe following:

Observation 1: Our results indicate angular regions of beam pointing directions with higher and lower coupling loss reflecting variations in the nearfield antenna pattern.

Observation 2: For 3500 MHz to 3500 MHz, the A-to-SA coupling loss varies between 55 ± 15 dB.

Observation 3: In this contribution identically sized AAS base stations have been studied. Previous studies [8] indicate such size matching is the worst-case coupling loss.

Observation 4: For 7000 MHz to 7000 MHz, the A-to-A coupling loss varies between 77 ± 15 dB.

Observation 5: For 7000 MHz to 7000 MHz, the A-to-SA coupling loss varies between 70 ± 15 dB.

Observation 6: As expected, A-to-A and A-to-SA coupling losses are different.

Observation 7: The non-uniform coupling loss with respect to subarray position in the Array-to-subarray and the impact for requirements derivation needs further study.

Observation 8: For 3500 MHz to 7000 MHz, the A-to-A coupling loss varies between 54 ± 10 dB.

Observation 9: The A-to-A coupling loss is lower for higher correlation.

Observation 10: When considering AAS BS coupling loss characteristics for deriving requirement levels, choosing the minimum coupling loss represents an outlier case which would risk driving design complexity and cost. A more suitable percentile would be in the range 95 to 99%.

Observation 11: For 3500 MHz to 7000 MHz, the A-to-SA coupling loss varies between 57 ± 10 dB.

Observation 12: The spread in the coupling is lower in the A-to-SA compared to the A-to-A case.

Observation 13: The simulation results indicates that the BS-to-BS coupling loss relevant for AAS BS operating at and above 3500 MHz is larger than current assumption of 30 dB.

Based on the observation we propose following:

Proposal 1: For transmitter spurious emission and receiver out-of-band blocking co-location coupling loss between two systems operating at different bands should be considered, assuming relevant antenna aperture sizes for victim and aggressor.

Proposal 2: For TDD OFF power and transmitter intermodulation co-location coupling loss between two systems operating at the same band should be considered.

Proposal 3: Co-location of systems with different array size should be studied, e.g. non-AAS to AAS and AAS to AAS.

Proposal 4: Study the impact on requirements of non-uniform coupling loss with respect to subarray position in the array-to-subarray case.

Proposal 5: When deriving co-location requirements, RAN4 need to agree on what percentile to use in the coupling loss statistics.

Proposal 6: Zero correlation of antenna weights can be used to derive worst case requirements for co-location between different bands well separated in frequency.

In a companion contribution a CR to TR 37.941 is created to capture simulation results as technical background for further work in the future.

# References

[1]	RP-243270, “Revised WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, ZTE Corporation, HiSilicon, Huawei, Ericsson, Nokia, NEC

[2]	R4-2418398, “Simulation of BS-to-BS isolation for edge-to-edge co-location scenario”, Ericsson

[3]	R4-2502160, “Simulation of BS-to-BS isolation for edge-to-edge co-location isolation at 7 GHz”, Ericsson

[4]	R4-2504013, “Simulation of BS-to-BS out-of-band isolation for edge-to-edge co-location isolation between 3.5 and 7 GHz”, Ericsson

[5]	R4-2502279, “Way Forward for [114][303] NR_BS_RF_Part2_CLTA”, Huawei, HiSilicon

[6] 	R4-2504701, “Way Forward for CLTA”, Huawei, HiSilicon

[7]	R4-2506308, “CR to TR 37.941: Addition of Rel-19 co-location requirement improvements in subclause 6.4”, Ericsson

[8]	R4-2009967, “On the criteria for selecting a proper CLTA”, Ericsson

[9] 	I. A. Sadovskyy, “Reduction of the scattering matrix array”, Physics-Uspekhi, vol. 58, pp.872-876, 2015, DOI: 10.3367/UFNe.0185.201509c.0941