3GPP TSG-RAN Working Group 4 (Radio) Meeting #115	R4-2506308

# St. Julian's, Malta, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.941 | CR | 0060 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 37.941: Addition of Rel-19 co-location requirement improvements in subclause 6.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BS_RF_req_evo-Core |  |  |  |  |  | Date: |  |  | 2025-05-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The reason for this CR is to capture essentail findings relevant for the evolution of co-location requirements part of Rel-19 BS RF evolution WI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - A structure is added to collect Rel-19 findings related to co-location requirement evolution. - Information related to how to simulate near-field coupling between antennas is captured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the finding from Rel-19 BS RF evolution will not be captured in TR 37.941. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Sub-clause 6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Sub-clause 6.4:

## 6.4	Co-location measurements

### 6.4.1	General

Co-location measurements are required for the OTA co-location requirements which are based on assuming the BS is co-located with another BS of the same base station class. They ensure that both co-located systems can operate with minimal degradation to each other.

Co-location requirements are specified as power levels into or out of the conducted interface of the co-location reference antenna. For conformance testing the requirements are translated to the input or output of a co-location test antenna (CLTA).

CLTA is a practical passive antenna that is used for conformance testing of the co-location requirements and is based on the definition of the co-location reference antenna. Basic principle of the CLTA are outlines in clause 6.4.2. For more details on the co-location test antenna characteristics, refer to TS 37.145-2 [4] clause 4.15.2.2, or TS 38.141-2 [6] clause 4.12.2.2. The CLTA is suitable for testing BS implemented with a planar antenna array.

The alignment between the BS under test and the co-location test antenna is depicted in figure 6.4.1-1. For more details on the co-location test antenna alignment and test arrangement, refer to TS 37.145-2 [4] clause 4.15.2.3, or TS 38.141-2 [6] clause 4.12.2.3. The same physical alignment applies to in-band and out-of-band co-location requirements.

Co-location requirements are not applicable to FR2.

Figure 6.4.1-1: Alignment of BS and CLTA

### 6.4.2	Co-location test antenna

The co-location test antenna is a practical passive antenna based on the definition of the co-location reference antenna, and it is used for conformance testing of the co-location requirements. Thus the CLTA should be within a certain tolerance of the co-location reference antenna definition, so that commonly available passive BS antennas can be used to test the co-location requirements without having to use a large number of test antennas and without having to design specialized co-location test antennas.

For example, where the co-location reference antenna definition specifies that the co-location reference antenna, should be a single column passive antenna which has the same vertical radiating dimension (h) as the composite antenna of BS, this would be very restrictive when selecting a test antenna. Hence the CLTA should be within a certain tolerance for the vertical radiating dimension (h) such that a suitable practical BS antenna can be found for each BS.

The CLTA is hence defined as follows:

-	The CLTA is a single column passive antenna which has a vertical radiating dimension (h) which is equal to the vertical radiating dimension of the BS composite antenna ±30 %.

-	For in-band requirements, the CLTA supports the same frequency range and polarizations as the BS composite antenna.

-	For out-of-band co-location requirements, the half power vertical beamwidth of the CLTA equals the narrowest declared vertical beamwidth ±3 °.

-	The polarization should match the in-band CLTA polarization.

-	The CLTA has a horizontal half-power beamwidth of 65 ° ± 10 °.

-	The front faces of the BS and the CLTA need to be aligned in a common plane perpendicular to the mechanical bore-sight direction.

-	The centre of the vertical radiating regions of the CLTA and the BS composite antenna need to be aligned.

### 6.4.3	Standard test antenna

Co-location requirements may also be tested with a standardised test antenna such as a dipole. This has a number of advantages:

-	Standard test antennas are easily available and easier to specify.

-	Repeatability between different conformance testing runs will be greater.

-	Using standard test antennas, fewer antennas will be required for conformance testing.

-	A standard test antenna can have higher gain in the direction of the BS than a CLTA, so that signal levels can be higher relative to the noise floor in emission measurements and lower test levels can be used in interference measurements.

In order to test with a standard test antenna, the translation between measured power levels in the standard test antenna and the co-location reference antenna must be shown. The method for translating the power levels in the co-location reference antenna to the standard test antenna and vice versa is not covered by the present release of this technical report.

### 6.4.4	Aspects relevant for co-location requirements

In this subclause essential findings relevant for the evolution of the co-location requirements from the Rel-19 BS RF evolution WI is captured.

#### 6.4.4.1	Near-field simulation of BS-to-BS coupling loss

In this section an overview of an approach to be used to evaluate near-field coupling between two antennas located close to each other is described.

#### 6.4.4.1.1	Methodology

To evaluate BS-to-BS coupling loss for a side-by-side co-location scenario the near-field coupling properties needs to be modelled properly. It would be natural to setup a complete coupling setup in EM simulation tool to model the coupling loss between AAS BS array antennas. However, such simulation would require a lot of computer memory and processing time.

Therefore, the approach we have used is split up in two parts:

- In the first stage calculate the element coupling matrix including elements for both victim and aggressor array antenna, placed according to decided co-location scenario side-by-side separated 0.1 m, as illustrated in Figure 6.4.4.1.1-2. The S-matrix including all elements can be calculated using PLM CST full-wave simulation or Ansys HFSS. If needed, the S-matrix is calculated for several frequencies within the frequency range that is considered.

- In the second stage circuit theory is used to create sub-arrays and apply beam forming excitations. For this stage MathWorks Matlab or Python can used since calculation can be described as linear algebra. The second stage can be divided in two sub-stages as:

  - Based on the S-matrix including elements the sub-array can be coupled together using S-matrix algebra based on circuit theory, as visualised in Figure 6.4.4.1.1-4.

  - Based on network deployment assumption distribute UEs within the sector and calculate beam directions antenna weights. First, the subarray ports are created using the RDN described earlier, and the coupling matrix between all subarray ports is calculated. Next, the deployment parameters, i.e., antenna height, mechanical tilt, pointing direction, etc. are used together with the position of UEs to determine the beamforming weights for the AAS of the aggressor network. These weights are then applied to the subarray ports, in order to obtain the coupling loss between the beam ports on the aggressor antenna and the subarray ports on the victim antenna. The resulting CDF is an aggregate of all UE positions and all subarray ports for the A-to-SA case.

The simulation methodology is visualized in Figure 6.4.4.1.1-1.

# Figure 6.4.4.1.1-1: Overview of the simulation methodology

For simulating the coupling between two arrays, two antenna arrays are placed next to each other (see Figure 6.4.4.1.1-2).

Figure 6.4.4.1.1-2: Two antenna arrays in an edge-edge colocation scenario

As en example for 3.5 GHz, each array comprises 12x8 dual-polarized radiating elements and a metallic reflector. The vertical element separation is 0.7λ @ 3.5 GHz and the horizontal element separation 0.5λ @ 3.5 GHz.

A simplified antenna element was used to get reasonable simulation time for the elem-to-elem S-matrix. Mechanical details, not relevant for the EM simulation was removed to speed up the simulation stage. These removed details include, e.g., as corners, mechanical support structures, logotypes, etc. The antenna element model used for simulation is shown in Figure 6.4.4.1.1-3. It consists of a thin crossed half-wavelength dipole which is fed in the centre by a coaxial port for each polarization (+/-45°) and a metallic reflector at quarter-wave distance below the dipoles. There is no connection between the reflector and the dipole.

Figure 6.4.4.1.1-3: Perspective, top and side view of antenna element model

A simplified antenna element was used to get reasonable simulation time for the element-to-element S-matrix.

The elements are combined to sub-arrays in the RDN, as visualised in Figure 6.4.4.1.1-4. As an example, a 3x1 sub-array is created in RDN by using standard RF circuit techniques using S-matrix cascading.

Z0, L13

Z0, L12

Z0, L23

Z0, L22

S1

S2

# Figure 6.4.4.1.1-4: Overview of the simulation methodology

An RDN, of a 3-element subarray, having 3 output ports and one feed port, is modelled with ideal transmission lines and T-splitters. To achieve equal power on the three elements, one of the T-splitters, S1, has an uneven power split ratio of 2:1, and the other, S2, has an even power split. Impedances on the quarter wave transformers on the output ports are chosen so that the desired power split ratio and impedance matching is achieved. The lengths of the lines L12, L13, L22 and L23 are chosen to achieve the desired fixed subarray tilt.

The placement of the victim base station and aggressor base station and the individual base station array antenna geometry is visualised in Figure 6.4.4.1.1-5.

# Figure 6.4.4.1.1-5: Array and subarray layout

The aggressor network AAS BS array antenna is located at the left and the victim network AAS BS array antenna is located at the right in Figure 6.4.4.1.1-5.

#### 6.4.4.1.2	Assumptions

In the simulation it is assumed that two AAS base stations are co-located at the same site according to the edge-by-edge scenario defined in Rel-15 for co-location requirements, see TS 38.104, subclause 4.9. In this scenario it is assumed that both the aggressor AAS base station and the victim AAS base station are mounted edge-by-edge separated 0.1 m.

# Figure 6.4.4.1.2-1: Network layout

The victim and aggressor networks are co-sited narrow beams (similar to layer 1 codebook generated beams) are formed from the aggressor AAS to the UEs of the aggressor network and the corresponding power is received on the subarray ports of the co-sited AAS of the victim network. Only one cell is considered in the analysis. Note, that there is no dependence of the UE positions of the victim system.

Details related to deployment and BS assumptions is listed in Table 6.4.4.1.2-1, and Table 6.4.4.1.2-2.

Table 6.4.4.1.2-1: Network assumptions

| Parameter | Assumption |
| --- | --- |
| Network layout | Hexagon |
| ISD | 500 m |
| Network grid shift | 0 % |
| BS height | 25 m |
| UE height | 1.5 m |
| Number of UEs per sector | 1500 |
| Minimum 2D distance between BS and UE | 35 m |

Table 6.4.4.1.2-2: BS assumptions

| Parameter | 3500 MHz | 7000 MHz |
| --- | --- | --- |
| Sub-array configuration | 3x1 | 3x1 |
| Array configuration | 4x8 | 8x16 |
| Pre-set sub-array down-tilt | 3 degrees | 3 degrees |
| Mechanical down-tilt | 3 degrees | 3 degrees |
| Vertical element separation | 0.7l | 0.7l |
| Horizontal element separation | 0.5l | 0.5l |

For the evaluation in this contribution the physical size of the AAS BS antennas considered for 3500 MHz and 7000 MHz is of equal size. Considering other band combinations, such as lower bands, it is expected that the physical size will differ.

To get enough data for a statistical analysis 1500 random UEs per system are used, and at each time instant a pair of randomly selected users are considered for the calculation of A-to-A and A-to-SA coupling loss, as visualised in Figure 6.4.4.1.2-2.

Figure 6.4.4.1.2-2: UE distribution

For an AAS BS array antenna operating outside the design frequency it is expected that the gain will drop due to decorrelation effects due to out-of-band RF filter characteristics, transmission effects in the feeder network part of Radio Distribution Network (RDN) and unwanted emission signal correlation.  To model the array antenna characteristics outside the design frequency the actual beam weights for out-of-band emissions are modelled as partly uncorrelated. The actual value of the correlation is unknown and is dependent on the specific implementation. Specifically, the digital predistortion will influence the out-of-band beam weights significantly.

Assuming a polynomial model, the beam weights for even harmonic frequencies equals the square of the beam weights of the carrier frequency, wc, while for odd harmonics the weights are equal to the beam weights of the carrier.

wnom=wcfor odd harmonicswc2wcfor even harmonics

where wnom, are the nominal beam weights.

All out-of-band beam weights, including the harmonics, are modelled as partly correlated with the correlation as a parameter. Partly correlated beam weights are modelled as:

w=r wnom+1-r wuncorr

where wuncorr are random weights:

wuncorr=x+jy2N

Here, x and y are normally distribute random variables with expected value 0, and N is the number of subarray ports.

#### 6.4.4.1.3	Results

We have considered co-location scenario where aggressor and victim are operating within the same band and where the aggressor and victim are operating in different bands. In general, co-location spurious emission and co-location receiver blocking requirement is considering coupling between different bands, while TDD OFF power requirement and transmitter intermodulation requirement consider coupling loss within the same band.

In this contribution we looked at BS-to-BS coupling loss for a situation where both victim and the aggressor have the same physical size and operating at the same frequency and different frequency.

Simulation results related to 3 different situations have been studied for 3500 MHz, 7000 MHz and when the victim and aggressor is operating at different frequencies, according to Table 6.4.4.1.3-1.

Table 6.4.4.1.3-1: Simulation results

| Case | Aggressor BS frequency (MHz) | Victim BS frequency (MHz) | Coupling loss |
| --- | --- | --- | --- |
| A | 3500 | 3500 | A-to-SA |
| B | 7000 | 7000 | A-to-A, A-to-SA |
| C | 3500 | 7000 | A-to-A, A-to-SA |

Figure 6.4.4.1.3-1: Case A, A-to-SA coupling loss levels as an empirical CDF

Figure 6.4.4.1.3-2: Case B, A-to-A coupling loss levels as an empirical CDF

Figure 6.4.4.1.3-3: Case B, A-to-SA coupling loss levels as an empirical CDF

| - 6425 MHz | - 6775 MHz | - 7125 MHz |
| --- | --- | --- |

Figure 6.4.4.1.3-4: Case C, A-to-A coupling loss, aggressor at 3500 MHz at different correlation levels and victim at three frequencies

| - 6425 MHz | - 6775 MHz | - 7125 MHz |
| --- | --- | --- |

Figure 6.4.4.1.3-5: Case C, A-to-SA coupling loss, aggressor at 3500 MHz at different correlation levels and victim at three frequencies

The results are summarized for BS-to-BS coupling for 3 different cases including coupling within the same band and the coupling between bands. To be able to compare the results the mean value at 50% is evaluated together with +/- spread approximated to 5% to 95% on CDFs. All the simulation results summarized and collected in Table 6.4.4.1.3-2.

Table 6.4.4.1.3-2: Overview of simulation results

| Case | A-to-A coupling loss (dB) | A-to-SA coupling loss (dB) |
| --- | --- | --- |
| A | No result | 55 +/- 15 |
| B | 77 +/- 15 | 70 +/- 15 |
| C | 54 +/- 10 | 57 +/- 10 |

The results indicates that the BS-to-BS coupling loss is very different for different coupling cases (Array-to-Array or Array-to-Sub-Array) and common band or cross bands. It is also worth noticing that results deviate significantly from the current assumption of 30 dB originally defined for 2 GHz and passive antennas in the early days of UTRA.

#### 6.4.4.2	Measurement results

{Editors note: Place holder for measurement results.}

#### 6.4.4.3	Test aspects

{Editors note: Place holder for test aspects, such as wide band Horn-Antennas, etc.}