3GPP TSG-RAN WG4 Meeting # 115	R4-2506310

# Malta, MT, 19th – 23rd May, 2025

Source:	LG Electronics

Title:	Discussion for ATG UE RF requirements

Agenda item:	7.12.2.1

Document for:	Discussion

Introduction

In the RAN4#114 meeting,  the WF on UE RF requirements for ATG was approved. In this discussion, we share our point of view for the UE RF requirements for ATG based on the approved WF[1]

Discussion

Cross band isolation MSD analysis for ATG UE

Errors were found in the MSD analysis[2] submitted in the last meeting, so it was revised and reanalyzed. Additionally, the MSD values according to the output power of the ATG UE were calculated based on the agreements made in the last meeting.

MSD for CA_n3A-n39A with n39 UL

Table 1 PRX noise analysis

| 30 MHz BW Tx PSD level | -47.77 | dBm/Hz |
| --- | --- | --- |
| 30 MHz BW leakage PSD level | -114.77 | dBm/Hz |
| PRx n3 Rx rejected power(Tx signal to LNA) | -152.77 | dBm/Hz |
| PRx n39 Tx rejected power(Tx leakage to LNA direct) | -164.77 | dBm/Hz |
| Total noise power | -151.47 | dBm/Hz |
| For 25 MHz DL noise power | -77.49 | dBm |

Table 2 DRX noise analysis

| DRx n3 Rx rejected power | -180.77 | dBm/Hz |
| --- | --- | --- |
| DRx n39 Tx rejected power | -182.77 | dBm/Hz |
| PCB isolation(Rx rejection) | -177.77 | dBm/Hz |
| PCB isolation(Tx rejection) | -164.77 | dBm/Hz |
| Total noise power | -158.28 | dBm/Hz |
| For 25 MHz DL noise power | -84.30 | dBm |

Table 3 MSD value after MRC with SNR and IM

| after MRC | -84.9 | dBm |
| --- | --- | --- |
| n3 REFSENS | -88.9 | dBm |
| MSD(-1 dB SNR, 2.5 dB IM) | 4 | dB |

Proposal 1: Cross band isolation MSD values for n39 UL n3 DL

Table 4 MSD value for n39 UL n3 DL

| ATG UE Output power (dBm) | MSD value (dB) |
| --- | --- |
| 23dBm | 4.0 |
| 24dBm | 5.3 |
| 25dBm | 5.6 |
| 26dBm | 6.0 |
| 27dBm | 6.4 |
| 28dBm | 6.8 |
| 29dBm | 7.3 |
| 30dBm | 7.9 |
| 31dBm | 8.4 |
| 32dBm | 9.1 |
| 33dBm | 9.8 |
| 34dBm | 10.5 |
| 35dBm | 11.3 |
| 36dBm | 12.1 |
| 37dBm | 13.0 |
| 38dBm | 13.9 |
| 39dBm | 14.8 |
| 40 dBm | 15.7 |

MSD for CA_n3A-n39A with n3 UL

Table 5 PRX noise analysis

| 25 MHz BW Tx PSD level | -46.98 | dBm/Hz |
| --- | --- | --- |
| 25 MHz BW leakage PSD level | -123.98 | dBm/Hz |
| PRx n39 Rx rejected power(Tx signal to LNA) | -156.98 | dBm/Hz |
| PRx n3 Tx rejected power(Tx leakage to LNA direct) | -153.98 | dBm/Hz |
| Total noise power | -152.07 | dBm/Hz |
| For 30 MHz DL | -77.30 | dBm |

Table 6 DRX noise ananlysis

| DRx n39 Rx rejected power | -184.98 | dBm/Hz |
| --- | --- | --- |
| DRx n3 Tx rejected power | -181.98 | dBm/Hz |
| PCB isolation(Rx rejection) | -176.98 | dBm/Hz |
| PCB isolation(Tx rejection) | -183.98 | dBm/Hz |
| Total noise power | -166.32 | dBm/Hz |
| For 30 MHz DL | -91.55 | dBm |

Table 7 MSD value after MRC with SNR and IM

| after MRC | -91.7 | dBm |
| --- | --- | --- |
| n39 REFSENS | -91.94 | dBm |
| MSD | 0.2 | dB |

Proposal 2: Cross band isolation MSD values for n3 UL n39 DL

Table 8 MSD value for n3 UL n39 DL

| ATG UE Output power (dBm) | MSD value (dB) |
| --- | --- |
| 23dBm | 0.2 |
| 24dBm | 0.4 |
| 25dBm | 0.6 |
| 26dBm | 0.9 |
| 27dBm | 1.2 |
| 28dBm | 1.5 |
| 29dBm | 1.9 |
| 30dBm | 2.3 |
| 31dBm | 2.8 |
| 32dBm | 3.4 |
| 33dBm | 4.0 |
| 34dBm | 4.6 |
| 35dBm | 5.3 |
| 36dBm | 6.1 |
| 37dBm | 6.9 |
| 38dBm | 7.7 |
| 39dBm | 8.5 |
| 40 dBm | 9.4 |

Conclusion

Proposal 1: Cross band isolation MSD values for n39 UL n3 DL

Table 4 MSD value for n39 UL n3 DL

| ATG UE Output power (dBm) | MSD value (dB) |
| --- | --- |
| 23dBm | 4.0 |
| 24dBm | 5.3 |
| 25dBm | 5.6 |
| 26dBm | 6.0 |
| 27dBm | 6.4 |
| 28dBm | 6.8 |
| 29dBm | 7.3 |
| 30dBm | 7.9 |
| 31dBm | 8.4 |
| 32dBm | 9.1 |
| 33dBm | 9.8 |
| 34dBm | 10.5 |
| 35dBm | 11.3 |
| 36dBm | 12.1 |
| 37dBm | 13.0 |
| 38dBm | 13.9 |
| 39dBm | 14.8 |
| 40 dBm | 15.7 |

Proposal 2: Cross band isolation MSD values for n3 UL n39 DL

Table 8 MSD value for n3 UL n39 DL

| ATG UE Output power (dBm) | MSD value (dB) |
| --- | --- |
| 23dBm | 0.2 |
| 24dBm | 0.4 |
| 25dBm | 0.6 |
| 26dBm | 0.9 |
| 27dBm | 1.2 |
| 28dBm | 1.5 |
| 29dBm | 1.9 |
| 30dBm | 2.3 |
| 31dBm | 2.8 |
| 32dBm | 3.4 |
| 33dBm | 4.0 |
| 34dBm | 4.6 |
| 35dBm | 5.3 |
| 36dBm | 6.1 |
| 37dBm | 6.9 |
| 38dBm | 7.7 |
| 39dBm | 8.5 |
| 40 dBm | 9.4 |

- References

# [1] R4-2505112, WF on UE RF reqquirements for ATG (CMCC)

# [2] R4-2503723, Discussion for ATG UE RF requirements (LGE)