# 3GPP TSG-RAN WG4 Meeting #115                               R4-2506315

# St Julian’s, Malta, 19th-23th May, 2025

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on REFSENS, ASCS, ACS of LP-WUR

# Agenda Item:   7.28.3.2

# Document for:  Approval

- Introduction

In this contribution, we provided our views and simulation results on REFSENS, ASCS and ACS for LP-WUR based on the latest output on signal design from RAN1.

- Discussion

## 2.1 How to map LP-WUR type with UE capability

In RAN4, there are two candidate methods to define LP-WUR type. One is to align with UE capability, i.e., OOK-based receiver and OFDM-based receiver. The other one is based on power consumption, i.e., Type 1 with ultra-low power consumption and Type 2 with moderate power consumption. OOK-based receiver/Type 1 WUR are related to IM+NF=18dB, and OFDM-based receiver/Type 2 WUR are related to IM+NF=13.5dB.

Based on the updated Rel-19 UE feature list provided by RAN1, for IDLE/INACTIVE mode, UE feature of NR_LPWUS is separated by LP-WUS operation based on OOK signal and OFDM overlaid sequence[1]. It can be found that OFDM-based receiver needs to achieve more functions than OOK-based receiver, such as sequence detection and SSB-based RRM measurement. In this case, OFDM-based receiver requires more complex hardware architecture and more power consumption to achieve these functions, while these are not needed for OOK-based receiver.

Whether OFDM-based receiver can achieve its functions with very simple architecture and ultra-low power consumption was not studied and discussed in RAN1, and there is not a common understanding whether ultra-low power consumption can be introduced for OFDM-based receiver.

Therefore, unless RAN1 has new outcomes, we prefer to align UE capability with LP-WUR types, i.e., define LP-WUR types as OOK-based receiver and OFDM-based receiver.

Fig. 1  Part of LP-WUS UE feature list (IDLE/INACTIVE mode) [1]

Observation 1: OFDM-based receiver needs to achieve more functions than OOK-based receiver, such as sequence detection and SSB-based RRM measurement.

Observation 2: In RAN1, there is not a common understanding whether ultra-low power consumption can be introduced for OFDM-based receiver.

Proposal 1: Propose to define LP-WUR types as OOK-based receiver and OFDM-based receiver.

## 2.2 REFSENS

With decided Set 1 and Set 2 NF+IM package [2], the SNR for REFSENS should still be evaluated. We provide the SNR results both with and without time error (TE) in Table 2.

Table 2	SNR for REFSENS of LP-WUR to achieve 1% MDR

| Receiver Type | RM coding | SNR without TE | SNR with 0.9us TE |
| --- | --- | --- | --- |
| OOK-based receiver (envelope detection) | 8bit | -3.2dB | -2.5dB |
|  | 16bit | -5.7dB | -5.2dB |
|  | 32bit | -7.7dB | -7.2dB |
| OFDM-based receiver (OFDM overlaid sequence detection) | 8bit | -6.0dB | -5.5dB |
|  | 16bit | -8.6dB | -8.0dB |
|  | 32bit | -10.4dB | -9.7dB |

It can be found that OFDM-based receiver outperforms OOK-based receiver by 2.5~3dB SNR and 0.9us TE increase around 0.7dB SNR.

Observation 3: OFDM-based receiver outperforms OOK-based receiver by 2.5~3dB SNR.

Observation 4: 0.9us TE increases around 0.7dB SNR.

Proposal 2: The REFSENS SNR could be -5.0dB and -8.0dB for OOK-based receiver and OFDM-based receiver, respectively.

REFSENS for MR can be expressed as

REFSENS(dBm) = kT0 + 10log10(NRB_WUS*12*SCS) + NF + SNR + IM - Diversity Gain

The REFSENS of MR is calculated based on: SNR=-1dB, NF+IM=11.5dB, 2Rx Diversity Gain = 3dB.

REFSENS for LP-WUR can be expressed as

REFSENS(dBm) = kT0 + 10log10(NRB_WUS*12*SCS) + NF + SNR + IM

Based on the conclusion in last meeting, the two sets of LP-WUR REFSENS are calculated based on:

- Set 1 (OOK-based receiver): SNR=-5.0dB, NF+IM=18dB, 1Rx Diversity Gain = 0dB;

- Set 2 (OFDM-based receiver): SNR=-8.0dB, NF+IM=13.5dB, 1Rx Diversity Gain = 0dB.

Therefore, REFSENS of OOK-based receiver could be 5.5dB higher than the MR with same operating condition, and REFSENS of OFDM-based receiver could be 2dB lower than the MR with same operating condition.

Proposal 3: REFSENS of OOK-based receiver could be 5.5dB higher than the MR with same operating condition, and REFSENS of OFDM-based receiver could be 2dB lower than the MR with same operating condition.

## 2.3 ASCS

In last meeting RAN4 agreed to define ASCS requirements for Type 1 UE and Type 2 UE [3]. But how to map type 1/type 2 capability and RAN1 UE feature still needs further discussion.

Issue 2-3-1: ASCS requirements for OOK-based receiver Agreement: - For Type1 UE requirements, which are derived based on the assumption that demodulation of OOK content of LP-WUS is targeted - 1 RB guard for ASCS and 0.5dB de-sens - For Type2 UE requirements, which are derived based on the assumption that demodulation of OFDM content of LP-WUS is targeted - 0 RB guard for ASCS and 0 dB de-sens - For Type 1 and Type 2 UE are not limited to specific LP-WUS waveform. - FFS how to define test case and requirement applicability for Type 1 and Type 2 - FFS how type 1 and type 2 capability related to RAN1 UE feature - A UE can only support either Type 1 or Type 2 but not both for LP-WUR

As discussed in Section 2.1, we prefer to define Type 1 UE with OOK-based LP-WUR and define Type 2 UE with OFDM-based receiver. To perform ASCS test, 0 guard RB and 0.5dB de-sense are defined for OOK-based receiver ASCS test, and 1 guard RB and 0dB de-sense are defined for OFDM-based receiver ASCS test. NR PDSCH should occupy all the RBs that are not used for LP-WUS with the same PSD as LP-WUS.

Moreover, we want to clarify that guard RBs can be defined in the test parameters but not as a core requirement.

Proposal 4: Propose not to define the number of guard RBs for ASCS as a core requirement.

Proposal 5: Define the test parameters for ASCS of LP-WUR in Table 3-1 and Table 3-2. For these test parameters, the missed detection rate shall be ≤1%.

Table 3-1  Test parameters for ASCS of OOK-based LP-WUR

| RX parameter | LP-WUS transmission bandwidth configuration (RB) |
| --- | --- |
|  | 11RB |
| LP-WUS Power (dBm) | LP-WUR REFSENS + 0.5 |
| BWinterferer | NR PDSCH is mapped on RBs not used for LP-WUS |
| In-band Pinterferer | Same PSD as LP-WUS |
| Guard RB | 1 |

Table 3-2  Test parameters for ASCS of OFDM-based LP-WUR

| RX parameter | LP-WUS transmission bandwidth configuration (RB) |
| --- | --- |
|  | 11RB |
| LP-WUS Power (dBm) | LP-WUR REFSENS |
| BWinterferer | NR PDSCH is mapped on RBs not used for LP-WUS |
| In-band Pinterferer | Same PSD as LP-WUS |
| Guard RB | 0 |

## 2.4 ACS

The agreements on ACS for LP-WUR are listed as follows [3].

Issue 2-4-2: Whether adopt the same wanted power level for Set-2 LR? Agreement: - The same set of the interference signal level defined for both two types of the LP-WUS requirements. Issue 2-4-4: Case 1 ACS test parameter Agreements: Specify case 1 and case 2 ACS test parameters for Type 1 and Type 2. - Define interferer as BWinterferer of 5 MHz.

It has been agreed that same interference signal level as MR for LP-WUR ACS requirements should be used. Namely, the ACS difference between LP-WUR and MR depends on the REFSENS difference between LP-WUR and MR. Therefore, the ACS requirement for OOK-based receiver could be 33dB-5.5dB=27.5dB. As for OFDM-based receiver, since the REFSENS of OFDM-based receiver can be lower than the MR, same ACS as MR can be used.

Observation 5: OFDM-based receiver outperforms OOK-based receiver a lot in terms of ACS.

Proposal 6: The ACS requirements could be 27.5dB and 33dB for OOK-based receiver and OFDM-based receiver, respectively.

Moreover, in order to achieve better receiving performance, guard RBs can be allocated. But this depends on the network configuration. Besides, it should be noticed that for 5MHz CBW with 30kHz SCS, all the RBs are occupied by LP-WUS and no additional RBs are left for guard RBs. In this case, we prefer not to limit the number of guard RBs as a core requirement and it can be left flexible.

Proposal 7: Propose not to define the number of guard RBs for ACS as a core requirement.

To further check the number of guard RBs in ACS test, we evaluate the MDR of OOK-based receiver and OFDM-based receiver versus different EPRE ratio between interference level and LP-WUS, under the condition of different guard RBs. In legacy spec, to perform ACS test, the wanted signal power is set under a good condition with MR REFSENS + 14dB, and 1.5dB margin is used in ACS test. Thus, we also provide the simulation results under LR REFSENS + 14dB condition for LP-WUR as shown in Fig. 1.

Fig. 2  LP-WUR performance in ACS case under different interference level and guard RB numbers for

OOK-based receiver and OFDM-based receiver

According to the simulation results, 1 guard RB is needed for OOK-based receiver and 2 guard RBs are needed for OFDM-based receiver. Therefore, we propose the test parameters in Table 4-7 as follows. Therefore, we prefer to define the test parameters for ACS of LP-WUR in Table 4-7. For these test parameters, the missed detection rate shall be ≤1%.

Proposal 8: For ACS test, to achieve 26dB ACS, 1 guard RB is needed for OOK-based receiver, and to achieve 31.5dB ACS, 0 guard RBs are needed for OFDM-based receiver. Consider the ACS test parameters in Table 4-1, 4-2 and Table 5-1, 5-2.

Table 4-1  Test parameters for ACS of OOK-based receiver, Case 1

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | LP-WUR REFSENS + 14 dB |
| Pinterferer | dBm | LP-WUR REFSENS + [40] dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| Guard RB | RB | 1 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier without adjacent sub-carrier interference with channel bandwidth BWChannel ≥ 5 MHz. |  |  |

Table 4-2  Test parameters for ACS of OOK-based receiver, Case 2

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | -56.5 |
| Pinterferer | dBm | [-30.5] |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| Guard RB | RB | 1 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier without adjacent sub-carrier interference with channel bandwidth BWChannel ≥ 5 MHz. |  |  |

Table 5-1  Test parameters for ACS of OFDM-based receiver, Case 1

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | LR REFSENS + 14 dB |
| Pinterferer | dBm | LR REFSENS + [45.5] dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| Guard RB | RB | 0 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier without adjacent sub-carrier interference with channel bandwidth BWChannel ≥ 5 MHz. |  |  |

Table 5-2  Test parameters for ACS of OFDM-based receiver, Case 2

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | -56.5 |
| Pinterferer | dBm | [-25] |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| Guard RB | RB | 0 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier without adjacent sub-carrier interference with channel bandwidth BWChannel ≥ 5 MHz. |  |  |

- Conclusion

Observations:

Observation 1: OFDM-based receiver needs to achieve more functions than OOK-based receiver, such as sequence detection and SSB-based RRM measurement.

Observation 2: In RAN1, there is not a common understanding whether ultra-low power consumption can be introduced for OFDM-based receiver.

Observation 3: OFDM-based receiver outperforms OOK-based receiver by 2.5~3dB SNR.

Observation 4: 0.9us TE increases around 0.7dB SNR.

Observation 5: OFDM-based receiver outperforms OOK-based receiver a lot in terms of ACS.

Proposals:

Proposal 1: Propose to define LP-WUR types as OOK-based receiver and OFDM-based receiver.

Proposal 2: The REFSENS SNR could be -5.0dB and -8.0dB for OOK-based receiver and OFDM-based receiver, respectively.

Proposal 3: REFSENS of OOK-based receiver could be 5.5dB higher than the MR with same operating condition, and REFSENS of OFDM-based receiver could be 2dB lower than the MR with same operating condition.

Proposal 4: Propose not to define the number of guard RBs for ASCS as a core requirement.

Proposal 5: Define the test parameters for ASCS of LP-WUR in Table 3-1 and Table 3-2. For these test parameters, the missed detection rate shall be ≤1%.

Table 3-1  Test parameters for ASCS of OOK-based LP-WUR

| RX parameter | LP-WUS transmission bandwidth configuration (RB) |
| --- | --- |
|  | 11RB |
| LP-WUS Power (dBm) | LP-WUR REFSENS + 0.5 |
| BWinterferer | NR PDSCH is mapped on RBs not used for LP-WUS |
| In-band Pinterferer | Same PSD as LP-WUS |
| Guard RB | 1 |

Table 3-2  Test parameters for ASCS of OFDM-based LP-WUR

| RX parameter | LP-WUS transmission bandwidth configuration (RB) |
| --- | --- |
|  | 11RB |
| LP-WUS Power (dBm) | LP-WUR REFSENS |
| BWinterferer | NR PDSCH is mapped on RBs not used for LP-WUS |
| In-band Pinterferer | Same PSD as LP-WUS |
| Guard RB | 0 |

Proposal 6: The ACS requirements could be 27.5dB and 33dB for OOK-based receiver and OFDM-based receiver, respectively.

Proposal 7: Propose not to define the number of guard RBs for ACS as a core requirement.

Proposal 8: For ACS test, to achieve 26dB ACS, 1 guard RB is needed for OOK-based receiver, and to achieve 31.5dB ACS, 0 guard RBs are needed for OFDM-based receiver. Consider the ACS test parameters in Table 4-1, 4-2 and Table 5-1, 5-2.

Table 4-1  Test parameters for ACS of OOK-based receiver, Case 1

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | LP-WUR REFSENS + 14 dB |
| Pinterferer | dBm | LP-WUR REFSENS + [40] dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| Guard RB | RB | 1 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier without adjacent sub-carrier interference with channel bandwidth BWChannel ≥ 5 MHz. |  |  |

Table 4-2  Test parameters for ACS of OOK-based receiver, Case 2

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | -56.5 |
| Pinterferer | dBm | [-30.5] |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| Guard RB | RB | 1 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier without adjacent sub-carrier interference with channel bandwidth BWChannel ≥ 5 MHz. |  |  |

Table 5-1  Test parameters for ACS of OFDM-based receiver, Case 1

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | LR REFSENS + 14 dB |
| Pinterferer | dBm | LR REFSENS + [45.5] dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| Guard RB | RB | 0 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier without adjacent sub-carrier interference with channel bandwidth BWChannel ≥ 5 MHz. |  |  |

Table 5-2  Test parameters for ACS of OFDM-based receiver, Case 2

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | -56.5 |
| Pinterferer | dBm | [-25] |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| Guard RB | RB | 0 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier without adjacent sub-carrier interference with channel bandwidth BWChannel ≥ 5 MHz. |  |  |

- Reference

- R4-2505303(R1-2502979) LS on updated Rel-19 RAN1 UE features lists for NR after RAN1#120bis, RAN4#115, St Julian’s, Malta, May 2025.

- R4-2503028 WF on UE RF requirements for LP-WUS, vivo, RAN4#114, Athens, Greece, Feb. 2025.

- R4-2505234 WF for [114bis][131] NR_LPWUS_UERF, vivo, RAN4#114bis, Wuhan, China, Apr. 2025.

- Annex: Simulation assumptions

The simulation assumptions as listed in Table 1, especially the channel structure part.

Table 1	Link level simulation assumptions for FR1 LP-WUS

| Attributes | Assumptions |
| --- | --- |
| Case name (waveform) | OOK-4 waveform |
| Center frequency | 900MHz, 2.6GHz |
| Channel structure | 5 information bits with 8/16/32bit RM coding No CRC is configured |
| Chip rate | M=1/2/4 |
| Coding | 1/2 rate Manchester coding |
| Repetition | None |
| Time error | 0/0.9us |
| Residual Frequency error | 0/10/20 ppm |
| SCS | 30kHz |
| UE Channel BW | 20MHz (51 RB)-case 1 10MHz (24 RB)-case 2 5MHz (11 RB)-case 3 |
| WUS RB | - Fixed 11RB ~ 3.96MHz for 10MHz and 20MHz cases |
| Position within channel | - For 10/20MHz CBW, Center for ASCS, edge for ACS [assume no ASCS impact] - For 5MHz CBW, fixed center of channel |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - For wanted signal: 505kHz for 5MHz, 665kHz for 10MHz, 805kHz for 20MHz - For interference cell2 5MHz: fixed 505kHz |
| Guard RB | - For ASCS: 0 or 1RB on each side of LP-WUS bandwidth - For ACS: 1/2/3/4 RB |
| Filter | - 3th/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case - Other order lowpass filter is not precluded - The filter bandwidth is adapted with actual WUS RBs, for 5MHz case |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal |
| ACS | PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; EPRE of PDSCH /EPRE of in-band LP-WUS = 15~33dB NOTE: decide the interference level depending on SNR |
| Wanted signal level | For ACS, REFSENS + 14 dB for LP-WUS |
| Sampling rate | 7.68MHz |
| ADC bit width | 4 bits ADC for ASCS/ACS |
| Non-linearities | Not modeled |
| Power boosting | None |
| Channel Model | Option 1: TDL-C 300 Option 2: AWGN Note: encourage companies to provide simulation results with both options |
| Performance metric | - 1% MDR |
| Note: the detailed Channel structure for LP-WUS is under discussion in RAN1, some parameters may be updated accordingly. |  |