3GPP TSG-RAN WG4 Meeting # 115	R4-2506327

# Malta, MT, 19th – 23rd May, 2025

Source:	LG Electronics

Title:	Discussion for UE RF Tx requirements of NTN_NR_HPUE

Agenda item:	7.10.3.2.1

Document for:	Discussion

Introduction

In RAN4#114bis meeting, WF on NR NTN HPUE requirements was approved. Based on the WF, we would like to share our  view for the requirements.

Discussion

PC1.5 1Tx configuration

In the same way as the contribution submitted in the last meeting[2], it is necessary to define the PC1.5 1Tx configuration. Additionally, since the PC1 non-handheld is already defined in the TN for the 1Tx configuration, the PC1.5 1Tx for non-handheld UE can also be defined.

Proposal 1: Specify 1Tx PC1.5 for non-handheld.

ACLR

In the last meeting, there was a tentative agreement on 34 dBc for PC1 ACLR. This value is a compromised one, higher than the result derived from the co-existence study but lower than the ACLR of the exsting TN. The environment in which NTN devices operate may differ from that of TN, and accordingly, the PC1 NTN ACLR value can be relaxed.

Proposal 2: Confirm the 34 dBc ACLR for PC1.

MPR simulation

In this section, PC1 1Tx MPR simulation and PC1.5 1Tx MPR simulation were performed. High order modulation was not simulated, taking into account the NTN communication environment, and only QPSK and 16 QAM were simulated.

PC1 MPR simulation

Depending on how PA calibration is performed, the PC1 MPR value can be defined differently. in this regard, simulations were conducted accrording to the following three cases.

- PC1 MPR simulation cases

  - Reference: ACLR 37 dBc with 37 dBc calibrated PA (TN PC1)

  - Case 1 : ACLR 34 dBc with 34 dBc calibrated PA

  - Case 2 : ACLR 34 dBc with 37 dBc calibrated PA

Reference (TN PC1)

| DFT-OFDM | edge | outer | inner |
| --- | --- | --- | --- |
| QPSK | note1 | 1 | 0 |
| 16QAM | note1 | 2 | 1 |
| CP-OFDM | edge | outer | inner |
| QPSK | note1 | 3 | 1.5 |
| 16QAM | note1 | 3 | 2 |

Case 1 MPR simulation results (ACLR 34 dBc with ACLR 34 dBc calibrated PA)

| DFT-OFDM | edge | outer | inner |
| --- | --- | --- | --- |
| QPSK | FFS | 1 | 0 |
| 16QAM | FFS | 2.5 | 1 |
| CP-OFDM | edge | outer | inner |
| QPSK | FFS | 3 | 1.5 |
| 16QAM | FFS | 3 | 2 |

Case 2: MPR simulation results (ACLR 34 dBc with 37 dBc calibrated PA)

| DFT-OFDM | edge | outer | inner |
| --- | --- | --- | --- |
| QPSK | FFS | 1 | 0 |
| 16QAM | FFS | 2 | 1 |
| CP-OFDM | edge | outer | inner |
| QPSK | FFS | 2 | 1.5 |
| 16QAM | FFS | 2 | 2 |

In the MPR inner region, the ACLR value does not affect the results, so the TN PC1 MPR of inner region can be reused for NTN PC1 MPR inner region. However, for the MPR outer region, the MPR value can vary depending on the ACLR value and the PA calibration. When using a PA calibrated with 34 dBc ACLR, as in case 1, the MPR values were generally the same as the existing TN PC1 MPR values. However, the 16 QAM outer MPR value for DFT-OFDM was found to be 0.5 dB higher than the existing TN PC1 MPR value. When using a PA calibrated with 37 dBc ACLR, as in case 2,the results indicate that the outer region could have lower MPR values than the existing TN PC1 MPR values. Thus, depending on the ACLR values used for PA calibration, the MPR values can be defined as either higher or lower than the existing values.

The NTN UL has a significantly limited link budget, which restricts the services that can be provided. Considering this, defining lower MPR values could help alleviate the NTN communication UL link budget issue.

Observation 1: Depending on the ACLR values used for PA calibration, the MPR values can be defined as either higher or lower than the existing values.

Proposal 3: Proposed NTN 1Tx PC1 MPR values according to PA calibration

  - Option 1: Reuse TN PC1 MPR value

  - Option 2: Consider Case1 (ACLR 34 dBc with ACLR 34 dBc calibrated PA)

| DFT-OFDM | edge | outer | inner |
| --- | --- | --- | --- |
| QPSK | FFS | 1 | 0 |
| 16QAM | FFS | 2.5 | 1 |
| CP-OFDM | edge | outer | inner |
| QPSK | FFS | 3 | 1.5 |
| 16QAM | FFS | 3 | 2 |

  - Option 3: Consider Case2 (ACLR 34 dBc with 37 dBc calibrated PA)

| DFT-OFDM | edge | outer | inner |
| --- | --- | --- | --- |
| QPSK | FFS | 1 | 0 |
| 16QAM | FFS | 2 | 1 |
| CP-OFDM | edge | outer | inner |
| QPSK | FFS | 2 | 1.5 |
| 16QAM | FFS | 2 | 2 |

PC1.5 MPR simulation

In the previous meeting, we reached an agreement on the PC1.5 MPR simulation. However, since 1 Tx PC1.5 has not been excluded, I would like to share the simulation results for 1Tx PC1.5 MPR.

The simulation results indicate that the 1Tx PC1.5 MPR values, except for DFT-s-OFDM QPSK, are 0.5 dB higher than the PC2 MPR values in both the inner and outer regions. Additionally, in the edge region, the values are 3 dB higher than the PC2 MPR values. Based on the 1Tx PC1.5 MPR simulation results, I propose the following 1Tx PC1.5 MPR values:

Proposal 4: Proposed NTN 1Tx PC1.5 MPR value based on the MPR simulation with 1Tx PC1.5

| DFT-OFDM | edge | outer | inner |
| --- | --- | --- | --- |
| QPSK | 6.5 | 1.5 | 0 |
| 16QAM | 6.5 | 2.5 | 1 |
| CP-OFDM | edge | outer | inner |
| QPSK | 6.5 | 3.5 | 2 |
| 16QAM | 6.5 | 3.5 | 2.5 |

Conclusion

Observation: Depending on the ACLR values used for PA calibration, the MPR values can be defined as either higher or lower than the existing values.

Proposal 1: Specify 1Tx PC1.5 for non-handheld.

Proposal 2: Confirm the 34 dBc ACLR for PC1.

Proposal 3: Proposed NTN 1Tx PC1 MPR values according to PA calibration

  - Option 1: Reuse TN PC1 MPR value

  - Option 2: Consider Case1 (ACLR 34 dBc with ACLR 34 dBc calibrated PA)

| DFT-OFDM | edge | outer | inner |
| --- | --- | --- | --- |
| QPSK | FFS | 1 | 0 |
| 16QAM | FFS | 2.5 | 1 |
| CP-OFDM | edge | outer | inner |
| QPSK | FFS | 3 | 1.5 |
| 16QAM | FFS | 3 | 2 |

  - Option 3: Consider Case2 (ACLR 34 dBc with 37 dBc calibrated PA)

| DFT-OFDM | edge | outer | inner |
| --- | --- | --- | --- |
| QPSK | FFS | 1 | 0 |
| 16QAM | FFS | 2 | 1 |
| CP-OFDM | edge | outer | inner |
| QPSK | FFS | 2 | 1.5 |
| 16QAM | FFS | 2 | 2 |

Proposal 4: Proposed NTN 1Tx PC1.5 MPR value based on the MPR simulation with 1Tx PC1.5

| DFT-OFDM | edge | outer | inner |
| --- | --- | --- | --- |
| QPSK | 6.5 | 1.5 | 0 |
| 16QAM | 6.5 | 2.5 | 1 |
| CP-OFDM | edge | outer | inner |
| QPSK | 6.5 | 3.5 | 2 |
| 16QAM | 6.5 | 3.5 | 2.5 |

- References

# [1] R4-2505113, WF on HPUE requirments for NR-NTN (Samsung)

# [2] R4-2503719, Discussion for UE RF Tx requirements of NTN_NR_HPUE (LGE)