3GPP TSG-RAN WG4 Meeting # 115	R4-2506328

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | Draft | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-1 for Rel-19 ATG UE REFSENS requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG_enh-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Specify REFSENS requirements for ATG intra-band contiguous CA and inter-band CA |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clauses to introduce ATG intra-band contiguous CA and inter-band CA - 7.3J.2A Reference sensitivity power level for ATG CA - 7.3J.2A.1 Reference sensitivity power level for ATG intra-band contiguous CA - 7.3J.2A.2 Reference sensitivity power level for ATG inter-band CA - 7.3J.2A.3 Reference sensitivity exceptions due to cross band isolation for ATG CA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ATG UE intra-band contiguous CA and inter-band CA requirements are not specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 7.3J.2A, New 7.3J.2A.1, New 7.3J.2A.2, New 7.3J.2A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change >>

7.3J	Reference sensitivity for ATG

7.3J.1	General

For ATG UE with multiple omni-directional antennas not indicating the capability antennaArrayType-r18, the reference sensitivity power level REFSENS is the minimum mean power per polarization at antenna connector, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

For ATG UE with antenna array indicating the capability antennaArrayType-r18,the reference sensitivity power level REFSENS is the minimum mean power per polarization at TAB antenna connector, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

7.3J.2	Reference sensitivity power level

For a ATG UE(s) equipped with 2 Rx antenna ports, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1) with parameters specified in Table 7.3.2-1a and Table 7.3.2-1b for the applicable operating bands.

For ATG UE(s) equipped with 4 Rx antenna ports, reference sensitivity for 2Rx antenna ports shall be modified by the amount given in ΔRIB,4R in Table 7.3.2-2 for the applicable operating bands.

The reference sensitivity (REFSENS) requirement for a ATG UE shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3.

7.3J.2A	Reference sensitivity power level for ATG CA

7.3J.2A.1	Reference sensitivity power level for ATG intra-band contiguous CA

For ATG UE intra-band contiguous carrier aggregation, the throughput of each component carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, Table 7.3.2-2a, and Table 7.3.2-3.

For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.2-3 and the downlink PCC carrier center frequency shall be configured closer to uplink operating band than any of the downlink SCC center frequency.

7.3J.2A.2	Reference sensitivity power level for ATG inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2 and A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 with parameters specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, Table 7.3.2-2a and Table 7.3.2-3. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active. Exceptions to reference sensitivity are allowed in accordance with clause 7.3J.2A.3.

The reference sensitivity exceptions due to cross band isolation is applicable to the UL aggressor band configured with either one Tx antenna connector or two Tx antenna connectors with UL MIMO or Tx diversity operation.

7.3J.2A.3	Reference sensitivity exceptions due to cross band isolation for ATG CA

Sensitivity degradation is allowed for a band if it is impacted by UL of another band part which belongs to NR band of the same NR CA configuration due to cross band isolation issues. The reference sensitivity degradation for the victim band due to cross band isolation is specified only for the specific uplink and downlink test points specified in Table 7.3J.2A.3-1, and the reference sensitivity degradation values according to UL output power are specified in Table 7.3J.2A.3-1a.

Table 7.3J.2A.3-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from an ATG UE aggressor NR UL band for NR CA in FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1817.5 | 25 | A11 | >ACLR2 |
| n3 | n39 | 1722.5 | 25 | 15 | 160 (RBstart=0) | 1900 | 30 | A22 | >ACLR2 |
| NOTE1: MSD is A1 value corresponding to UL output power in Table 7.3J.2A.3-1a. NOTE2: MSD is A2 value corresponding to UL output power in Table 7.3J.2A.3-1a. |  |  |  |  |  |  |  |  |  |

Table 7.3J.2A.3-1a: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from an ATG UE aggressor for different NR UL output powers

| UL Output power (dBm) | A1 (dB) | A2 (dB) |
| --- | --- | --- |
| 23 |  | [4.4] |  |  | [1.1] |
| 24 |  | [5.0] |  |  | [1.2] |
| 25 |  | [5.4] |  |  | [1.3] |
| 26 |  | [5.9] |  |  | [1.5] |
| 27 |  | [6.5] |  |  | [1.6] |
| 28 |  | [7.0] |  |  | [1.8] |
| 29 |  | [7.7] |  |  | [1.9] |
| 30 |  | [8.4] |  |  | [2.1] |
| 31 |  | [9.1] |  |  | [2.4] |
| 32 |  | [9.9] |  |  | [2.7] |
| 33 |  | [10.8] |  |  | [3.0] |
| 34 |  | [11.6] |  |  | [3.3] |
| 35 |  | [12.5] |  |  | [3.7] |
| 36 |  | [13.4] |  |  | [4.2] |
| 37 |  | [14.4] |  |  | [4.8] |
| 38 |  | [15.4] |  |  | [5.4] |
| 39 |  | [16.4] |  |  | [6.2] |
| 40 |  | [17.5] |  |  | [6.9] |

<< End of Change >>