# 3GPP TSG-RAN WG4 Meeting #115	R4-2506332

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.18.3.2

Source:	China Telecom

Title:	Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Document for:	Discussion

- Introduction

In RAN4 #114bis meeting, FR2-1 SSB based L3 measurement delay reduction for connected mode was discussed and the WF was approved [1].

In this paper, we present our views on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC issues captured in the WF [1] and summary [2].

- Discussion

Status in the WF [1]:

Issue 1-2-2: Expand solution 1 to FR1 case [Way forward]: - FFS: - Option 1 (Nokia, Apple, CTC): Expand the solution 1 to FR1 band. - Option 1a (Nokia): - Expand the solution 1 to at least FR1 intra-band contiguous CA. - Details: - Network indicates to enable or disable this CSSF enhancement i.e. measuring one serving CC per band for FR1 and FR2 separately. - When network indicates to enable Tentative solution 1 for FR1 bands, UE is required to measure serving and neighbor cells on one serving CC per band when configured with FR1 intra-band contiguous CA. - Option 2 (Huawei): Not to expand solution 1 to FR1 band. - Option 3 (moderator): Expand the solution 1 to FR1 intra-band contiguous CA, i.e., measure one serving CC per FR1 band if all to-be-measured serving CCs in this FR1 band are contiguous.

Solution 1 is also beneficial for FR1 and it doesn’t cost much workload to expand solution 1 to FR1 case, since there also exists the demand on L3 measurement delay reduction for FR1 band. Besides, expand to at least FR1 intra-band contiguous CA is ok for us, this can be ensured that the frequency difference is not too large.

Proposal 1: Expand the solution 1 to FR1 intra-band contiguous CA, i.e., measure one serving CC per FR1 band if all to-be-measured serving CCs in this FR1 band are contiguous.

Issues related UE capability for CSSF enhancement

Status in the WF [1]:

Issue 1-4-1: UE capability for CSSF enhancement Agreement: - UE feature #1: Define UE capability for solution 1, further discuss whether it is per-UE or per-FR. - UE feature #2: Define UE capability for solution 3, the UE capability is defined separately for {FR1 only CA and FR1 only NR-DC}, {FR1 only EN-DC}, {FR1+FR2 CA}, {FR1+FR2 NR-DC} in the same FG.

It was agreed in last meeting to define UE capability for solution 1, for the granularity of UE capability, per-UE capability is preferred.

Proposal 2: For the definition of UE capability for solution 1, per-UE capability is preferred.

- Conclusion

The contribution presents our views on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC, with the following proposals:

Proposal 1: Expand the solution 1 to FR1 intra-band contiguous CA, i.e., measure one serving CC per FR1 band if all to-be-measured serving CCs in this FR1 band are contiguous.

Proposal 2: For the definition of UE capability for solution 1, per-UE capability is preferred.

- References

- R4-2504899, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple, RAN WG4 Meeting #114bis, 7 – 11 April, 2025.

- R4-2503618, Topic summary for [114bis][208] NR_RRM_Ph5_Part1, Apple, RAN WG4 Meeting #114bis, 7 – 11 April, 2025.