# 3GPP TSG-RAN WG4 Meeting #115	R4-2506336

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.25.4.1

Source:	China Telecom

Title:	Discussion on RRM requirements for UE-to-UE CLI handling for evolution of NR duplex operation

Document for:	Discussion

- Introduction

In RAN4 #114bis meeting, RRM requirements for UE-to-UE CLI handling for evolution of NR duplex operation was discussed and the WF was approved [1].

In this paper, we present our views on RRM requirements for UE-to-UE CLI handling for evolution of NR duplex operation issues captured in the WF [1] and summary [2].

- Discussion

General issues

Status in the summary [2]:

- Issue 1-1-5: Requirement applicability - Proposals - Proposal 1 (vivo, CATT, Nokia, LGE, Samsung, HW, CTC, QC): - The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is fully confined within BW of DL active BWP - when CLI-RSSI measurement resource is configured within active BWP - Proposal 2 (Samsung): - The L1 CLI measurement requirements are applicable provided that L1 CLI measurement resource are measurable - Proposal 3 (ZTE): - The following applicability rules are specified for UE-to-UE CLI measurement: - For SRS-RSRP resource, it should be within the UL active BWP, UE could measure the SRS-RSRP resource within the UL active BWP. - For CLI-RSSI resource - For CLI-RSSI measurement within UL SB, no restriction on resource configuration, but UE only measure the resource within UL active BWP. - For CLI-RSSI measurement within DL SB, no matter the CLI-RSSI resource cross two DL SBs or not, UE measures the CLI-RSSI resource within one or two DL SBs, no restriction on resource configuration. - No gap is needed for UE-to-UE CLI measurement - Proposal 4 (MTK): - L1 CLI measurement resources must be within the DL active BWP of the UE, where L1-SRS-RSRP confined within UL subband, while L1-CLI-RSSI based method #1 and #3 to be confined within DL and UL subband, respectively. - Recommended WF - P1 and P3 are similar in that the L1 CLI measurement resources are limited to be within active BWP, while in P4 the L1 CLI measurement resources are limited to be within UL or DL subband. It is suggested to agree on P1 based on majority view. - P2 is for a separate issue, and it is suggested to agree on P2 as it is quite straightforward.

For requirement applicability, we think R16 applicability can be reused, the L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency:

- when SRS-RSRP measurement resource is fully confined within BW of DL active BWP.

- when CLI-RSSI measurement resource is configured within active BWP.

Proposal 1: The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency:

  - when SRS-RSRP measurement resource is fully confined within BW of DL active BWP.

  - when CLI-RSSI measurement resource is configured within active BWP.

L1-SRS-RSRP

Status in the WF [1]:

#### Issue 1-2-3: Measurement restriction - Agreements - Measurement requirements do not apply for L1-SRS-RSRP when the measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps. - When measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI, FFS the following options - Option 1: Prioritize L1-SRS-RSRP measurement - Option 2: Prioritize L1-CLI-RSSI measurement - Option 3: Leave it to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI

For collision between L1-SRS-RSRP and L1-CLI-RSSI, when measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI, since measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are both used to UE-to-UE CLI handling, it’s preferred to leave it to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI.

Proposal 2: When measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI, leave it to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI.

L1-CLI-RSSI

Status in the summary [2]:

- Issue 1-3-5: Measurement accuracy - Proposals for DUD case with method #1 and resource type#2 - Proposal 1 (CATT): - Measurement accuracy requirements are defined by considering both DL subbands. - Proposal 2 (Nokia, Samsung, HW): - Measurement accuracy requirements are same as Method #1 with resource type#1 and as Method #3. - Proposal 3 (ZTE): - Similar as the condition defined in legacy CLI-RSSI accuracy, as long as the conditions for reference sensitivity are fulfilled, no impact on the accuracy. - Recommended WF - It is suggested to further discuss in performance part whether existing CLI-RSSI accuracy requirements can be re-used for L1-CLI-RSSI with - Method 1 and resource type#1 - Method 1 and resource type#2 - Method 3

For measurement accuracy for L1-CLI-RSSI DUD case, UE shall measure both DL subbands and report a wideband measurement report, in our views, measurement accuracy is not impacted by two separate DL subbands, therefore, measurement accuracy requirements can be the same for Method #1 resource type#1, Method #1 resource type#2 and Method #3.

Proposal 3: For measurement accuracy for L1-CLI-RSSI DUD case, measurement accuracy requirements can be the same for Method #1 resource type#1, Method #1 resource type#2 and Method #3.

- Conclusion

The contribution presents our views on RRM requirements for UE-to-UE CLI handling for evolution of NR duplex operation, with the following proposals:

Proposal 1: The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency:

  - when SRS-RSRP measurement resource is fully confined within BW of DL active BWP.

  - when CLI-RSSI measurement resource is configured within active BWP.

Proposal 2: When measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI, leave it to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI.

Proposal 3: For measurement accuracy for L1-CLI-RSSI DUD case, measurement accuracy requirements can be the same for Method #1 resource type#1, Method #1 resource type#2 and Method #3.

- References

- R4-2504904, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, RAN WG4 Meeting #114bis, 7 – 11 April, 2025.

- R4-2503623, Topic summary for [114bis][213] NR_duplex_evo, Huawei, HiSilicon, RAN WG4 Meeting #114bis, 7 – 11 April, 2025.