# 3GPP TSG-RAN WG4 Meeting #115	R4-2506339

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.25.5

Source:	China Telecom

Title:	Discussion on RRM performance requirements for evolution of NR duplex operation

Document for:	Discussion

- Introduction

In this paper, we present our initial discussion on RRM performance requirements for evolution of NR duplex operation.

- Discussion

Based on the core part discussion in RAN4 previous meetings, there are two objectives been discussed and defined, i.e., RRM requirements for UE-to-UE CLI handling and RRM impacts for SBFD operation.

For RRM requirements for UE-to-UE CLI handling, L1-SRS-RSRP measurements and L1-CLI-RSSI measurements are considered for UE-to-UE CLI handling, it’s proposed to design UE-to-UE CLI handling test cases based on L1-SRS-RSRP measurements and L1-CLI-RSSI measurements.

L3 SRS-RSRP measurements and L3 CLI-RSSI measurements test cases have been designed in the spec and can be used as baseline for test cases design of L1-SRS-RSRP measurements and L1-CLI-RSSI measurements. In our views, test cases for FR1 and FR2 can be considered.

Proposal 1: It’s proposed to design UE-to-UE CLI handling test cases based on L1-SRS-RSRP measurements and L1-CLI-RSSI measurements.

Proposal 2: L3 SRS-RSRP measurements and L3 CLI-RSSI measurements test cases can be used as baseline for test cases design of L1-SRS-RSRP measurements and L1-CLI-RSSI measurements.

Proposal 3: For RRM requirements for UE-to-UE CLI handling, test cases for FR1 and FR2 can be considered.

For RRM impacts for SBFD operation, scenarios including CSI-RS colliding with UL transmission, CSI-RS puncturing in frequency domain in SBFD symbols and CSI-RS distributed over SBFD and non-SBFD symbols are considered in the core part discussion, and impact of SBFD operation for CSI-RS based L1-RSRP/ L1-SINR measurement and CSI-RS based RLM/ BFD/ CBD evaluation are to be defined in the core part, related test cases need to be designed in performance part. Besides, test cases for FR1 and FR2 can be considered, and test cases with DRX and non-DRX mode can be considered.

Proposal 4: For RRM impacts for SBFD operation, it’s proposed to define test cases for the following:

- CSI-RS based L1-RSRP measurement with impact of SBFD operation;

- CSI-RS based L1-SINR measurement with impact of SBFD operation;

- CSI-RS based RLM evaluation with impact of SBFD operation;

- CSI-RS based BFD evaluation with impact of SBFD operation;

- CSI-RS based CBD evaluation with impact of SBFD operation.

Proposal 5: For RRM impacts for SBFD operation, test cases for FR1 and FR2 can be considered.

Proposal 6: For RRM impacts for SBFD operation, test cases with DRX and non-DRX mode can be considered.

- Conclusion

The contribution presents our views on RRM performance requirements for evolution of NR duplex operation, with the following proposals:

Proposal 1: It’s proposed to design UE-to-UE CLI handling test cases based on L1-SRS-RSRP measurements and L1-CLI-RSSI measurements.

Proposal 2: L3 SRS-RSRP measurements and L3 CLI-RSSI measurements test cases can be used as baseline for test cases design of L1-SRS-RSRP measurements and L1-CLI-RSSI measurements.

Proposal 3: For RRM requirements for UE-to-UE CLI handling, test cases for FR1 and FR2 can be considered.

Proposal 4: For RRM impacts for SBFD operation, it’s proposed to define test cases for the following:

- CSI-RS based L1-RSRP measurement with impact of SBFD operation;

- CSI-RS based L1-SINR measurement with impact of SBFD operation;

- CSI-RS based RLM evaluation with impact of SBFD operation;

- CSI-RS based BFD evaluation with impact of SBFD operation;

- CSI-RS based CBD evaluation with impact of SBFD operation.

Proposal 5: For RRM impacts for SBFD operation, test cases for FR1 and FR2 can be considered.

Proposal 6: For RRM impacts for SBFD operation, test cases with DRX and non-DRX mode can be considered.