# 3GPP TSG-RAN WG4 Meeting #115	R4-2506340

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.27.3.1

Source:	China Telecom

Title:	Discussion on On-demand SSB SCell operation for enhancements of NES for NR

Document for:	Discussion

- Introduction

In RAN4 #114bis meeting, On-demand SSB SCell operation for enhancements of network energy savings for NR was discussed and the WF was approved [1].

In this paper, we present our views on On-demand SSB SCell operation for enhancements of network energy savings for NR issues captured in the WF [1], summary [2] and ad-hoc minutes [3].

- Discussion

OD-SSB based deactivated SCell L3 measurement procedure (Case 1)

Status in the WF [1]:

Issue 1-1-2: OD-SSB based deactivated SCell measurement – Procedure - Background - The deactivated SCell measurement requirement in FR1 is as follow. DRX cycle; T SSB_measurement_period_intra. No DRX; Ceil(5 x Kp) x measCycleSCell x CSSFintra. DRX cycle≤ 320ms; Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra. DRX cycle> 320ms; Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra. - The OD-SSB based deactivated SCell measurement procedure is shown as follow. Sub-issue 1: The start point of T2 Wayforward The start point of T2 is - Option 1: - The time point of OD-SSB deactivation - Option 2: - The time point of Tend, i.e. the time point of OD-SSB activation plus Tuncertain +Tidentify. - Option 2a: - The time point of OD-SSB activation Sub-issue 2: The minimum interval(T2) between multiple FMWs Wayforward - Option 1: 5* measCycleSCell - Option 1a: 5*(max(measCycleSCell, DRX cycle) – OD-SSB periodicity) - Option 1b: 5*max (measCycleSCell, DRX cycles) - Option 2: Minimum measurement period defined in existing spec. - Option 3: The time period of known Scell condition but with OD-SSB periodicity for FR1 Sub-issue 3 –UE’s behaviour when OD-SSB further activation/reconfiguration within T2 Wayforward - Option 1: The UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement. - Option 2: The UE is assumed to perform fast mode measurement without cell identification period. - Option 3: No requirement for UE within T2.

For the start point of T2, it’s preferred to have a fixed point while dynamic point needs to be avoided to make it’s clear for NW and UE, therefore, the time point of OD-SSB deactivation is preferred since it’s clear for NW and UE. The minimum interval(T2) between multiple FMWs can be defined as 5* max(measCycleSCell, DRX cycles).

When NW re-activates the OD-SSB within the T2, we understand that it’s a little frequently to re-activate the OD-SSB within the T2, thus we are ok for option 1 that UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement.

Proposal 1: The start point of T2 is the time point of OD-SSB deactivation.

Proposal 2: The minimum interval(T2) between multiple FMWs can be defined as 5* max(measCycleSCell, DRX cycles).

Proposal 3: When NW re-activates the OD-SSB within the T2, UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement.

OD-SSB based deactivated SCell L3 measurement (Case 2)

Status in the WF [1]:

Issue 1-2-3: Requirement for RAN1 Alt Time-C2 Agreement - Option 1: Define the requirement based on OD-SSB periodicity. - Option 2: Not to define requirement. Issue 1-2-4: Requirement for scenario 3 (OD-SSB and always-on SSB are within different frequencies) Agreement RAN4 to first discuss the RAN4 impact based on RAN2 LS R2-2503015. - Option 1: RAN4 to define requirement for different frequencies - Option 2: RAN4 not to define requirements for different frequencies

If UE support the capability of RAN1 Alt Time-C2, the requirement can be based on OD-SSB periodicity to simplify the definition of measurement requirement for Alt Time-C2.

If UE support the capability of On-demand SSB SCell operation indicated by RRC based signaling in Case #2 for different center frequencies, since RAN2 also has the work assumption that when A-SSB and OD-SSB have different center frequencies, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, it’s proposed to define requirement for different frequencies.

Proposal 4: RAN4 to define the requirement for RAN1 Alt Time-C2 based on OD-SSB periodicity.

Proposal 5: RAN4 to define requirement for OD-SSB and always-on SSB with different frequencies.

OD-SSB based SCell activation

Status in the WF [1]:

Issue 1-3-2: The known cell condition – period of known condition - Wayforward Reuse the legacy known/unknown cell condition period for FR1 and FR2, i.e. max(5*measCycleSCell, 5*DRX cycles) in FR1 or 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 in FR2.

UE performs deactivated SCell measurement based on OD-SSB within a time window, and after the time window, UE performs deactivated SCell measurement with a slow mode, which means UE shall meet legacy deactivated SCell measurement requirement after the time window. Therefore, it’s better to reuse the legacy period of known/unknown cell condition, i.e., max(5*measCycleSCell, 5*DRX cycles) in FR1, 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 in FR2, to cover both the case within the time window and the case after the time window.

Proposal 6: The time period in OD-SSB SCell activation known condition reuses the time period in legacy known/unknown cell condition:

- max(5*measCycleSCell, 5*DRX cycles) in FR1;

- 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 in FR2.

OD-SSB general issues

Status in the WF [1]:

Issue 1-4-1: Additional Processing time for OD-SSB reception – scenario Issue 1-4-2: Additional Processing time for OD-SSB reception - value - Background In RAN4 #113 meeting, RAN4 achieved agreements as follow. Agreement: - RAN4 to confirm that T_min equals to m+3Nslotsubframe,μ+1 from network transmission perspective. - Note 1: This does not imply UE shall be able to process/prepare the OD-SSB from time instance A from RAN4 requirement perspective. - Meanwhile, RAN4 is discussing the additional UE processing/preparation time for OD-SSB reception for RAN4 requirement.. - Information RAN1 #120-bis Agreement - Upon the reception of MAC CE for deactivating on-demand SSB, UE expects that on-demand SSB is NOT transmitted from time instance B. - Time instance B’ is T slots after the slot where UE receives a signalling from gNB to deactivate on-demand SSB. - If time instance B’ falls within an on-demand SSB burst, time instance B is the ending of the slot containing the last actually transmitted SSB index within the on-demand SSB burst, otherwise, time instance B = time instance B’. - As agreed in previous RAN1 meeting, the SSB time domain positions of on-demand SSB burst are configured by gNB. - As agreed in previous RAN1 meeting, the location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - (Working assumption) T is not less than T_min=m+3Nslotsubframe,μ. Agreement UE applies the additional processing timing only in deactivated SCell measurement. - Note: The additional UE processing timing is not applied to SCell activation procedure. The additional processing timing is T_processing, the value of T_processing: - Introduce different values with UE capability: [2 or 3]ms and 5ms.

For the value of T_processing, since it was agreed to introduce different values with UE capability for the value of [2 or 3]ms and 5ms, and 5ms is by default when no UE capability is reported, therefore, 2ms is preferred for the new optional UE capability to achieve better delay performance.

Proposal 7: The value of T_processing can be 2ms and 5ms.

Status in the WF [1]:

Issue 1-4-3: Additional Processing time for OD-SSB deactivation Wayforward - FFS RAN4 to consider defining the same timeline for activating and deactivating OD-SSB

It was agreed the additional processing timing is only in deactivated SCell measurement, and the additional UE processing timing is not applied to SCell activation procedure. Besides, if there is the additional processing time for OD-SSB deactivation, we think the additional processing time can be included in legacy deactivation delay requirements. Therefore, additional processing time for OD-SSB deactivation is not needed.

Proposal 8: Additional processing time for OD-SSB deactivation is not needed.

- Conclusion

The contribution presents our views on On-demand SSB SCell operation for enhancements of network energy savings for NR, with the following proposals:

Proposal 1: The start point of T2 is the time point of OD-SSB deactivation.

Proposal 2: The minimum interval(T2) between multiple FMWs can be defined as 5* max(measCycleSCell, DRX cycles).

Proposal 3: When NW re-activates the OD-SSB within the T2, UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement.

Proposal 4: RAN4 to define the requirement for RAN1 Alt Time-C2 based on OD-SSB periodicity.

Proposal 5: RAN4 to define requirement for OD-SSB and always-on SSB with different frequencies.

Proposal 6: The time period in OD-SSB SCell activation known condition reuses the time period in legacy known/unknown cell condition:

- max(5*measCycleSCell, 5*DRX cycles) in FR1;

- 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 in FR2.

Proposal 7: The value of T_processing can be 2ms and 5ms.

Proposal 8: Additional processing time for OD-SSB deactivation is not needed.

- References

- R4-2504906, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN WG4 Meeting #114bis, 7 – 11 April, 2025.

- R4-2503625, Topic summary for [114bis][215] Netw_Energy_NR_enh_Part1, Ericsson, RAN WG4 Meeting #114bis, 7 – 11 April, 2025.