# 3GPP TSG-RAN WG4 Meeting #115	R4-2506342

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.27.3.2

Source:	China Telecom

Title:	Discussion on adaptation of common signal channel transmission for enhancements of NES for NR

Document for:	Discussion

- Introduction

In RAN4 #114bis meeting, adaptation of common signal/channel transmission for enhancements of network energy savings for NR was discussed and the WF was approved [1].

In this paper, we present our views on adaptation of common signal/channel transmission for enhancements of network energy savings for NR issues captured in the WF [1] and summary [2].

- Discussion

SSB adaptation – L1 Requirements impacts

Status in the summary [2]:

Issue 1-2-2: L1 requirements after transition - DRX - Proposals - Option 1: For L1-RSRP measurement based on the SSB periodicity after transition, if the adapted SSB periodicity is less than DRX cycle, UE to measure the SSB resource for adaptation using the measurement period corresponding to non-DRX mode. (CTC, Samsung) - Option 2: keep (TDRX,TSSB) when DRX cycle≤320 ms in the L1-RSRP for serving cell. (Vivo) - Recommended WF: - Discuss above options and whether there is RRM requirements impacts in terms of DRX configurations.

For L1-RSRP measurement based on the SSB periodicity after transition, if the adapted SSB periodicity is less than DRX cycle, in order to guarantee the benefits of network energy saving with SSB adaptation, we support UE to measure the SSB resource for adaptation using the measurement period corresponding to non-DRX mode.

Proposal 1: For L1-RSRP measurement based on the SSB periodicity after transition, if the adapted SSB periodicity is less than DRX cycle, UE to measure the SSB resource for adaptation using the measurement period corresponding to non-DRX mode.

Status in the summary [2]:

Issue 1-2-4: L1 requirements – Measurement restriction impact - Proposals - Option 1: RAN4 to discuss whether existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS. i.e. Whether UE can choose to measure one of them or only to measure SSB. (Huawei) - Recommended WF: - Discuss above option.

In our understanding, it can be guaranteed that no collision exists between CSI-RS and SSB before SSB adaptation, however, after SSB adaptation, the adapted SSB may collide with CSI-RS, therefore, whether existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS needs to be discussed. In our views, in order to reduce the impact of legacy CSI-RS measurement, existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS, i.e., to measure one of adapted SSB and CSI-RS.

Proposal 2: When adapted SSB colliding with CSI-RS, existing L1 measurement restriction can apply.

SSB adaptation – L3 Requirements impacts

Status in the WF [1]:

Issue 1-3-1: L3 requirements for SSB adaptation Agreement: - RAN4 to define requirements based legacy L3 measurement framework for SSB adaptation. - For neighbour cell L3 measurement, legacy requirements apply. Open issue for further discuss: - For SCell with SSB adaptation (deactivated SCell or activated SCell), wait for more RAN2 progress on additional SMTC configuration, and discuss following aspects: - Whether to define L3 measurement requirement at transition. - Any other RRM requirements impacts due to additional SMTC configurations.

It was agreed in RAN2 meeting that for SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC together with the mapping between SSB burst periodicity and SMTC configuration, upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly. In our views, based on RAN2 agreement, after SSB adaptation, UE performs L3 measurement based on updated SMTC with the mapping between SSB burst periodicity and SMTC configuration.

In our understanding, regarding whether to define L3 measurement requirement at transition, it’s proposed to define L3 measurement requirement at transition, L3 measurement period is the maximum of the period corresponding to the SSB before and after adaptation.

Proposal 3: After SSB adaptation, UE performs L3 measurement based on updated SMTC with the mapping between SSB burst periodicity and SMTC configuration.

Proposal 4: It’s proposed to define L3 measurement requirement at transition, L3 measurement period is the maximum of the period corresponding to the SSB before and after adaptation.

SSB adaptation – Other requirements impacts

Status in the summary [2]:

Issue 1-4-1: SCell activation requirement for SSB adaptation – SSB adaptation command related - Proposals - Option 1: Define the SCell activation requirements based on the SSB periodicity after SSB adaptation. (Xiaomi, CMCC) - The start point can be the slot UE receiving SCell activation command if UE receiving SSB adaptation command (MAC) is not later than it - Option 2: For SCell activation requirements for SSB adaptation, RAN4 to wait for more progress from RAN1/2 about triggering command and discuss the conditions that “SSB adaption command and SCell activation command comes together. (Huawei) - Option 3: If separate MAC CE to activate SSB adaptation and SCell, define the SCell activation requirements based on the SSB periodicity after SSB adaptation. (Samsung) - For the case SSB adaptation command comes together with SCell activation command - Option 4: If MAC CE to activate SCell but group-common DCI to activate SSB adaptation, RAN4 to discuss the feasibility of defining the SCell activation requirements for the case SSB adaptation command comes together with SCell activation command. (Samsung) - Option 5: RAN4 to consider the scenario for SSB adaptation command sent together with direct SCell activation command and investigate the impact to SCell addition requirements and align them to OD-SSB based direct SCell activation as much as possible. (Nokia) - Recommended WF: - RAN4 to discuss the conditions for SCell activation with SSB adaptation (i.e. based SSB periodicity after SSB adaptation), considering following aspects: - SSB adaptation command (e.g. DCI/MAC-CE/RRC), which subject to conclusion from other WG - Time relation between SSB adaptation command and SCell activation command.

It was agreed in RAN2 meeting that not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1. Since SCell activation command is based on MAC CE message, SSB adaptation command is based on DCI message, it’s difficult to align the time point for MAC CE command and DCI command. In our views, for the conditions for SCell activation with SSB adaptation, time relation between SSB adaptation command and SCell activation command can be that SSB adaptation command comes no later than SCell activation command.

Proposal 5: For the conditions for SCell activation with SSB adaptation, SSB adaptation command comes no later than SCell activation command.

- Conclusion

The contribution presents our views on adaptation of common signal/channel transmission for enhancements of network energy savings for NR, with the following proposals:

Proposal 1: For L1-RSRP measurement based on the SSB periodicity after transition, if the adapted SSB periodicity is less than DRX cycle, UE to measure the SSB resource for adaptation using the measurement period corresponding to non-DRX mode.

Proposal 2: When adapted SSB colliding with CSI-RS, existing L1 measurement restriction can apply.

Proposal 3: After SSB adaptation, UE performs L3 measurement based on updated SMTC with the mapping between SSB burst periodicity and SMTC configuration.

Proposal 4: It’s proposed to define L3 measurement requirement at transition, L3 measurement period is the maximum of the period corresponding to the SSB before and after adaptation.

Proposal 5: For the conditions for SCell activation with SSB adaptation, SSB adaptation command comes no later than SCell activation command.

- References

- R4-2504907, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei, RAN WG4 Meeting #114bis, 7 – 11 April, 2025.

- R4-2503626, Topic summary for [114bis][216] Netw_Energy_NR_enh_Part2, Huawei, RAN WG4 Meeting #114bis, 7 – 11 April, 2025.