3GPP TSG-RAN WG4 Meeting #115	R4-2506349

St Julian’s, Malta, 19th – 23rd May 2025

Source: 	Ericsson

Title:  	Discussion on RRM requirements of NTN with channel BW less than 5MHz

Agenda Item:	7.10.4.5

Document for:	Discussion

# Introduction

In the WI NR Enhanced requirements and test methodology for NR and IoT NTN, the objective: RRM of NTN with channel BW less than 5MHz was discussed in the last meeting, the consensuses and unresolved issues, were documented in WF [1].

This contribution outlines our point of view on the issues concerning performance requirements as follow.

# Discussion

### Issue 2-2-2: Test cases to verify the core requirements reusing NTN

Agreement: - Not define the new test cases for the following requirements RRM requirements; NTN requirements in TS38.133 v18.5.0. RRC connection mobility control; 6.2C RRC Connection Mobility Control for Satellite Access. UE Tx timing, MTTD/ MRTD, timer accuracy, TA accuracy; 7.1C, 7.2C, 7.3C. - Further discuss whether to define the new test case for the following requirement IDLE/inactive mode mobility; 4.2C Cell Re-selection for NR UE for Satellite Access.

Recall the online discussion content in the last meeting, there is an additional period for less than 5MHz, which needs to be extended in the test case as what is done in TN.

Proposal 1: Support to define the new test case of cell reselection for less than 5MHz in NTN, following the same additional period as for less than 5MHz in TN.

### Issue 2-2-3 Test cases to verify the new core requirements

Agreement: - Define the new test cases for the core requirements for NTN less than 5MHz introduced in Rel19.e.g. - HO-1: Intra-frequency HO, [unknown cell] - CHO-1: [Inter-frequency CHO], time based with L3 measurement - SatSwitch-1: Hard switch - RLM-1: RLM OOS for 15RB - RLM-2: RLM IS for 15RB - BFR-1: BFR - MR-1: Intra-frequency with SSB index reading - [MR-2: Inter-frequency without SSB index reading] - FFS the detailed configuration for each test case

Below are the test cases for less than 5MHz in TN appearing in the existing TS 38.133 V19.0.0.

- A.6.1.1.9	Cell reselection to FR1 intra-frequency NR case for UE operating on a cell with less than 5 MHz BW

- A.6.3.1.18	Intra-frequency handover from FR1 to FR1; unknown target cell operating with 12 PRB SSB bandwidth

- A.6.5.5.8	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for a UE operating on a cell with less than 5 MHz BW

- A.6.5.1.13	 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for UE operating on a cell with less than 5 MHz BW

- A.6.5.1.14	 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for UE operating on a cell with less than 5 MHz BW

- A.6.6.2.15	 (inter) SA event triggered reporting tests for FR1 with 3 MHz Channel Bandwidth configured without SSB time index detection when DRX is used

- A.6.6.1.12 (intra) SA event triggered reporting tests without gap under non-DRX with SSB index reading and 12 PRB SSB

In a similar manner, taking the agreed test cases into consideration, the test cases and configurations concerning less than 5MHz in NTN may be defined as follow

- Cell reselection

- HO-1: Intra-frequency HO, unknown cell

- CHO-1: Inter-frequency CHO, time based with L3 measurement

- SatSwitch-1: Hard switch

- RLM-1: RLM OOS for 15RB, in DRX mode

- RLM-2: RLM IS for 15RB, in non-DRX mode

- BFR-1: BFR, in non-DRX mode

- MR-1: Intra-frequency, without gap under non-DRX with SSB index reading

- MR-2: Inter-frequency, without SSB time index detection when DRX is used

Proposal 2: Define the new test cases and corresponding configurations for the core requirements for NTN less than 5MHz:

- Cell reselection:

  - Intra-frequency NR case,

    - Normal and enhanced requirements

    - Time-based

- HO-1: Intra-frequency HO,

  - Unknown cell

- CHO-1: Inter-frequency CHO,

  - Time-based with L3 measurement

- SatSwitch-1:

  - RACH-based hard switch

- RLM-1: RLM OOS for 15RB,

  - DRX mode

- RLM-2: RLM IS for 15RB,

  - Non-DRX mode

- BFR-1: BFR,

  - Non-DRX mode

- MR-1: Intra-frequency,

  - Without gap under non-DRX with SSB index reading

- MR-2: Inter-frequency,

  - Without SSB time index detection when DRX is used

# Conclusion

Proposal 1: Support to define the new test case of cell reselection for less than 5MHz in NTN, following the same additional period as for less than 5MHz in TN.

Proposal 2: Define the new test cases and corresponding configurations for the core requirements for NTN less than 5MHz:

- Cell reselection:

  - Intra-frequency NR case,

    - Normal and enhanced requirements

    - Time-based

- HO-1: Intra-frequency HO,

  - Unknown cell

- CHO-1: Inter-frequency CHO,

  - Time-based with L3 measurement

- SatSwitch-1:

  - RACH-based hard switch

- RLM-1: RLM OOS for 15RB,

  - DRX mode

- RLM-2: RLM IS for 15RB,

  - Non-DRX mode

- BFR-1: BFR,

  - Non-DRX mode

- MR-1: Intra-frequency,

  - Without gap under non-DRX with SSB index reading

- MR-2: Inter-frequency,

  - Without SSB time index detection when DRX is used

# References

- R4-2502589, WF for RRM of NTN with channel BW less than 5MHz, Xiaomi