3GPP TSG-RAN WG4 Meeting #115	R4-2506355

St Julian’s, Malta, 19th – 23rd May 2025

Source: 	Ericsson

Title:  	Discussion on other RRM requirements for NTN for NR Phase 3

Agenda Item:	7.31.5.2

Document for:	Discussion

# Introduction

In the light of the WI NR Non-Terrestrial Networks (NTN) for NR Phase 3, the outcomes of the last meeting's debate, encompassing consensuses and unresolved issues, were documented in WF [1].

This contribution outlines our point of view on on the underlying impacts to RRM requirements mandated by the issues listed in the WF.

# Discussion

## Topic #1: SSB periodicity larger than 20 ms

## Issue 1-2-A: RRC Re-establishment (default TSMTC,i)

Agreement (online): - In RRC re-establishment requirements, for case where the SMTC for the inter-frequency carrier is not provided, the value of TSMTC,i is assumed to be 160ms: - Discuss the applicability of the requirement for with the updated value of TSMTC,i to 160ms after the related RAN1/2 agreement. FFS: Wait for RAN2 progress and discuss the following - The requirements for intra-frequency and inter-frequency RRC re-establishment are applicable provided that the configured SMTC period is not shorter than the periodicity of the actual SSB transmission of the target cell.

We quote RAN2’s agreements in the below.

RAN2#129:

- RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer - RAN2 assumes that in a NR NTN cell, SSB beam sweeping in different spatial directions is possible as in a NR TN cell: the whole cell is covered by the different SSB beams in half-frame - RAN2 also assumes that, with the current status of RAN1 discussion, if one cell is defined by multiple “satellite beams”, the satellite beams are all simultaneously active or inactive (“beam hopping” applies equally to all the satellite beams of a given cell) - The number of SMTC/gaps a UE needs to consider at any time will not be increased further

RAN2#129i:

- From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits - We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring - RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases). - We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking) - Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

In the light of RAN2’s proposal to introduce SMTCs with different periodicities in same frequency layer, there’s no necessity to further check the condition of the configured SMTC period is not shorter than the periodicity of the actual SSB transmission of the target cell. The network shall be able to manage proper SMTCs and the contained SSBs in the SMTCs respectively.

Proposal 1: No need to consider the condition that the configured SMTC period is not shorter than the periodicity of the actual SSB transmission of the target cell, if different SMTC periodicities are appliable.

The cause regarding the applicability of the requirement for with the updated value of TSMTC,i to 160ms arises in the context of initial access extension. It seems appear that the RRC re-establishment procedure occurs after the UE has already completed the initial cell selection process, successfully detecting and decoding the SS/PBCH block. Consequently, if the UE is required to perform an RRC connection re-establishment procedure, the expectation is that the configured periodicity should align with the SS/PBCH block periodicity used during initial cell selection. That said, there exists a problem that the SSB periodicity of neighbor cells may not be same as the serving cell that served the UE. In this case, the UE may use 160ms periodicity measurement opportunities to detect 20ms initial access.

Observation 1: Depending on network implementation, 20ms or 160ms both may coexist in neighbour cells in practice.

Proposal 2: For RRC connection re-establishment to NR inter-frequency cell, the SSB periodicity is assumed same as the SSB periodicity used during initial cell selection, provided that same SSB periodicity is applied for neighbour cells.

Proposal 3: For RRC connection re-establishment to NR inter-frequency cell, if different SSB periodicity is applied for neighbour cells, i.e., some cells apply DL-CE and others don’t apply DL-CE, 160ms is assumed.

## Issue 1-2-B: RRC Re-establishment (SSB Ês/Iot < -8dB and SSB periodicity >20 ms)

Agreement (online): - In RRC re-establishment requirements, for the case SSB Ês/Iot < -8dB and unknown NR cell, - Make the following updates - Remove Note 1 in Table 6.2C.1.2.1-1 and Table 6.2C.1.2.1-2 for the applicable scenarios, and the value of TSMTC and TSMTC,i is FFS - Tidentify_intra_NR: MAX (800 ms, [22] x TSMTC) for intra-frequency carrier - Tidentify_inter_NR,i: MAX (800 ms, [22] x TSMTC,i) for inter-frequency carrier

We’re open to the exact number in the bracket, but a concern is that a UE may spend 22*160ms, almost 1 minute, to detect a cells, the long detection time may not help for a moving satellite case.

Proposal 4: Differentiate RRC Re-establishment requirements for GSO and NGSO satellite cases:

For GSO satellite,

- Tidentify_intra/inter_NR: MAX (800 ms, [22] x TSMTC) for intra-frequency carrier

For NGSO satellite,

- Tidentify_ intra/inter _NR: MAX (800 ms, [17] x TSMTC) for intra-frequency carrier

# Conclusion

Observation 1: Depending on network implementation, 20ms or 160ms both may coexist in neighbour cells in practice.

Proposal 1: No need to consider the condition that the configured SMTC period is not shorter than the periodicity of the actual SSB transmission of the target cell, if different SMTC periodicities are appliable.

Proposal 2: For RRC connection re-establishment to NR inter-frequency cell, the SSB periodicity is assumed same as the SSB periodicity used during initial cell selection, provided that same SSB periodicity is applied for neighbour cells.

Proposal 3: For RRC connection re-establishment to NR inter-frequency cell, if different SSB periodicity is applied for neighbour cells, i.e., some cells apply DL-CE and others don’t apply DL-CE, 160ms is assumed.

Proposal 4: Differentiate RRC Re-establishment requirements for GSO and NGSO satellite cases:

For GSO satellite,

- Tidentify_intra/inter_NR: MAX (800 ms, [22] x TSMTC) for intra-frequency carrier

For NGSO satellite,

- Tidentify_ intra/inter _NR: MAX (800 ms, [17] x TSMTC) for intra-frequency carrier

# References

- R4-2504913, The WF on RRM requirements for Rel-19 NR NTN phase3, QC