3GPP TSG-RAN WG4 Meeting #115	R4-2506358

St Julian’s, Malta, 19th – 23rd May 2025

Source: 	Ericsson

Title:  	Reply LS to RAN2 on SMTC enhancement

Agenda Item:	7.31.5.2

Document for:	Discussion

# Introduction

In RAN2 #129bis meeting, a LS was sent to RAN4 [1]:

… # Overall description For downlink coverage enhancement, RAN2 assumes that it will be possible to have different SSB periodicities among neighbour cells in the same frequency layer, and for the case of intra-frequency neighbouring cells, the SSBs of surrounding neighbour cells may be transmitted at different times. Thus, RAN2 made the following agreements, given that at any time the UE will not use more SMTCs/Gaps in parallel than in previous releases: - RAN2 considers to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements. - RAN2 agrees to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs. RAN2 would like RAN4 to provide feedback on whether it is feasible to support these agreements in Rel-19 timeframe. # Actions To RAN4: ACTION: RAN2 kindly asks RAN4 to provide feedback on the feasibility to support those agreements in Rel-19 timeframe. …

This paper presents our analysis of LS.

# 2	Discussion

In the light of RAN2’s assumption, different SSB periodicities among neighbor cells in the same frequency layer. It can be interpreted that there are at the least two cells (the serving cell and a neighbour cell or two neighbour cells) served by same or different satellites, one operates with 20ms SSB periodicity without DL-CE and the other one operates with 160ms SSB periodicity with DL-CE.

In this case, two different SMTC periodicities, can contain respective SSB with different periodicities, e.g., 20ms and 160ms, may be configured to a UE, below cases may be considered.

- For connected mode, at the least one SMTC periodicity may be configured for the UE provided the network is aware of the proper SMTC offset for the UE with respect to UE’s location reporting. The UE may follow the SMTC periodicities and offsets for measurements.

- For idle mode, more than one SMTC may be configured for the UE since the network doesn’t know which SMTC and the corresponding SSB can be detected by the UE. According to the below reference of RAN2’s agreements, the UE may acquire some assistance information to identify which SMTC shall be measured.

Agreements in RAN2#129bis are duplicated in the below.

- From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits - We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring - RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases). - We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking) - Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

The association between SMTC and location may lead to UE implementation-specific optimization, but it does not necessitate any modification to the existing RRM requirements. This is because the procedure for UE handling in such scenarios is already specified in Clause 4.2C.2.3 of TS 38.133, as quoted below, can cover such UE implementation.

The requirements in this clause apply provided that the number of SMTCs for intra-frequency carrier does not exceed the parallelSMTC-r17, otherwise UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. The selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in table 4.2C.2.3-1 and table 4.2C.2.3-2 is expected.

For same reason, regarding the proposal requesting more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs, we don’t see it is a problem for an NTN UE.

In spite of that, it is noted that the legacy UE capability supports up to 4 SMTCs per frequency layer, where the 4 SMTCs have same periodicity, but now RAN2 proposes that [6] SMTCs per frequency layer may be configured with 2 periodicities. This update raises a question: among the [6] SMTCs, can up to 4 SMTCs with same SMTC periodicity or up to [6] SMTCs with same periodicity be configured per frequency layer? We prefer the first option because it’s impact to legacy requirement is minimized and such limitation actually doesn’t conflict with the DL-CE purpose.

Proposal 1: RAN4 to confirm that the below configurations, as stated in RAN LS, are supported.

- Configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements.

- Configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs.

Proposal 2: Among [6] SMTCs per frequency layer, RAN4 may determine whether the [6] SMTCs can be configured with a same SMTC periodicity.

Proposal 3: RAN4 to study intra-frequency layer case, no need to take inter-frequency layer into consideration.

The aforementioned reply is primarily based on implementation feasibility considerations. Moreover, from RRM requirements perspective, one of issues by introduction of multiple SMTC periodicities is how the measurement delay is evaluated with respect to more than one SMTC in a frequency layer.

In RRM specification, the measurement delay is defined with respect one SMTC periodicity per frequency layer in the conventional way. For connected/idle mode, to deal with multiple SMTC periodicities per frequency layer, one straightforward solution is to define the measurement delay based on the SMTC with the longest periodicity, e.g., replacing the SMTC with Max(SMTCs). By this, in the measurement delay calculated with the longest SMTC periodicity, the UE may complete all SSB (including 20ms and 160ms) measurements.

Another issue is collision of SMTCs. In the existing specification, the total delay extension by the factor Kmulti_SMTC=i=1NSMTC,overlapNLEO,iNLEO,simul , e.g., for LEO, is defined, as quoted in the below.

Kmulti_SMTC is the scaling factor for measurement of multiple SMTCs or multiple satellites, and if SMTCs within a measurement gap do not overlap with each other, - Kmulti_SMTC=1, if GEO satellites are measured on the carrier; - Kmulti_SMTC=NLEO,iNLEO,simul, if LEO satellites are measured on the carrier; if SMTCs within a measurement gap partially overlap with each other, - Kmulti_SMTC=NSMTC,overlap, if only GEO satellites are measured on the carrier; - Kmulti_SMTC=i=1NSMTC,overlapNLEO,iNLEO,simul, if only LEO satellites are measured on the carrier; where - NLEO,i is the number of LEO satellites to be measured within i-th SMTC, - NLEO,simul is the number of LEO satellites that UE can measure in parallel within an SMTC, - NSMTC,overlap is the number of SMTCs that partially overlap with each other.

For a single SMTC periodicity case, it may be sufficient. But, for two different SMTC periodicities, e.g., 20ms and 160ms, the total measurement delay due to collision between them is unfair to 160ms SSB measurements. For example, skipping a measurement opportunity for an SSB with 160ms periodicity results in an additional delay of 160ms, which is significantly longer compared to the impact of skipping a 20ms SSB opportunity.

Observation 1: More than 4 SMTCs per frequency (i.e. 6) isn’t an issue for idle/inactive UEs, it doesn’t impact the existing specification.

Proposal 4: To deal with multiple SMTC periodicities per frequency layer, measurement delay requirements, which is designed for one frequency layer, shall be updated with the SMTC with the longest periodicity.

- One solution is replacing the SMTC with Max (SMTCs) in all relevant measurement delay requirements,

- Another solution is adding clarification statement in applicability section, indicating that SMTC in measurement delay is one with the longest periodicity.

Proposal 5: RAN4 to enhance requirements concerning SMTC collision in case of different SMTC periodicities in one frequency layer.

# Conclusion

Observation 1: More than 4 SMTCs per frequency (i.e. 6) isn’t an issue for idle/inactive UEs, it doesn’t impact the existing specification.

Proposal 1: RAN4 to confirm that the below configurations, as stated in RAN LS, are supported.

- Configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements.

- Configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs.

Proposal 2: Among [6] SMTCs per frequency layer, RAN4 may determine whether the [6] SMTCs can be configured with a same SMTC periodicity.

Proposal 3: RAN4 to study intra-frequency layer case, no need to take inter-frequency layer into consideration.

Proposal 4: To deal with multiple SMTC periodicities per frequency layer, measurement delay requirements, which is designed for one frequency layer, shall be updated with the SMTC with the longest periodicity.

- One solution is replacing the SMTC with Max (SMTCs) in all relevant measurement delay requirements,

- Another solution is adding clarification statement in applicability section, indicating that SMTC in measurement delay is one with the longest periodicity.

Proposal 5: RAN4 to enhance requirements concerning SMTC collision in case of different SMTC periodicities in one frequency layer.

# References

- R2-2503068, LS on SMTC enhancements, Xiaomi

# Appendix draft LS reply

3GPP TSG-RAN WG4 Meeting #115 																		R4-xxxx

St Julian’s, Malta, 19th – 23rd May 2025

Title:	Reply LS to RAN2 on SMTC enhancements

Response to:	R2-2503068

Release:	Release 19

Work Item:	NR_NTN_Ph3-Core

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact person:

Name:	Ming Li

E-mail Address:	ming.l.li@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1 Overall description

RAN4 would like to thank RAN2 for their LS R2-2503068.

RAN4 confirm to support the below configurations for the UE capable of DL-CE operation.

- Configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements.

- Configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs.

The following clarifications are provided by RAN4 regarding the supported configurations:

- Measurement Delay with Multiple SMTC Periodicities:

For configurations involving different SMTC periodicities within a single frequency layer, RAN4 confirms that the measurement delay is defined based on the maximum of the configured SMTC periodicities. In other words, the presence of a shorter SMTC periodicity does not impact the measurement delay requirements from the RAN4 perspective.

- Number of SMTCs per Frequency for Idle/Inactive UEs:

Regarding the configuration of [6] SMTCs per frequency layer for idle or inactive UEs, RAN4 agrees that up to four SMTCs with the same periodicity may be configured among the [6] SMTCs, in alignment with current measurement capability considerations.

# 2 Actions

To RAN2:

ACTION: 	RAN4 respectfully requests RAN2 to take the above feedback into consideration.

# 3 Dates of next TSG-RAN WG2 meetings