3GPP TSG-RAN WG4 Meeting #115	R4-2506359

St Julian’s, Malta, 19th – 23rd May 2025

Source: 	Ericsson

Title:  	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Agenda Item:	7.18.3.1

Document for:	Discussion

# Introduction

In the light of the WI of NR Radio Resource Management (RRM) enhancements (WID in [1]) for Rel-19, the outcomes of the last meeting's debate, encompassing consensuses and unresolved issues, were documented in WF [2].

This contribution outlines our point of view on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor as follows.

# Discussion

## Topic #1: FR2-1 SSB based L3 measurement delay reduction for connected mode

Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”:

Agreement in ad-hoc session: - UE is not required to perform FBS based fast L3 measurements when threshold is not configured to UE for scenario 1/2/3/4. Further discuss this issue.

Under certain conditions, the UE may exit FBS without an explicit indication from the network after FBS has been enabled. In our view, these conditions should be explicitly defined in the requirements. Without clear specifications, the UE might deactivate FBS arbitrarily, leading to inconsistencies in network configuration and scheduling.

For instance, consider a scenario where the FBS activation condition remains valid, and the UE has completed at least one measurement in the FBS state. Under such circumstances, the UE may determine that there is no urgent need to continue operating in FBS mode, for example, if the UE remains in a stationary state or if there are no qualified neighboring cells available for switching. In such cases, the UE may autonomously exit FBS.

Proposal 1: UE may quit FBS provided the UE has completed at least one measurement in the FBS way and the UE identifies that there isn’t any urgent task to continue FBS, e.g., the UE remains a stationary state, or no qualified neighbour cells are available for switching.

To avoid too frequent ping-pong between FBS and non-FBS, a minimal duration time for FBS/ non-FBS shall be defined.

- FBS deactivation restriction: Before the minimum duration expires, FBS shall not be deactivated, even if the activation condition changes.

- FBS activation restriction: Similarly, if non-FBS is activated, FBS shall not be re-activated unless the non-FBS state has persisted for at least the defined minimum duration.

Proposal 2: UE may quit FBS in certain case upon satisfying certain conditions, but at least after the minimal time period ‘time to deactivate’.

A leftover is, if the UE has quit FBS, whether the UE shall re-evaluate the FBS activation condition to enable FBS?

We understand the issue can be resolved in more than one way. Among them, a simple solution is that UE shall re-evaluate the FBS activation condition which is maintained by the UE. But, as described in the last proposal, a minimal during time in FBS/ non-FBS, to avoid too frequent ping-pong between FBS and non-FBS.

Proposal 3: After deactivating FBS, the UE may re-evaluate the condition of enabling FBS, at least after a minimal time interval ‘time to active’ after deactivated or disabled.

### Issue 1-1-3: Status report of fast beam sweeping

The network is expected to be aware of whether the UE has enabled FBS, it may need the information regarding the UE's activation or entry into the FBS state. One potential solution is to have the UE explicitly report when it satisfies the FBS condition or activates FBS functionality.

Proposal 4: The UE shall notify the network upon entering the FBS state.

Proposal 5: The UE shall notify the network upon exiting the FBS state.

Issue 1-2-1: Applicability of L3 measurement delay reduction by optimizing Rx BSF

In the last meeting, we recapped and agreed that for the baseline scenario agreed in RAN4 #112, limit the serving cell to a FR2 serving cell. With respect to the granularity of capability specification, a preference is defining it on a per-UE basis. However, there may be concerns regarding a UE's ability to support FBS operation in one frequency band but not in another band, such that a per-band capability specification may also be deemed acceptable.

Proposal 6: We prefer per-UE capability, but can accept per-band capability.

# Conclusion

Proposal 1: UE may quit FBS provided the UE has completed at least one measurement in the FBS way and the UE identifies that there isn’t any urgent task to continue FBS, e.g., the UE remains a stationary state, or no qualified neighbour cells are available for switching.

Proposal 2: UE may quit FBS in certain case upon satisfying certain conditions, but at least after the minimal time period ‘time to deactivate’.

Proposal 3: After deactivating FBS, the UE may re-evaluate the condition of enabling FBS, at least after a minimal time interval ‘time to active’ after deactivated or disabled.

Proposal 4: The UE shall notify the network upon entering the FBS state.

Proposal 5: The UE shall notify the network upon exiting the FBS state.

Proposal 6: We prefer per-UE capability, but can accept per-band capability.

# References

- RP-240830, New WID: WI resulting from discussion on RRM enhancements, vivo (Moderator)

- R4-2502593, WF for NR_RRM_Ph5_part2, CATT