3GPP TSG-RAN4 Meeting #115	R4-2506365

# Malta, MT, 19th – 23rd, May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | - | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38.101-1 to Introduce R19 ATG enhancement UE RF requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG_enh-Core |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce R19 ATG enhancement UE RF requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Specify operating band and configurations for ATG with CA and UL MIMO. 2) Specify output RF spectrum emissions requirements for ATG with UL MIMO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | R19 ATG enhancement UE RF requirements would be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2J, 5.5, 6.5J |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change for TS 38.101-1>>

5.2J	Operating band for ATG

5.2J.1	General

NR operating bands n1, n3, n34, n39, n41, n78, n79, which are defined in Table 5.2-1, can be applied for ATG operation.

5.2J.1A	Operating band for ATG with CA

NR intra-band carrier aggregation CA_n79, which is defined in Table 5.2A.1-1, can be applied for ATG CA operation.

NR inter-band carrier aggregation CA_n3-n39, which is defined in Table 5.2A.2.1-1, can be applied for ATG CA operation.

5.2J.1D	Operating band for ATG with UL MIMO

NR operating bands n1, n3, n34, n39, n41, n78, n79, which are defined in Table 5.2D-1 to support UL MIMO, can be applied for ATG UL MIMO operation.

<<Next of Change>>

5.5J	Void

5.5J.1A	Configurations for ATG with CA

For NR ATG CA operation, the ATG CA channel bandwidths for intra-band contiguous are specified in clause 5.5J.1A.1.  The ATG CA channel bandwidths for inter-band contiguous are specified in clause 5.5J.1A.2.

5.5J.1A.1	Configurations for ATG intra-band contiguous CA

Table 5.5J.1A.1-1 NR ATG CA configurations and bandwidth combination set for intra-band contiguous CA in FR1

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations or single uplink carrier | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n79C | CA_n79C | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |
|  |  | See n79 channel bandwidths in Table 5.3.5-1 for each carrier |  |  |  |  | 200 | 4 and 5 |

5.5J.1A.2	Configurations for ATG inter-band CA

Table 5.5J.1A.2-1 NR ATG CA configurations and bandwidth combination set for inter-band CA in FR1

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n39A | n3 n39 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 35, 40 |  |

<<Next of Change>>

6.5J	Output RF spectrum emissions for ATG

6.5J.1	Occupied bandwidth for ATG

The requirements for occupied bandwidth in clause 6.5.1 apply. For ATG UE, the requirements for occupied bandwidth are defined at each transmit antenna connector or each TAB connector.

6.5J.1D	Occupied bandwidth for ATG with UL MIMO

The requirements for occupied bandwidth in clause 6.5D.1 apply. For ATG UE with UL MIMO, the single antenna port related requirement for TxD is preclude.

6.5J.2	Out of band emission for ATG

6.5J.2.1	General

This clause contains requirements for out of band emissions for ATG UE, the requirement defined in general part of clause 6.5.2.1 should apply.

6.5J.2.2	Spectrum emission mask

If the actual transmission power of ATG UE is less than or equal to 31dBm, the requirements for spectrum emission mask in clause 6.5.2.2 apply; if the actual transmission power of ATG UE is larger than 31dBm, the requirements of spectrum emission mask in clause 6.5.2.2 shall be relaxed with scaling factor equal to (the actual transmission power minus 31) dB. For ATG UE, the requirements for spectrum emission mask are defined as the sum of the emissions from all UE transmit antenna connectors or all TAB connectors.

NOTE:	This scaling factor is only applicable to ATG airborne UE.

6.5J.2.3	Adjacent channel leakage ratio

NR Adjacent Channel Leakage power Ratio (NRACLR) is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent NR channel frequency at nominal channel spacing. For ATG UE, the requirements for ACLR are defined as the sum of the emissions from all UE transmit antenna connectors or all TAB connectors.

The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.5J.2.3-1.

If the measured adjacent channel power is greater than –50 dBm then the NRACLR shall be higher than the value 30dBc.

Table 6.5J.2.3-1: NR ACLR measurement bandwidth

| Channel bandwidth | (MHz) | 5,10,15,20,25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 | 30 |
| NR ACLR measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |
| NOTE : “NRB” in the formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. |  |  |  |

6.5J.2D	Out of band emission for ATG with UL MIMO

The requirements for out of band emission in clause 6.5D.2 apply. For ATG UE with UL MIMO, the single antenna port related requirement for TxD is preclude.

6.5J.3	Spurious emissions for ATG

The requirements for spurious emission in general part of clause 6.5.3.0 and clause 6.5.3.1 apply. For ATG UE, the requirements for Spurious emissions are defined as the sum of the emissions from all UE transmit antenna connectors or all TAB connectors.

6.5J.3D	Spurious emissions for ATG with UL MIMO

The requirements for spurious emissions in clause 6.5D.3 apply. For ATG UE with UL MIMO, the single antenna port related requirement for TxD is preclude.

6.5J.4D	Transmit intermodulation for ATG with UL MIMO

The requirements for transmit intermodulation in clause 6.5D.4 apply. For ATG UE with UL MIMO, the single antenna port related requirement for TxD is preclude.

<<End of Change>>