3GPP TSG-RAN WG4 Meeting # 115                                                                R4-2506366

Malta, MT, 19th –23rd May, 2025

Agenda item:	5.5.1

Source: 	CMCC

Title: 	Discussion on R18 ATG UE RF requirement applicability

Document for:	Discussion

# 1. Introduction

ATG UE RF requirement applicability at antenna connectors were discussed during RAN4#114 meeting. The discussion on this issue has made some progress, but there is still some controversy over how to amend the specification. This paper will focus on these issues for further discussion.

# 2. Discussion

## 2.1. Tx requirement

During RAN4#114 meeting, we have reached the agreement as below.

Issue 1-1: Tx requirements definition for two types of ATG UE antenna Proposals from Apple, Keysight, CMCC: - All transmit signal quality requirements should be specified at the antenna connector(s) of the ATG UE with one or multiple omni-directional antennas(s) or at the transceiver array boundary (TAB) connectors of the ATG UE with the antenna array.. WF: - All transmit signal quality requirements should be specified at the antenna connector(s) of the ATG UE with one or multiple omni-directional antennas(s) - FFS for the ATG UE with the antenna array.

In the test specification TS 38.521, for UE supporting UL MIMO, the test procedures related to Transmit signal quality clearly specify that testing requirements should be specified at each layer. Moreover, each layer has explicitly corresponding connectors. Therefore, under the condition that R19 ATG UE supports UL MIMO, the relevant Transmit signal quality requirements could be specified at per layer.

In R18, however, since ATG UE does not support the UL MIMO capability in R18, all transmit signal quality requirements should be specified at the connector.

Proposal 1: Since ATG UE support the UL MIMO capability in R19, transmit signal quality requirements could be specified at per layer.

Proposal 2: For the R18 ATG UE, all transmit signal quality requirements should be specified at the connector.

# 3. Conclusions

In this contribution, ATG UE RF requirement applicability are discussed with the following proposals:

Proposal 1: Since ATG UE support the UL MIMO capability in R19, transmit signal quality requirements could be specified at per layer.

Proposal 2: For the R18 ATG UE, all transmit signal quality requirements should be specified at the connector.

# 4. References