3GPP TSG-RAN WG4 Meeting # 115                                                                R4-2506367

Malta, MT, 19th –23rd May, 2025

Agenda item:	7.12.2.1

Source: 	CMCC

Title: 	Discussion on UE RF requirements for ATG with inter-band CA

Document for:	Discussion

# 1. Introduction

RAN#106 meeting approved revised WID on Enhancements for Air-to-ground network for NR in Rel-19 [1]. After discussions of several meetings, we have made good progress on most of the ATG inter-band CA requirement, and we will focus on the n3 UL MSD requirements for CA_n3A-n39A.

# 2. Discussion

## 2.1. n3 UL for CA_n3A-n39A

During RAN4#114 meeting, we have reached the agreement as below.

Issue 2-3: Assumed parameters for CA_n3A-n39A with n3 UL WF: - Use the following parameters in the approved Tdoc R4-2410641 with 20dB antenna isolation as baseline to analysis the MSD value for n39 DL. - Other parameters are not precluded. Parameters; Value. Antenna isolation; [20dB]. Tx attenuation at 1880~1920MHz for band n3 duplexer; [30dB]. Rx attenuation at 1710~1785MHz for band n3+n39 Rx filter; [40dB]. Issue 2-4: MSD for CA_n3A-n39A with n3 UL WF: - Companies are encouraged to provide MSD values based on the following Tables. - Other MSD test configurations are not precluded. - All of these MSD values are only applicable to ATG UE. - Further study on how to capture these MSD values to in the specification. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n3; n39; [1722.5]; [25]; [15]; [160 (RBstart=0)]; [1900]; [30]; MSD Table Y; [>ACLR2]. Table Y CA_n3-n39 with band n3 UL MSD requirements for different output powers Output power; MSD (Based on Antenna Architecture with common antenna). 23dBm; TBD. 24dBm; TBD. …; …. 39dBm; TBD. 40 dBm; TBD.

Based on the parameters above, the MSD for different is show in the following table.

Table 5 The MSD calculation for CA_n3-n39 with n3 UL

|  | parameters | n39 main path | n39 diversity path |
| --- | --- | --- | --- |
| transmit power for n3, dBm |  | 23 | 23 |
| Tx band n3 BW(MHz, Lcrb@15kHz) | 25 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n3 uplink freq，dB | 0 |  |  |
| antenna isolation, dB | 20 |  |  |
| n39 received signal at ANT port, dBm |  | 23 | 3 |
| n39 filter rejection at 1710~1785MHz, dB | 40 |  |  |
| signal After n39 filter, dBm |  | -21 | -41 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -92 | -132 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -88 | -128 |
| noise figure dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -165 |  |  |
| Rx band n39 BW(MHz, NRB) | 28.8 |  |  |
| Thermal noise, dBm |  | -90.4 | -90.4 |
| n3 PA noise PSD at PA output port, dBm/Hz | -130 |  |  |
| n3 PA noise PSD at PA output port, dBm/RxBW |  | -55.4 |  |
| n3 Tx filter rejection at 1880~1920MHz, dB | 30 |  |  |
| n3 PA noise power at Rx n39 ant port , dBm |  | -85.4 | -105.4 |
| Total noise level at ANT port |  | -82.7 | -90.3 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(30MHz BW) |  | -83.7 | -91.3 |
| Implementation Margin, dB | 2.5 |  |  |
| combined REFSENS(30MHz BW), dBm | -89.5 |  |  |
| MSD | 2.4 |  |  |

Table 6 CA_n3-n39 with band n39 UL MSD test configuration

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n39 | [1722.5] | [25] | [15] | [160 (RBstart=0)] | [1900] | [30] | MSD Table Y | [>ACLR2] |

Table Y CA_n3-n39 with band n3 UL MSD requirements for different output powers

| Output power | MSD (Based on Antenna Architecture with common antenna) |
| --- | --- |
| 23dBm | 2.4 |
| 24dBm | 2.5 |
| 25dBm | 2.7 |
| 26dBm | 2.8 |
| 27dBm | 2.9 |
| 28dBm | 3 |
| 29dBm | 3 |
| 30dBm | 3.1 |
| 31dBm | 3.1 |
| 32dBm | 3.2 |
| 33dBm | 3.2 |
| 34dBm | 3.3 |
| 35dBm | 3.3 |
| 36dBm | 3.4 |
| 37dBm | 3.6 |
| 38dBm | 3.8 |
| 39dBm | 4.2 |
| 40 dBm | 4.7 |

# 3. Conclusions

In this contribution, ATG UE with inter band CA requirements are discussed with the following proposals:

Proposal 1: MSD values of ATG UE should be specified as follow.

Table 6 CA_n3-n39 with band n39 UL MSD test configuration

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n39 | [1722.5] | [25] | [15] | [160 (RBstart=0)] | [1900] | [30] | MSD Table Y | [>ACLR2] |

Table Y CA_n3-n39 with band n3 UL MSD requirements for different output powers

| Output power | MSD |
| --- | --- |
| 23dBm | 2.4 |
| 24dBm | 2.5 |
| 25dBm | 2.7 |
| 26dBm | 2.8 |
| 27dBm | 2.9 |
| 28dBm | 3 |
| 29dBm | 3 |
| 30dBm | 3.1 |
| 31dBm | 3.1 |
| 32dBm | 3.2 |
| 33dBm | 3.2 |
| 34dBm | 3.3 |
| 35dBm | 3.3 |
| 36dBm | 3.4 |
| 37dBm | 3.6 |
| 38dBm | 3.8 |
| 39dBm | 4.2 |
| 40 dBm | 4.7 |

# 4. References

[1] RP-242385, Revised WID on Enhancements for Air-to-ground network for NR