3GPP TSG-RAN WG4 Meeting # 115                                                                R4-2506369

Malta, MT, 19th –23rd May, 2025

Agenda item:	7.28.3.2

Source: 	CMCC

Title: 	Discussion on LP-WUS UE RF Rx requirements of REFSENS, ASCS and ACS

Document for:	Discussion

# 1. Introduction

Revised WID of LP-WUS is approved with following RF part objectives:

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

In this contribution, we focus on discuss of UE LP-WUS RF requirements including REFSENSE, ACS and ASCS.

- Discussion

During the last meeting, RAN4 reached the following agreement below.

Issue 2-1-1: SNR for OOK-based LP-WUS Agreements: - Companies are encouraged to provide the simulation results with the assumption of the RM coding of 8bit, 16bit and 32bit to align simulations in RAN4. It is targeted to conclude SNR value for envelope detection and OFDM sequence detection next RAN4 meeting. Issue 2-1-5: How to map two LP-WUR types with UE capability Agreement: - RAN4 define two LP-WUR types (Type 1 and Type 2), no need 1-to-1 mapping between waveform and LP-WUR type. - Requirement applicability should be clearly defined. - RAN4 need to further discuss how to design the RMC for each type, how to ensure the performance of each type properly. Issue 2-3-1: ASCS requirements for OOK-based receiver Agreement: - For Type1 UE requirements, which are derived based on the assumption that demodulation of OOK content of LP-WUS is targeted - 1 RB guard for ASCS and 0.5dB de-sens - For Type2 UE requirements, which are derived based on the assumption that demodulation of OFDM content of LP-WUS is targeted - 0 RB guard for ASCS and 0 dB de-sens - For Type 1 and Type2 UE are not limited to specific LP-WUS waveform. - FFS how to define test case and requirement applicability for Type 1 and Type 2 - FFS how type 1 and type 2 capability related to RAN1 UE feature - A UE can only support either Type 1 or Type 2 but not both for LP-WUR Issue 2-3-3: Channel bandwidth for ASCS Test case Agreements: - ASCS requirement should be applied to all supported UE CBW. Conformance test case and potential down-selection will be defined by RAN5. - For conformance testing, it is expected that not all CBW need to be tested. Issue 2-3-4: whether high in-band signal (both LP-WUS and NR) case is considered for alternative ASCS requirements Agreements: FFS For ASCS, whether additional test case that the wanted signal is 14dB higher than LP-WUS REFSENS, i.e., REFSENSLP_WUR + 14 dBm, should be considered or not. Issue 2-4-1: Baseline ACS requirements (same interference) and required guard RB for SET-1 (worse NF) LP-WUR WF: - Encourage companies to update the simulation analysis based on latest agreements. Target to conclude ACS requirements and test parameters next meeting. Issue 2-4-2: Whether adopt the same wanted power level for Set-2 LR? Agreement: - The same set of the interference signal level defined for both two types of the LP-WUS requirements. Issue 2-4-4: Case 1 ACS test parameter Agreements: Specify case 1 and case 2 ACS test parameters for Type 1 and Type 2. - Define interferer as BWinterferer of 5 MHz.

2.1 REFSENSE requirements

Considering that SNR is relevant to RF and demodulation, and that RAN1 has two options of OFDM signal and OOK signal, the required SNR for demodulation of OOK and OFDM signals could be different, so two sets of SNR values could be specified. Based on the simulation results, the OFDM receiver performs better than the OOK receiver. Since the case OOK-4 (M=4) has the worst performance, the target SNR should be chosen based on this case. According to discussions in RAN1, the length of RM coding is not fixed and can vary from 8 bits to 32 bits. Based on the agreement during last meeting, our SNR results based on different receiver type and RM coding are listed as below.

Table 1 SNR@1%BLER with 4bit ADC

| Modulation | RM coding | Receiver type | SNR@1% BLER |
| --- | --- | --- | --- |
| OOK-4 M=4 | 8bit | Envelope | -2.5dB |
|  |  | OFDM | -4.7dB |
|  | 16bit | Envelope | -5.3dB |
|  |  | OFDM | -7.4dB |
|  | 32bit | Envelope | -7.1dB |
|  |  | OFDM | -9.2dB |

Proposal 1: SNR results based on different receiver type and RM coding are listed as below.

2.1 ASCS and ACS

When defining ACS and ASCS for LP-WUR, two factors should be considered: the ASCS value on a dB scale and the required guard RB. If we only define the guard RB requirement without specifying an ASCS value, there would be no restriction on the in-band selectivity of LP-WUR. In extreme cases, even LP-WUR with the worst performance might pass the test, as there is only a guard RB limitation. Conversely, if we only define the ASCS value without any guard RB limitation, LP-WUR with the worst performance could also pass the test in extreme cases, as long as the guard RB is large enough.

Regarding the ASCS requirements, in the last meeting we reached an agreement on the performance for Type1 UE and Type2 UE. However, different companies have varying interpretations of the two UE types. Our understanding is that the rationale for defining two UE types is due to the different demodulation capabilities of the receivers corresponding to the two distinct waveforms. Based on this, we think it is necessary to define two separate sets of RF requirements. Additionally, from the perspective of UE capability reporting, the UE needs to report its own UE type. In terms of UE capability, the UE type should be more explicitly defined.

Proposal 2: The UE capabilities of Type 1 and Type 2 UE should be further clarified.

For ACS requirement, we can use the legacy requirement as the starting point, and further consider to relax the value with guard RB. Based on the current simulation assumptions, we conducted simulations of 3rd-order and 5th-order filters under the conditions of OOK-4 and M=4, and the results are shown in the figure below. The simulation results show that the number of guard RBs has a significant impact on the ACS value. Under the condition of 1 guard RB, the ACS value ranges from 22 to 24 dB. Considering the impact of 1 RB, and combining the above analysis, we recommend selecting 1 RB as the guard RB and setting the ACS value to [22-24 dB].

Proposal 3: Select 1 RB as the guard RB and setting the ACS value to [22-24 dB].

# 3. Conclusions

In this contribution, ASCS, ACS and REFSENSE are discussed with following observations and proposals.

Proposal 1: SNR results based on different receiver type and RM coding are listed as below.

Table 1 SNR@1%BLER with 4bit ADC

| Modulation | RM coding | Receiver type | SNR@1% BLER |
| --- | --- | --- | --- |
| OOK-4 M=4 | 8bit | Envelope | -2.5dB |
|  |  | OFDM | -4.7dB |
|  | 16bit | Envelope | -5.3dB |
|  |  | OFDM | -7.4dB |
|  | 32bit | Envelope | -7.1dB |
|  |  | OFDM | -9.2dB |

Proposal 2: The UE capabilities of Type 1 and Type 2 UE should be further clarified.

Proposal 3: Select 1 RB as the guard RB and setting the ACS value to [22-24 dB].

# 4. Reference

- R4-2505234, WF for [114bis][131] NR_LPWUS_UERF