3GPP TSG-RAN WG4 Meeting # 115                                    R4-2506372

Malta, MT, 19th –23rd May, 2025

Agenda item:	7.25.2

Source: 	CMCC

Title: 	Discussion on SBFD general part

Document for:	Discussion

# 1. Introduction

During the previous session, the WF on SBFD BS requirements was approved with multiple consensus points [2]. In this submission, we proceed with the discussion on the general aspects, building upon the WF that was agreed upon in the last meeting.

- Discussion

  - DL/UL subband configurations support for SBFD

In RAN4 114bis meeting, agreements are listed as below.

Issue 1-1-1: Declaration and flexibility for SBFD operation Agreement: - For FR1 LA SBFD capable BSs, which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested. - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested. - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing. - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains. Sub-topic 1-2: DL/UL subband configurations support for SBFD Issue 1-2-1: Restriction on the number of UL subband sizes supported Agreement: - For a specific channel BW supported for SBFD, the maximum number of specific UL subband sizes are allowed to be declared - Maximum 3 subband configurations allowed for FR1 WA BS - These 3 suband configurations shall cover all DUD and DU/UD ones supported by this BS. - One UL subband size for DU/UD cases, should be counted as 2 configurations, e.g., UD 20-80MHz and DU 80-20MHz should be counted as 2 configurations. - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, maximum 3 subband configurations allowed as FR1 WA BS; Below this threshold, the restriction on the number of UL subband sizes supported is not applicable. - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs. - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains. Issue 1-2-2: Restriction on the number of DL subband sizes supported Agreement: - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS - For FR1 MR BS SBFD-capable BSs, a power level threshold could be established: Above this threshold, this restriction is applicable as FR1 WA BS; Below this threshold, the restriction on the number of DL subband sizes supported is not applicable. - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs. - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains.

During the previous meeting, we engaged in an in-depth discussion regarding the configurations of DL and UL sub-band, and reached a consensus on several key points. In an effort to make a compromise, it was decided to impose restrictions on the number of UL/UL sub-band sizes supported by WA BS. However, this restriction was not limited to WA BS alone; some MR BS were also required to adhere to the same limitations. Despite this agreement, a crucial issue remains unresolved: how to delineate power levels to clearly define the scope of these restrictions.

From our perspective, this limitation should be specifically targeted at MR BS with higher transmission power. For the vast majority of MR BS, such restrictions are not only unnecessary but also impractical. It is important to consider that for SBFD in FR1, the maximum bandwidth is 100MHz. Given this, setting the parameter X to 33 would effectively impose limitations on most MR BS with larger bandwidths, which could hinder their performance and efficiency.

Taking into account the transmission power of MR BS in current networks, we propose setting the parameter X to 47. This value is more aligned with the operational characteristics of the majority of MR BS and would allow them to function optimally without being unduly restricted. By adopting this proposal, we can ensure a more balanced and practical approach to managing the configurations of DL and UL sub-band across different types of BS.

Proposal 1: Considering the transmission power of MR BS in current networks, use 47 dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs.

# 3. Conclusions

In this contribution, SBFD RF requirements are discussed with following observations and proposals.

Proposal 1: Considering the transmission power of MR BS in current networks, use 47 dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs.

# 4. Reference

[1] TR 38.858,

[2] R4-2504751, Way Forward for [114bis][306] NR_duplex_evo_General