3GPP TSG-RAN WG4 Meeting # 115                                    R4-2506373

Malta, MT, 19th –23rd, May, 2025

Agenda item:	7.25.3.2

Source: 	CMCC

Title: 	Discussion on existing Tx requirements for SBFD

Document for:	Discussion

# 1. Introduction

In TR 38.858, the impacts on SBFD requirements have been analyzed. RAN4 has identified which legacy requirements are still applicable, which legacy requirements are not applicable and which new requirements are needed.

In last meeting, WF on SBFD BS requirement is approved with several agreements.[1]

In this contribution, we continue the RF requirements discussion based on the output of study item phase.

- Discussion

2.1 Deployment scenarios for FR1 SBFD macro BS

Way forward - Encourage companies to provide more inputs on whether SBFD-SBFD co-ex case should be in Rel-19 scope. - If it is in the scope of Rel-19, companies are also encouraged to provide input on whether/how to derive the additional requirements. FFS aspects that may need to be taken into account include: - Expected emissions and blocking levels, which could be estimated by means of considering, e.g.: - Option 1: Potential expected coupling loss and total receiver desensitization (considering self-interference, co-channel interference and adjacent operator interference) considering relevant site deployment scenarios - Option 2: Co-existence simulation and potentially analysis of co-located scenarios - Expected spectrum allocation and sub-band sizes - Expected feasible filtering performances considering the above two aspects

Regarding the current discussion on the coexistence scenario of SBFD-SBFD, the main point of contention is whether to define the RF requirements for the coexistence scenario of SBFD-SBFD in the current version. Given the potential coexistence scenarios of SBFD-SBFD, we think it necessary to establish RF requirements. However, considering the current deployment status of this technology and the fact that SBFD can be further explored as a 6G day one future research topic, we also find Option 1 acceptable.

Proposal 1: For operators, additional requirements for SBFD-SBFD scenario should be defined in current or future WI.

In addition to the SBFD-SBFD scenario, we have not defined requirements for the SBFD co-located scenario in R19. However, from the perspective of future deployment, it is also very important to define RF requirements for the co-located scenario. This is because co-located deployments are becoming increasingly common in modern wireless networks, where multiple systems are installed in close proximity to each other. It can also provide guidance for network operators on how to design and deploy their systems in a way that maximizes performance and minimizes potential issues.

Proposal 2: RF requirements for co-located scenario should also be defined in future WI.

If we cannot determine the RF requirements for the SBFD-SBFD scenario in R19, we could consider clarifying in the current release which requirement need to be defined for the coexistence of SBFD-SBFD in the future, such as additional emission requirements, to facilitate the definition of requirements in future release.

Proposal 3: Consider which requirement need to be defined in future release for the coexistence of SBFD-SBFD in R19.

# 3. Conclusions

In this contribution, SBFD RF requirements are discussed with following observations and proposals.

Proposal 1: For operators, additional requirements for SBFD-SBFD scenario should be defined in current or future WI.

Proposal 2: RF requirements for co-located scenario should also be defined in future WI.

Proposal 3: Consider which requirement need to be defined in future release for the coexistence of SBFD-SBFD in R19.

# 4. Reference

[1] R4-2504752, Way Forward for [114bis][307] NR_duplex_evo_BSRF