3GPP TSG-RAN4 Meeting # 115	R4-2506380

Malta, May.19th – 23th, 2025

# Title: 	On 6Rx SRS antenna IL

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.1.2.3.4

# Document for:	Approval

# 1 Introduction

In last meeting, the agreements on SRS IL imbalance issue captured in the WF[1] are:

- Sub-topic 4-1: General considerations for SRS IL imbalance issue Issue 4-1-1: Whether to solve SRS IL imbalance issue in Rel-19 Way forward: - No further discussion on the issue in future meetings and focus on identification of possible SRS IL imbalance issue solutions. - NOTE: This does not mean that any solution for SRS IL imbalance issue is specified in Rel-19 Issue 4-1-2: SRS IL solution applicability to 2Rx/4Rx/6Rx/8Rx Agreement - FFS on the potential SRS IL solution applicability to 2Rx/4Rx/8Rx UEs in addition to 6Rx UEs Issue 4-1-3: SRS IL imbalance solution as optional feature Agreement - If any new requirements or procedures for SRS IL handling are defined in Rel-19, they will be defined as optional UE features - Sub-topic 4-2: SRS IL imbalance issue solutions Way forward - RAN4 to further discuss the candidate SRS IL imbalance solution framework based on agreements for issues 4-2-1 and 4-2-2. - Companies are encouraged to provide views on additional aspects including - SRS output power testing - Enhancements to PCmax equations to improve SRS IL compensation Issue 4-2-1: SRS IL imbalance UE self-compensation Agreement: - RAN4 assumptions on UE behavior for Rel-19 SRS IL handling: - For SRS transmissions Rel-19 UE is expected to perform self-compensation of SRS IL up to its configured maximum output power - FFS if any additional clarifications on behavior are needed - No RAN1/4 specification impacts (i.e. this behavior is aligned with existing specifications) Issue 4-2-2: SRS IL imbalance UE assistance and reporting Agreement: - The following two possible general solutions may be considered for further consideration - Solution 1: Introduce UE capability indication on whether UE performs SRS IL self-compensation - FFS on the detailed solution including clarification on self-compensation and whether static or dynamic indication will be needed - Solution 2: Extend Type 3 SRS PHR reporting to carriers with PUSCH subject to RAN1 confirmation on feasibility - Further check whether solution 2 is in the scope or not - Considerations of any solutions should be based on system level throughput evaluations, and implementation complexity

- For the issues on the reporting SRS antenna IL imbalance, it was already agreed to focus on identification of possible SRS IL imbalance issue solutions, instead to discuss whether or not to solve SRS IL imbalance issue in Rel-19. Although there were some concrete progress in the last RAN4 meeting, there are still some FFS issue which needs further discuss.

In this contribution, we continue discussing the SRS IL imbalance issues and reiterate some proposals in [2].

# 2	Discussion

In our understanding, current NW has no knowledge about the UE SRS IL imbalance, instead the NW can dynamically adjust the power control parameters based on network conditions and UE performance, this helps in managing the power imbalance and optimizing the overall system performance. The UE adjusts the transmission power of SRS based on the power control parameters provided by the base station.

Generally speaking, the SRS transmission power should be sufficient to overcome the path loss and channel impairments. Too weak SRS transmission power would lead to poor measurement quality, but excessive power can lead to interference with other UEs or even affect network performance. Hence, in our understanding, the UE should adjusts its power dynamically to manage imbalance and maintain proper signal balance. Meanwhile, if the SRS transmission power has an imbalance (up to >10dB for PC2 UE defined in RAN4 spec), the channel estimate will be biased. This leads the gNodeB to overestimate or underestimate the gain on specific antennas or spatial paths in the DL channel, causing errors in channel state information (CSI).

In current RAN1 TS38.213 spec, the SRS transmission power are depends on the configured maximum output power PCMAX,f,c limit, and some parameters such as p0 , α, IL and etc.

For the term of Pcmax,f,c, it is defined in the TS38.101-1, i.e. PCMAX_L,f,c ≤ PCMAX,f,c ≤ PCMAX_H,f,c; wherein the ∆TRxSRS parameter is only incorporated in the lower boundary, i.e. PCMAX_L,f,c. It should be noted that ∆TRxSRS is not exactly the SRS insertion loss, it is actually the delta SRS insertion loss values under the worse case where SRS powers are transmitted from the main antenna path and the antenna path other than main antenna who usually has longest RF path and includes more switchers. With more Rx branches introduced, the ∆TRxSRS requirements become larger and more serious.

In terms of the previous discussion on how to derive the ∆TRxSRS parameter for 4Rx/6Rx/8Rx, it can be found different companies assumed different example RF antenna switcher architectures and different switcher loss and RF cable loss, also different implementation margin for the final value. We believe ∆TRxSRS parameter could be better than the requirements defined in the RAN4 specification.

In addition, we believe UE has ability to do some power compensation to overcome the larger IL imbalance among all the Tx/Rx SRS antenna port, especially UE has more power to be transmitted or in the scenario where the SRS transmission power (PSRS) is below PCMAX,L, i.e. PSRS,PC ≤ PCMAX,L. The UL self-compensation  depends on the UE implementation and we think it is optional, rather than mandatory. This compensation is aimed at maximizing the transmission of SRS power through antennas other than the primary Tx antenna, ensuring that as much power as possible is utilized efficiently.

For example, when the SRS transmission power is below the UE configured maximum output power (i.e. PCMAX,L), then UE can do the power compensation to compensate the IL and also there exist +2dB power tolerance for the Pcmax. Even UE can re-calculate the PCMAX,L with the new ΔIL to replace ∆TRxSRS to achieve higher boundary of PCMAX,L:

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS,UE, P-MPRc) }

But of course, if the UE has already achieve the PCMAX,H, and there is no power headroom, then there are no room left for UE to perform the IL compensation. Once the UE performs this power compensation, the ∆TRxSRS between antennas will be significantly reduced, leading to a more balanced and optimized signal distribution. Once the NW requests such information, it is advantageous for UE to inform the Network (NW) of this compensation information as it facilitates more precise CSI estimation.

When UE performs self-compensation of SRS IL up to its configured maximum output power, we think it would not introduce any additional RAN4 performance since SRS is optional configured only in UL timing adjustment, although SRS can be used to obtain the CSI information and beamforming, and higher SRS power, less BLER, there is no performance requirements related to SRS.

Another way would be dynamic report although it is admittedly more complicated than the UE power compensation. It offers a wealth of real-time information that is invaluable in certain scenarios. This is particularly true when the RF routing for SRS changes over time, yet the RFFE/RF routing information remains unknown to the NW. UE can report actual SRS IL in dynamic reporting, providing crucial insights into the real-time performance of the system. Furthermore, the establishment of several thresholds for these actual SRS IL may be necessary to ensure that the NW can effectively monitor and manage the system's performance. Even the consideration of different capability classes based on the reported actual SRS IL could be explored taking into account the diverse capabilities and operational characteristics of individual UEs, which is to use the similar methodology of lower-MSD capability reporting.

In Rel-19, RAN1 and RAN2 are not involved in the WID, instead only RAN4 is included. In our understanding, the solutions discussed in RAN4 should little impact the RAN1/2 work as much as possible, which means the possible solution in RAN4 shall be in the scope of the existing RAN1/2’s framework.

Regarding the two solutions, obviously solution 2 is subjected to RAN1 confirmation on feasibility. It needs RAN1 additional work to discuss the feasibility to extend Type 3 SRS PHR reporting to carriers with PUSCH. For solution 1, we think there is no RAN1’s work, and it is business as usual for RAN2 if a new UE capability is introduced for a feature. LS can be sent to RAN2 for the necessary changes once there are concrete agreements in RAN4.

Moreover, Due to ∆TRxSRS requirements are defined for 2Rx, 4Rx, 6Rx and 8Rx, we think the problem would be similar for all of them although the problem is minor for 2Rx. Thus if ∆TRxSRS indication from UE to NW is optionally introduced for 6Rx, it can also apply to 2Rx/4Rx/8Rx case.

With the above consideration, we have the following proposals:

Proposal 1. UE SRS IL power compensation is up to UE implementation and optional, and if UE hasn’t reached its Tx power upper limit, UE should indicate the power compensation behaviour to NW once the NW requests such information.

Proposal 2: Introduce UE capability indication on whether UE performs SRS IL self-compensation, and it should be per SRS resource reporting.

Proposal 3. If dynamic reporting for actual SRS IL reporting for each SRS-TxSwitch pattern is considered, and several thresholds associated with capability class for the actual SRS IL reporting can be considered.

Proposal 4. The SRS IL imbalance reporting mechanism in 6Rx discussion should be also specified for 2Rx, 4Rx and 8Rx.

# 3 Conclusion

In this contribution, we give some further discussions on the SRS IL imbalance related to 6Rx. The conclusions are:

Proposal 1. UE SRS IL power compensation is up to UE implementation and optional, and if UE hasn’t reached its Tx power upper limit, UE should indicate the power compensation behaviour to NW once the NW requests such information.

Proposal 2: Introduce UE capability indication on whether UE performs SRS IL self-compensation, and it should be per SRS resource reporting.

Proposal 3. If dynamic reporting for actual SRS IL reporting for each SRS-TxSwitch pattern is considered, and several thresholds associated with capability class for the actual SRS IL reporting can be considered.

Proposal 4. The SRS IL imbalance reporting mechanism in 6Rx discussion should be also specified for 2Rx, 4Rx and 8Rx.

# 4	Reference

[1] R4-2505101, WF on 6Rx RF requirements and SRS IL imbalance, Intel

[2] R4-2503967, Views on 6Rx SRS antenna IL,, ZTE, Sanechips