3GPP TSG-RAN4 Meeting # 115	R4-2506384

Malta, May.19th – 23th, 2025

# Source: 	ZTE Corporation

# Title: 	On PC2 RedCap in FR1

# Agenda item:	7.6.2

# Document for:	Approval

# 1 Introduction

In RAN #106 meeting, the revised WID[1] of PC2 (e)RedCap FR1 to include PC2 FDD (e)RedCap was approved, in which n1 and n3 are selected as example bands. In the previous meeting, there are several agreements on FR1 PC2 TDD (e)RedCap, in which the PC2 TDD (e)RedCap UE was introduced as band agnostic way and PC2 RF requirements for non-RedCap UE are re-used. Consequently, there are less impacts on the specification. Moreover, the draft CR to introduce FR1 PC2 TDD (e)RedCap was endorsed.

For FR1 PC2 FDD (e)RedCap, the agreements are captured in the WF [2], which are:

Issue 1-1-1: REFSENS for 1Rx in FD-FDD Agreement: Consider the following formats for specifying REFSENS for 1Rx PC2 RedCap in FD-FDD - Option1: REFSENS (1Tx (26dBm) /2Rx) + ΔR1R : ΔR1R may be specified based on RSD ranges - Flat 3dB ΔR 1R for all CBW no larger than 20MHz or use values for FDD bands in table 7.3I.2-1 for bands whose PC2 1Tx/2RX RSD is 0dB. - Companies to evaluate REFSENS for other bands. - The format in which the REFSENS is shown in the specs is TBD pending evaluations. - Option2: REFSENS (1Tx (26dBm) /2Rx) + ΔR1R : ΔR1R may be specified based on the following conditions - ΔR 1R value may be considered from LTE TS 36.101 Table 7.3.1E-1A for bands and CBW which are already defined in TS 36.101 Table 7.3.1E-1A. The ΔR 1R value is calculated by comparing the REFSESN specified in TS 36.101 Table 7.3.1E-1A with TS 36.101 Table 7.3.1. - Companies are encouraged to bring measurement based evaluation for ΔR1R for other bands and CBW which are not covered by LTE TS 36.101 Table 7.3.1E-1A. - The format in which the REFSENS is shown in the specs is TBD pending evaluations. Way forward: It is encouraged for companies to provide analysis for option 1 and 2. Note: Companies are encouraged to evaluate all specified PC2 FDD bands. The WI is intended to be completed in May meeting according to WI plan.

In this contribution, we give some further discussion on PC2 FDD (e)RedCap.

# 2	Discussion

The PC2 RF requirements include: UE maximum output power, Tx power tolerance, MPR, A-MPR and RSD requirements if needed.

In terms of the previous discussion, the RF requirements agreements for PC2 1Tx FDD (e)RedCap can be summarized in the Table 1.

Table 1. RF requirements agreements for PC2 1Tx FDD (e)RedCap

|  | 1Tx HD-FDD | 1Tx FD-FDD |
| --- | --- | --- |
| Maximum output power Tx power tolerance | Reuse requirements of non-RedCap PC2 FDD band in clause 6.2.1 |  |
| MPR/A-MPR | 1 Tx MPR/AMPR requirements for FDD PC2 non-RedCap can be reused |  |
| REFSEN/RSD | For both 1Rx and 2Rx: Reuse REFSEN requirements for RedCap PC3 for HD-FDD in sub-clause 7.3I Note: No RSD for HD-FDD | 1Rx: FFS 2Rx: RSD is reused from 1Tx/2Rx non-Redcap UE PC2 FDD in Table 7.3.2-1c |
| Other requirements | No impact |  |

For PC2 HD-FDD (e)RedCap UE, there are no new RF requirements defined, and the existing requirements are reused. In other words, PC2 (e)RedCap requirements can be specified in the band agnostic for HD-FDD. The only additional work for 1Tx FDD PC2 (e)RedCap UEs is to develop RSD for 1Rx/1Tx FD-FDD case.

For the two options in the WF, the formats of REFSENS (1Tx (26dBm) /2Rx) + ΔR1R are used to specify the REFSENS for 1Rx PC2 RedCap in FD-FDD.

For option 2, ΔR1R is calculated by comparing the REFSESN specified in TS 36.101 Table 7.3.1E-1A with TS 36.101 Table 7.3.1 shown in table 1.

Table 1: ΔR1R for LTE category 0

| E-UTRA Band | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 2 | 2.5 | 2.5 | 2.5 | 2.5 |
| 3 | 2.5 | 2.5 | 2.5 | 2.5 |
| 4 | 2.5 | 2.5 | 2.5 | 2.5 |
| 5 | 2.5 | 2.5 |  |  |
| 8 | 2.5 | 2.5 |  |  |
| 13 | 3 | 3 |  |  |
| 20 | 2.5 | 2.5 | 3 | 3 |
| 25 | 2.5 | 2.5 | 2.5 | 2.5 |
| 26 | 2.5 | 2.5 | 2.5 |  |
| 28 | 2.5 | 2.5 | 2.5 | 2.5 |
| 39 | 2.5 | 2.5 | 2.5 | 2.5 |
| 40 | 2.5 | 2.5 | 2.5 | 2.5 |
| 41 | 2.5 | 2.5 | 2.5 | 2.5 |

Comparing to the ΔR1R for NR PC3 (e)RedCap UE, the latter one is more stringent for most of the bands, and we are discussing the NR PC2 RedCap, it is straightforward to use the NR approach (i.e. option 1). In addition to that, we think the supported bands for PC2 (e)RedCap UE should be the subset of the bands for PC2 RedCap UE, also we think RAN4 should try to develop a band agnostic way to define the REFSENS for 1Rx PC2 RedCap in FD-FDD. Therefor, we prefer to option 1.

As we discussed in [3], PC2 (e)RedCap 1Rx REFSEN = PC2 (e)RedCap 2Rx REFSEN - RSD + ΔR1R + RSD’, where RSD is existing reference sensitivity degradation for PC2 non-RedCap 2Rx, and RSD’ is reference sensitivity degradation for PC2 (e)RedCap 2Rx. In last meeting, we assume RSD=RSD’, then we get PC2 (e)RedCap 1Rx REFSEN = PC2 (e)RedCap 2Rx REFSEN + ΔR1R. However, as commented by some companies, the RSD may not equal to RSD’ due to diversity path has no contribute to the whole degradation and it seems the problem exists for PC2 non-RedCap 2Rx either. General speaking, the noise in diversity path is smaller than the noise in primary path, and it will reduce the RSD value after MRC calculation.

We use the similar method in [4] to analysis the sensitivity degradation for some bands. On top of that, we calculate the delta RSD (=RSD’-RSD) which can be seen as the additional value on top of the ΔR1R. The results can be found in the Annex part.

In order to develop a band agnostic method for all the bands, the format for REFSEN degradation for PC2 (e)RedCap 1Rx comparing with PC2 (e)RedCap 2Rx based on RSD ranges could be:

| RSD Range | REFSEN degradation for PC2 (e)RedCap 1Rx comparing with PC2 (e)RedCap 2Rx |
| --- | --- |
| 0 | ΔR1R |
| 0<RSD ≤ 0.5 | ΔR1R + [0.3] |
| 0.5<RSD ≤ 1.5 | ΔR1R + [0.8] |
| 1.5<RSD ≤ 3 | ΔR1R + [1.5] |
| NOTE: ΔR1R is defined in table 7.3I.2-1 |  |

Therefore, we have the following proposal:

Proposal 1: To develop a band agnostic method for all the bands, the format for REFSEN degradation for PC2 (e)RedCap 1Rx comparing with PC2 (e)RedCap 2Rx based on the RSD ranges could be:

| RSD Range | REFSEN degradation for PC2 (e)RedCap 1Rx comparing with PC2 (e)RedCap 2Rx |
| --- | --- |
| 0 | ΔR1R |
| 0<RSD ≤ 0.5 | ΔR1R + [0.3] |
| 0.5<RSD ≤ 1.5 | ΔR1R + [0.8] |
| 1.5<RSD ≤ 3 | ΔR1R + [1.5] |
| NOTE: ΔR1R is defined in table 7.3I.2-1 in TS38.101-1 |  |

# 3 Conclusion

In this contribution, we give some further discussion on PC2 FDD (e)RedCap, the conclusions are summarized below:

Proposal 1: To develop a band agnostic method for all the bands, the format for REFSEN degradation for PC2 (e)RedCap 1Rx comparing with PC2 (e)RedCap 2Rx based on the RSD ranges could be:

| RSD Range | REFSEN degradation for PC2 (e)RedCap 1Rx comparing with PC2 (e)RedCap 2Rx |
| --- | --- |
| 0 | ΔR1R |
| 0<RSD ≤ 0.5 | ΔR1R + [0.3] |
| 0.5<RSD ≤ 1.5 | ΔR1R + [0.8] |
| 1.5<RSD ≤ 3 | ΔR1R + [1.5] |
| NOTE: ΔR1R is defined in table 7.3I.2-1 in TS38.101-1 |  |

# 4	Reference

[1] RP-243267, Revised WID on NR power class 2 RedCap (Reduced Capability) UE in FR1, China Telecom

[2] R4-2505231, WF on NR PC2 RedCap UE in FDD bands, China Telecom

[3] R4-2503961, On PC2 RedCap in FR1, ZTE Corporation

[4] R4-2505231, On PC2 RedCap n1 and n3 1Rx REFSENS, Apple

# Annex:

| n2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5MHz |  | 10MHz |  | 15MHz |  | 20MHz |  |
|  | Main path | Div path | Main path | Div path | Main path | Div path | Main path | Div path |
| PC3 2Rx REFSENS (dBm) | -98 |  | -94.8 |  | -93 |  | -90.7 |  |
| Diversity gain | 3 |  | 3 |  | 3 |  | 3 |  |
| PC3 Thermal noise | -95 | -95 | -91.8 | -91.8 | -90 | -90 | -87.7 | -87.7 |
| PC3 Tx noise | -99.5 | -105.5 | -96 | -102 | -94 | -100 | -90 | -96 |
| Degradation | 1.3 | 0.4 | 1.4 | 0.4 | 1.5 | 0.4 | 2.0 | 0.6 |
| MRC MSD(i.e.PC2 1Tx 2Rx RSD defined in spec) | 0.8 |  | 0.9 |  | 0.9 |  | 1.2 |  |
| PC2 1Tx 1Rx RSD (dB) | 1.3 |  | 1.4 |  | 1.5 |  | 2.0 |  |
| delta RSD | 0.5 |  | 0.5 |  | 0.6 |  | 0.8 |  |
| n3 |  |  |  |  |  |  |  |  |
|  | 5MHz |  | 10MHz |  | 15MHz |  | 20MHz |  |
|  | Main path | Div path | Main path | Div path | Main path | Div path | Main path | Div path |
| PC3 2Rx REFSENS (dBm) | -97 |  | -93.8 |  | -92 |  | -90.8 |  |
| Diversity gain | 3 |  | 3 |  | 3 |  | 3 |  |
| PC3 Thermal noise | -94 | -94 | -90.8 | -90.8 | -89 | -89 | -87.8 | -87.8 |
| PC3 Tx noise | -101 | -107 | -97.5 | -103.5 | -96 | -102 | -94.5 | -100.5 |
| Degradation | 0.8 | 0.2 | 0.8 | 0.2 | 0.8 | 0.2 | 0.8 | 0.2 |
| MRC MSD(i.e.PC2 1Tx 2Rx RSD defined in spec) | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  |
| PC2 1Tx 1Rx RSD (dB) | 0.8 |  | 0.8 |  | 0.8 |  | 0.8 |  |
| delta RSD | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  |
| n7 |  |  |  |  |  |  |  |  |
|  | 5MHz |  | 10MHz |  | 15MHz |  | 20MHz |  |
|  | Main path | Div path | Main path | Div path | Main path | Div path | Main path | Div path |
| PC3 2Rx REFSENS (dBm) | -98 |  | -94.8 |  | -93 |  | -90.7 |  |
| Diversity gain | 3 |  | 3 |  | 3 |  | 3 |  |
| PC3 Thermal noise | -95 | -95 | -91.8 | -91.8 | -90 | -90 | -87.7 | -87.7 |
| PC3 Tx noise | -102 | -108 | -99 | -105 | -97 | -103 | -94.5 | -100.5 |
| Degradation | 0.8 | 0.2 | 0.8 | 0.2 | 0.8 | 0.2 | 0.8 | 0.2 |
| MRC MSD(i.e.PC2 1Tx 2Rx RSD defined in spec) | 0.5 |  | 0.5 |  | 0.5 |  | 0.5 |  |
| PC2 1Tx 1Rx RSD (dB) | 0.8 |  | 0.8 |  | 0.8 |  | 0.8 |  |
| delta RSD | 0.3 |  | 0.3 |  | 0.3 |  | 0.3 |  |
| n8 |  |  |  |  |  |  |  |  |
|  | 5MHz |  | 10MHz |  | 15MHz |  | 20MHz |  |
|  | Main path | Div path | Main path | Div path | Main path | Div path | Main path | Div path |
| PC3 2Rx REFSENS (dBm) | -97 |  | -93.8 |  | -91.4 |  | -85.8 |  |
| Diversity gain | 3 |  | 3 |  | 3 |  | 3 |  |
| PC3 Thermal noise | -94 | -94 | -90.8 | -90.8 | -88.4 | -88.4 | -82.8 | -82.8 |
| PC3 Tx noise | -101 | -107 | -96 | -102 | -93 | -99 | -81.5 | -87.5 |
| Degradation | 0.8 | 0.2 | 1.1 | 0.3 | 1.3 | 0.4 | 3.7 | 1.3 |
| MRC MSD(i.e.PC2 1Tx 2Rx RSD defined in spec) | 0.5 |  | 0.7 |  | 0.8 |  | 2.3 |  |
| PC2 1Tx 1Rx RSD (dB) | 0.8 |  | 1.1 |  | 1.3 |  | 3.7 |  |
| delta RSD | 0.3 |  | 0.4 |  | 0.5 |  | 1.4 |  |
| n13 |  |  |  |  |  |  |  |  |
|  | 5MHz |  | 10MHz |  |  |  |  |  |
|  | Main path | Div path | Main path | Div path |  |  |  |  |
| PC3 2Rx REFSENS (dBm) | -97 |  | -93.8 |  |  |  |  |  |
| Diversity gain | 3 |  | 3 |  |  |  |  |  |
| PC3 Thermal noise | -94 | -94 | -90.8 | -90.8 |  |  |  |  |
| PC3 Tx noise | -98.5 | -104.5 | -95 | -101 |  |  |  |  |
| Degradation | 1.3 | 0.4 | 1.4 | 0.4 |  |  |  |  |
| MRC MSD(i.e.PC2 1Tx 2Rx RSD defined in spec) | 0.8 |  | 0.9 |  |  |  |  |  |
| PC2 1Tx 1Rx RSD (dB) | 1.3 |  | 1.4 |  |  |  |  |  |
| delta RSD | 0.5 |  | 0.5 |  |  |  |  |  |
| n14 |  |  |  |  |  |  |  |  |
|  | 5MHz |  | 10MHz |  |  |  |  |  |
|  | Main path | Div path | Main path | Div path |  |  |  |  |
| PC3 2Rx REFSENS (dBm) | -97 |  | -93.8 |  |  |  |  |  |
| Diversity gain | 3 |  | 3 |  |  |  |  |  |
| PC3 Thermal noise | -94 | -94 | -90.8 | -90.8 |  |  |  |  |
| PC3 Tx noise | -100 | -106 | -95.5 | -101.5 |  |  |  |  |
| Degradation | 1.0 | 0.3 | 1.3 | 0.4 |  |  |  |  |
| MRC MSD(i.e.PC2 1Tx 2Rx RSD defined in spec) | 0.6 |  | 0.8 |  |  |  |  |  |
| PC2 1Tx 1Rx RSD (dB) | 1.0 |  | 1.3 |  |  |  |  |  |
| delta RSD | 0.4 |  | 0.5 |  |  |  |  |  |
| n25 |  |  |  |  |  |  |  |  |
|  | 5MHz |  | 10MHz |  | 15MHz |  | 20MHz |  |
|  | Main path | Div path | Main path | Div path | Main path | Div path | Main path | Div path |
| PC3 2Rx REFSENS (dBm) | -96.5 |  | -93.3 |  | -91.5 |  | -90.3 |  |
| Diversity gain | 3 |  | 3 |  | 3 |  | 3 |  |
| PC3 Thermal noise | -93.5 | -93.5 | -90.3 | -90.3 | -88.5 | -88.5 | -87.3 | -87.3 |
| PC3 Tx noise | -98 | -104 | -95 | -101 | -92.5 | -98.5 | -90.5 | -96.5 |
| Degradation | 1.3 | 0.4 | 1.3 | 0.4 | 1.5 | 0.4 | 1.7 | 0.5 |
| MRC MSD(i.e.PC2 1Tx 2Rx RSD defined in spec) | 0.8 |  | 0.8 |  | 0.9 |  | 1.1 |  |
| PC2 1Tx 1Rx RSD (dB) | 1.3 |  | 1.3 |  | 1.5 |  | 1.7 |  |
| delta RSD | 0.5 |  | 0.5 |  | 0.6 |  | 0.6 |  |
| n28 |  |  |  |  |  |  |  |  |
|  | 5MHz |  | 10MHz |  | 15MHz |  | 20MHz |  |
|  | Main path | Div path | Main path | Div path | Main path | Div path | Main path | Div path |
| PC3 2Rx REFSENS (dBm) | -98.5 |  | -95.5 |  | -93.5 |  | -90.8 |  |
| Diversity gain | 3 |  | 3 |  | 3 |  | 3 |  |
| PC3 Thermal noise | -95.5 | -95.5 | -92.5 | -92.5 | -90.5 | -90.5 | -87.8 | -87.8 |
| PC3 Tx noise | -101.5 | -107.5 | -97.5 | -103.5 | -95 | -101 | -90 | -96 |
| Degradation | 1.0 | 0.3 | 1.2 | 0.3 | 1.3 | 0.4 | 2.0 | 0.6 |
| MRC MSD(i.e.PC2 1Tx 2Rx RSD defined in spec) | 0.6 |  | 0.7 |  | 0.8 |  | 1.3 |  |
| PC2 1Tx 1Rx RSD (dB) | 1.0 |  | 1.2 |  | 1.3 |  | 2.0 |  |
| delta RSD | 0.4 |  | 0.5 |  | 0.5 |  | 0.8 |  |
| n85 |  |  |  |  |  |  |  |  |
|  | 5MHz |  | 10MHz |  | 15MHz |  |  |  |
|  | Main path | Div path | Main path | Div path | Main path | Div path |  |  |
| PC3 2Rx REFSENS (dBm) | -97 |  | -93.8 |  | -84 |  |  |  |
| Diversity gain | 3 |  | 3 |  | 3 |  |  |  |
| PC3 Thermal noise | -94 | -94 | -90.8 | -90.8 | -81 | -81 |  |  |
| PC3 Tx noise | -100 | -106 | -94.5 | -100.5 | -79 | -85 |  |  |
| Degradation | 1.0 | 0.3 | 1.5 | 0.4 | 4.1 | 1.5 |  |  |
| MRC MSD(i.e.PC2 1Tx 2Rx RSD defined in spec) | 0.6 |  | 1.0 |  | 2.6 |  |  |  |
| PC2 1Tx 1Rx RSD (dB) | 1.0 |  | 1.5 |  | 4.1 |  |  |  |
| delta RSD | 0.4 |  | 0.6 |  | 1.5 |  |  |  |