3GPP TSG-RAN4 Meeting # 115	R4-2506392

Malta, May.19th – 23th, 2025

# Title: 	Clarification on UL CA_n41C IMD MSD test point

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.3.3

# Document for:	Approval

# 1 Introduction

In the approved WF[1], the ULCA_n41C MSD test point and IMD range definition for UL CA_n40A-n41C and CA_n41C-n79A are approved, in which the second optional IMD test points with Full+Full RB UL configuration are introduced associated with the corresponding MSD values.

Since it is the first time for RAN4 to introduce the Full+Full RB UL configuration for a band combination, thus how to reflect/capture it are needed to be further discussed.

Some agreements are captured as below for the discussion convenience.

2.1 How to capture the second Full+Full test point Way Forward: The mandatory 1RB+1RB test point and Full+Full test points are captured in the current MSD table due to IMD in section 7.3A.5 of TS 38.101-1. - 2.3 Source of IMD definition Way Forward: Companies are encouraged to provide a clarification of the source of IMD for the test point where there is contiguous fully allocated RB in each CC based on the IMD order dominating the interference power. Companies are encouraged to provide their view on which document(s) should capture such clarification: - In the PRD as a rule and/or explanation to the TP to TR process. - In the related band combination TP to TR as an addtional information to the table calculating the IMD collisions for the mandatory test point. - In the section 7.3A.5 of TS 38.101-1 as a note or general text.

In last meeting, we give some initial thinking for the clarification of the source of IMD for the test point in [2]. Due to lack of time, there were no discussions on this topic. Thus in this contribution, we reiterate some proposals on how to capture the second Full+Full test point.

# 2	Discussion

Before RAN4 started to discuss the UL configurations of UL CA_n40A-n41C and CA_n41C-n79A, RAN4 only define one IMD test point with partial RB configurations for each CC (most of them are 1RB+1RB configurations) for IMD MSD source, no matter for FDD-TDD CA band configuration or TDD-TDD CA band configuration supporting non-simultaneous Rx/Tx operation. When UL CA_n40A-n41C and CA_n41C-n79A were started to discuss, operator think higher IMD MSD values(>42dB) caused by 1RB+1RB configuration are not acceptable, and 1RB+1RB configuration can not reflect the operator’s deployment in reality. Consequently, per operator’s demand, extensive discussions happen in the previous meeting.

Except for these two UL configurations of CA_n40A-n41C and CA_n41C-n79A, there are no other CA configurations falls into the similar discussion since there are no other operators ask RAN4 to reevaluate the IMD MSD requirements with other than 1RB+1RB test points configuration.

Thus we have the following observations for UL CA_nXA-nYC configuration:

Observation 1: Before CA_n40A-n41C and CA_n41C-n79A started to discuss, partial RB configurations for each CC (almost all of are 1RB+1RB configurations) for IMD MSD source are used.

Observation 2: CA_n40A-n41C and CA_n41C-n79A are the only two CA configurations with the second mandatory IMD test points with Full+Full RB UL configuration so far.

Observation 3: Except the CA_n40A-n41C and CA_n41C-n79A, there are no other operators ask RAN4 to reevaluate the IMD MSD requirements with other than 1RB+1RB test points.

With the above three observations, we think the second optional IMD test points with Full+Full RB UL configuration is not a common case. Otherwise, the second mandatory IMD test points with Full+Full RB UL configuration is needed to be added for all the UL CA configuration on top of 1RB+1RB configurations UL configurations. Obviously, the workload is huge and the impacts on RAN5 test in unknown, so we think it is impossible to do so.

Regarding which document(s) should capture such clarification, we think a note mark for these two configuration is needed in the section 7.3A.5 of TS 38.101-1. However, using general text is not recommended the clarification is not a general one.

To reflect the note in the TS 38.101-1, additional information in the related band combination TP to TR should be provided from rapporteur basket big CR procedure, like the approach captured in the WF as follow:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2397.5 | 41.7 | TDD | IMD314 |
|  | n41 | 2565 | 100 | 1 (RBSTART=0) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 1 (RBSTART=52) | 2644.8 | N/A | TDD | N/A |
|  | n40 | N/A | 5 | N/A | 2397.5 | 35.5 | TDD | IMD314,X |
|  | n41 | 2565 | 100 | 270 (RBSTART=3) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 162 (RBSTART=0) | 2644.8 | N/A | TDD | N/A |
| Note X: This second test point is optional. NOTE 14: Applicable when n41 spectrum is restricted to 2515-2675MHz. |  |  |  |  |  |  |  |  |

Also some other information such as background are also recommended in the TP to TR.

Regarding the PRD, it would be benefit to capture the clarification in the PRD as a rule/guidance. However, we think it should be clarified that it is not for all band combination, instead it is exceptional case for some UL CA configuration which relies on the operator request.

Considering the above, the proposals are:

Proposal 1. To include the clarification as a note in IMD table in the related band combination TP to TR and TS38.101-1.

Proposal 2. To clarify it as an exceptional case for some UL CA configuration which relies on the operator request in the PRD.

Note that a new NOTE of ‘NOTE 20: This second test point is optional.’ have already been added to reflect that the Full+Full RB UL configuration IMD MSD test point is optional in the endorsed draft CR [3] and TR 38719-02-01,

# 3 Conclusion

In this contribution, we give some discussions on how to capture the clarifications for the second IMD MSD test point, i.e. Full-Full RB configurations, for UL CA_n40A-n41C and CA_n41C-n79A.

The observations, proposals and conclusions are:

Observation 1: Before CA_n40A-n41C and CA_n41C-n79A started to discuss, partial RB configurations for each CC (almost all of are 1RB+1RB configurations) for IMD MSD source are used.

Observation 2: CA_n40A-n41C and CA_n41C-n79A are the only two CA configurations with the second mandatory IMD test points with Full+Full RB UL configuration so far.

Observation 3: Except the CA_n40A-n41C and CA_n41C-n79A, there are no other operators ask RAN4 to reevaluate the IMD MSD requirements with other than 1RB+1RB test points.

Proposal 1. To include the clarification as a note in IMD table in the related band combination TP to TR and TS38.101-1.

Proposal 2. To clarify it as an exceptional case for some UL CA configuration which relies on the operator request in the PRD.

# 4	Reference

[1] R4-2502955, WF on ULCA_n41C MSD test point and IMD range definition, Skyworks, Qualcomm, ZTE, Huawei

[2] R4-2503972,Clarification on ULCA_n41C IMD MSD test point, ZTE Corporation, Sanechips

[3] R4-2503733, draft big CR 38.101-1  for NR_CADC_R19_2BDL_xBUL, ZTE Corporation, Sanechips