3GPP TSG-RAN WG4 Meeting #115   		R4-2506418

# Malta, MT, 19th - 23rd May, 2025

Agenda Item:	7.17.3

Source: 	CMCC

Title: 			 Simulation results for 8Rx UE interference suppressing

Document for:	Discussion

- Introduction

Based on the simulation assumption approved in last meeting[1], we give our simulation results and observations.

- Simulation results

PDSCH requirements with inter cell interference

| HomoNet |  |  | SNR (dB) |
| --- | --- | --- | --- |
| FDD | 2T8R Rank 2 | MMSE-IRC baseline receiver | 8.41 |
|  |  | MMSE-IRC simplified receiver | 10.82 |
|  |  | Gain | 2.41 |
| TDD | 2T8R Rank 2 | MMSE-IRC baseline receiver | 8.99 |
|  |  | MMSE-IRC simplified receiver | 11.07 |
|  |  | Gain | 2.08 |

| HetNet |  |  | SNR (dB) |
| --- | --- | --- | --- |
| FDD | 4T8R Rank 4 | MMSE-IRC baseline receiver | 9.14 |
|  |  | MMSE-IRC simplified receiver | 13.42 |
|  |  | Gain | 4.28 |
| TDD | 4T8R Rank 4 | MMSE-IRC baseline receiver | 9.16 |
|  |  | MMSE-IRC simplified receiver | 13.48 |
|  |  | Gain | 4.32 |

- HomoNet：

- HetNet：

PDSCH requirements with intra cell inter user interference

|  |  |  | SNR(dB) |
| --- | --- | --- | --- |
| FDD | 4T8R Rank 2+2 | MMSE-IRC baseline receiver | 10.57 |
|  |  | MMSE-IRC simplified receiver | 13.75 |
|  |  | Gain | 3.18 |
| TDD | 4T8R Rank 2+2 | MMSE-IRC baseline receiver | 10.67 |
|  |  | MMSE-IRC simplified receiver | 13.86 |
|  |  | Gain | 3.19 |

- Simulation assumption

PDSCH requirements with inter cell interference

Simulation assumptions

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD, FDD |
| TDD pattern |  |  | 7D1S2U S=6D:4G:4U |
| SCS(kHz)/Bandwidth (MHz) |  |  | TDD: 30/40 FDD: 15/10 |
| MIMO layer and Antenna configuration |  |  | Rank 2: 2x8 ULA Low Rank 4: 4x8 ULA Low |
| Propagation condition and antenna correlation |  |  | TDLA30-10 |
| MCS |  |  | Rank 2: MCS 17 (Table 1) Rank 4: MCS 13 (Table 1) |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols (maxLength) for DL |  | Single-symbol |
| TRS configuration |  |  | Symbol#{5, 9} |
| Number of HARQ Processes |  |  | TDD: 8 FDD: 4 |
| HARQ ACK/NACK bundling |  |  | Multiplexed |
| Maximum HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Test metric |  |  | SNR@70% max TP |
| Receiver |  |  | Baseline SU-MIMO 8Rx receiver Simplified SU-MIMO 8Rx receiver |

| Physical cell ID | Physical cell ID of 0 for the serving cell, and cell ID 1 for interferer 1, cell ID 2 for interferer 2 |
| --- | --- |
| PDSCH allocation in frequency domain | In frequency domain: Full PRB In time domain: Full PDSCH allocation on all slots where serving cell has PDSCH grant |
| Precoding of interfering PDSCH | Random precoding with single panel type I codebook per slot and per PRB bundling granularity, with PRB bundling size of 2. |
| SSB configuration | All SSBs (serving cell and interference cell(s)) are in the same time/frequency resources |
| INR values for Homogeneous deployment | INRs 7.77 and 2.29 dB in case of 2 interference cells |
| INR values for Heterogeneous deployment | INRs 7.58 dB in case of 1 interference cells |
| Transmission rank of interfering PDSCH | random rank with 70% and 30% probability for rank 1 and rank 2 transmission in the interfering cell(s) |
| Modulation order of interfering PDSCH | 16QAM randomly modulated symbols |
| Timing and frequency offset | The serving cell is 3 us for interfering cell 1 and -1 us for interfering cell 2 The serving cell is 300 Hz for interfering cell 1 and -100 Hz for interfering cell 2 |
| Interference cell modeling | Interfering cell has 2Tx and random precoding with PRB granularity of 2. Interfering cell has same channel model as for serving cell and it is independently generated for each cell. |

PDSCH requirements with intra cell inter user interference

| Parameter |  | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | Target UE | Co-scheduled UE |  |
| Channel Bandwidth/SCS |  | 10MHz/15kHz for FDD and 40MHz/30kHz for TDD |  |  |
| Duplex mode |  | FDD and TDD |  |  |
| TDD pattern |  | 7D1S2U S=6D+4G+4U |  |  |
| MCS |  | Rank 2+2: MCS17 | 16 QAM random symbols |  |
| Allocation for interference UE and target UE | Rank allocation | 2 | 2 |  |
|  | DMRS port allocation | For rank 2+2: Port 1000 and 1001 | For rank 2+2: Port 1002 and 1003 |  |
| MIMO configuration |  | -Rank 2+2 for 4T8R ULA Low |  |  |
| Number of CDM groups without data |  | 2 for case with rank 2+2 |  |  |
| HARQ process number |  | 4 for FDD and 8 for TDD |  |  |
| Precoding model | Target UE | Random precoding with Single panel Type 1 4Tx: (N1, N2) = (2,1) | For case with rank 2+2 : Select the precoding matrix to ensure orthogonality with Target UE |  |
| PDSCH configuration | Mapping type | Type A |  |  |
|  | Starting symbol (S) | 2 |  |  |
|  | Length (L) | 12 |  |  |
|  | PRB bundling size | 2 |  |  |
|  | PRB bundling type | Static |  |  |
| PDSCH DMRS configuration | DMRS Type |  | DMRS Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
| Propagation conditions |  |  | TDLA30-10 |  |
| Receiver type |  |  | Baseline SU-MIMO 8Rx receiver Simplified SU-MIMO 8Rx receiver | N/A |
| Test metric |  |  | SNR @ %70 of maximum Throughput SNR = (STargetUE+ SInterfUE)/N | N/A |

- Reference

[1] R4-2504694, Way forward for [114bis][322] NR_demod_Ph5_Part2_UE, Moderator (Nokia)