3GPP TSG-RAN WG4 Meeting #115		R4-2506423

# Malta, MT, 19th - 23rd May, 2025

Agenda Item:	7.27.3.2

Source: 	CMCC

Title: 			  Discussion on RRM requirement of signal adaptation for NES enhancement

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed SSB signal adaptation related issues, agreements and open issues have been approved in the WF [1]. In this paper, we further discuss open issues, and identify RAN4 impacts according to the progress of other working group.

- Discussion

Issue 1-2-1: L1 requirements at transition

Agreement

  - RAN4 FFS how to define Tfirst_SSB considering the processing/preparation time based on further RAN1/2 progress on SSB adaptation command timeline.

  - No impacts on section 9.5.3 of L1 reporting requirements due to SSB adaptation, unless identified by other WGs.

Agreement from RAN1#120

For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell

Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI]

Agreement from RAN2#129bis

Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1

Based on the RAN1/2 progress, only DCI 2-9 will be used to indicate SSB periodicity adaptation. Therefore, Tfirst_SSB could further consider the DCI 2-9 processing time.

Proposal 1: DCI 2-9 processing time can be further considered in Tfirst_SSB.

Issue 1-3-1: L3 requirements for SSB adaptation

Agreement:

- RAN4 to define requirements based legacy L3 measurement framework for SSB adaptation.

    - For neighbour cell L3 measurement, legacy requirements apply.

Open issue for further discuss:

- For SCell with SSB adaptation (deactivated SCell or activated SCell), wait for more RAN2 progress on additional SMTC configuration, and discuss following aspects:

  - Whether to define L3 measurement requirement at transition.

Any other RRM requirements impacts due to additional SMTC configurations.

Agreement from RAN2#129

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

- For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

Agreement from RAN2#129bis

- For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration.

- Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

As designed by RAN2, the additional SMTC configuration is coupled with SSB burst periodicity. Upon reception of DCI 2-9, one SMTC configuration will be selected by UE according to the adapted SSB periodicity.

Then for intra-frequency measurement requirement, the delay requirement for SCell and intra-frequency neighbour cell shall be distinguished due to different SMTC may be used. A Note can be added that the accordingly adapted SMTC shall be applied for SCell upon the reception of DCI format 2-9 which indicates the SSB time-domain adaptation.

Proposal 2: For intra-frequency measurement requirement, add one note about SMTC that the accordingly adapted SMTC shall be applied for SCell upon the reception of DCI format 2-9 which indicates the SSB time-domain adaptation.

For transition requirement, the legacy framework can be reused for SCell measurement, that when SMTC configuration changes, the cell identification and measurement period requirements apply based on the longer delay before or after the transition.

Proposal 3: For SCell L3 measurement transition requirements, reuse the legacy transition framework, that when SMTC configuration changes, the cell identification and measurement period requirements apply based on the longer delay before or after the transition.

SCell activation requirement for SSB adaptation

Agreement:

For SCell activation:

- Define the SCell activation requirements based on the SSB periodicity after SSB adaptation.

  - For the case SSB adaptation command comes together with SCell activation command.

- The requirement can be updated based on further progress in other WG

Agreements from RAN1

For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell

- UE is configured with SSB burst periodicity using legacy signalling for the SCell

- UE is configured with X additional SSB burst periodicity for the SCell

  - FFS: Value of X

- SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity

  - FFS: Whether there is specification impact

  - Note: This does not impact the discussion on OD-SSB

- For switching the periodicity, down-select between

  - (MAC-CE)

    - MAC-CE details for SSB burst periodicity adaptation is up to RAN2.

  - (DCI)

    - Alt 1-3: DCI based signalling is used

Agreement

For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell

- Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI]

- FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1)

Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

Agreement from RAN2#129bis

Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1

Based on the input from RAN1 and RAN2, the SSB adaptation signal can be Group common DCI, therefore, it is hard to determine that the SSB adaptation command and SCell activation command come together or not. Therefore, we propose to refine the agreement as:

For SCell activation:

- Define the SCell activation requirements based on the SSB periodicity after SSB adaptation.

  - For the case SSB adaptation command comes not later than with SCell activation command

- Proposal 4: Refine the agreement from last meeting:

For SCell activation:

- Define the SCell activation requirements based on the SSB periodicity after SSB adaptation.

  - For the case SSB adaptation command comes not later than with SCell activation command

- Conclusion

Based on the discussion above, the following proposals are concluded.

Proposal 1: DCI 2-9 processing time can be further considered in Tfirst_SSB.

Proposal 2: For intra-frequency measurement requirement, add one note about SMTC that the accordingly adapted SMTC shall be applied for SCell upon the reception of DCI format 2-9 which indicates the SSB time-domain adaptation.

Proposal 3: For SCell L3 measurement transition requirements, reuse the legacy transition framework, that when SMTC configuration changes, the cell identification and measurement period requirements apply based on the longer delay before or after the transition.

- Proposal 4: Refine the agreement from last meeting:

For SCell activation:

- Define the SCell activation requirements based on the SSB periodicity after SSB adaptation.

  - For the case SSB adaptation command comes not later than with SCell activation command

- Reference

- R4-2504907, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei