3GPP TSG-RAN WG4 Meeting #115		R4-2506425

# Malta, MT, 19th - 23rd May, 2025

Agenda Item:	7.27.3.3

Source: 	CMCC

Title: 			  Discussion on RRM requirement of OD-SIB1 for NES enhancement

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed the paging requirements and timing requirements for OD-SIB1, the WF has been approved in [1]. In this contribution, we further discuss the paging interruption requirements.

- OD-SIB1 requirement

Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption

Way forward:

- Option A: Clarify that paging interruption includes UL WUS transmission and RAR reception for OD-SIB1 acquisition. (HW, OPPO, Apple, Nokia, Xiaomi, CATT, MTK, vivo)

- Option B: No impacts on core requirements. Discuss whether to extend TSI-NR in test case. (CMCC, QC, LGE)

- Option C: No RRM impacts on the requirements, i.e., UL-WUS can be postponed during the paing time. (E///, LGE, MTK)

- Agreement from RAN2#129bis

- Leave this issue (how to handle paging interruption during OD-SIB1 acquisition) to RAN4

In legacy, the paging interruption requirement is TSI-NR + 2*Ttarget_cell_SMTC_period ms, which includes the time period for fine time sync, fine frequency sync, and system information (SIB1 and SI messages) acquisition. We copy the definition in RAN4 spec for information:

TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell.

For the NES case, UE camps on the Cell A and try to re-select to NES cell based on cell re-selection criteria. After the sync is ready, UE will trigger the UL-WUS, verify the RAR in RAR window, acquire OD-SIB1 in OD-SIB1 window, and acquire other SI messages.

Firstly, we need try to figure out whether this OD-SIB1 procedure can be contained in current TSI-NR.

From definition point of view, we think the current definition can be reused, since the OD-SIB1 is also part of the relevant system information data reception procedure.

From the test case point of view, may be more investigation is needed about whether the value we used now can contain the UL-WUS and OD-SIB1 reception procedure. Our initial thinking is that UE can avoid PRACH (UL-WUS) transmission and paging reception collision by its implementation, and network can avoid the collision among RAR window, OD-SIB1 monitoring window and paging occasion by its configuration. Hence, no additional interruption time is needed. Anyway, this can be done in perf phase.

Proposal 1: Reuse the current paging interruption requirement and the definition of TSI-NR.

Proposal 2: No additional paging interruption time is needed for OD-SIB1 procedure, this can be reflected in Perf phase when defining the test requirement.

- Conclusion

Based on the discussion above, the following proposals are concluded.

Proposal 1: Reuse the current paging interruption requirement and the definition of TSI-NR.

Proposal 2: No additional paging interruption time is needed for OD-SIB1 procedure, this can be reflected in Perf phase when defining the test requirement.

- Reference

- R4-2504907, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei