3GPP TSG-RAN WG4 Meeting #115		R4-2506426

Malta, MT, 19th - 23rd May, 2025

Agenda Item:	7.27.4

Source: 	CMCC

Title: 			 Discussion on test cases for R19 NES

Document for:	Discussion

- Introduction

Rel-19 NES core requirements were discussed during RAN4#112 to RAN4#114bis. According to the TU allocation and work plan, RAN4 is supposed to start the performance part for the WI. In this contribution, we will provide our views on the test scopes for R19 NES.

- Discussion

  - Test scope for OD-SSB

OD-SSB based deactivated SCell measurement

The fast measurement in FMW is the new measurement procedure introduced for OD-SSB based measurement. The measurement delay requirement and the interruption from FMW requirement shall be verified by test cases.

Proposal 1: At least introduce OD-SSB based deactivated SCell measurement test case to verify the fast measurement requirement and interruption requirement due to FMW.

OD-SSB based SCell activation

For MAC-CE triggered SCell activation, the OD-SSB based activation procedure, delay requirements and interruption requirements shall be tested. Both known and unknown case shall be verified. We list some test configuration in detail:

| Sub Test 1 (unknown SCell): | No AO-SSB and OD-SSB before SCell activation command OD-SSB activation indication together with SCell activation command | Case#1 Scenario#2A |
| --- | --- | --- |
| Sub Test 2 (known SCell) | OD-SSB triggered before SCell activation command measCycleSCell=160ms | Case#1 Scenario#2 |
| Sub Test 3 (known SCell) | OD-SSB triggered before SCell activation command measCycleSCell=640ms | Case#1 Scenario#2 |
| Sub Test 4 (known SCell): | AO-SSB (NCD-SSB) and OD-SSB before SCell activation command OD-SSB activation indication together with SCell activation command measCycleSCell=160ms | Case#2 Scenario#2 |
| Sub Test 5 (known SCell) | AO-SSB (NCD-SSB) before SCell activation command OD-SSB activation indication together with SCell activation command measCycleSCell=640ms | Case#2 Scenario#2A |
| Sub Test 6 (known SCell) | AO-SSB (CD-SSB) before SCell activation command OD-SSB activation indication together with SCell activation command measCycleSCell=160ms | Case#2 Scenario#2A |
| Sub Test 7 (unknown SCell) | AO-SSB (NCD-SSB) before SCell activation command OD-SSB activation indication together with SCell activation command | Case#2 Scenario#2A |

Proposal 2: For MAC-CE triggered SCell activation, the OD-SSB based activation procedure, delay requirements and interruption requirements shall be tested, both known and unknown case, combination between Case#1, Case#2 and Scenario 2, Scenario 2A shall be covered in the test case. Detail configuration can refer to the table above.

For Direct SCell activation, both known and unknown case shall be covered. For known case, two cases, that OD-SSB carrier same and different with SSB measured in known condition shall be covered.

Proposal 3: For Direct SCell activation, the OD-SSB based activation procedure, delay requirements and interruption requirements shall be tested, both known and unknown case shall be covered. For known case, design two test cases that OD-SSB carrier same or different with SSB measured in known condition.

  - Test scope for OD-SIB1

The cell re-selection test case with UL-WUS for OD-SIB1 shall be introduced, the UL-WUS procedure and the paging interruption requirement will be verified together in cell re-selection test case.

Although the UL-WUS and OD-SIB1 reception procedure will be contained in TSI-NR, with the consideration that UE can avoid PRACH (UL-WUS) transmission and paging reception collision by its implementation, and network can avoid the collision among RAR window, OD-SIB1 monitoring window and paging occasion by its configuration, we think the legacy TSI-NR 1280ms requirement can be reused.

Proposal 4: Introduce cell re-selection test case with UL-WUS for OD-SIB1. For the embedded paging interruption requirements, the legacy TSI-NR = 1280ms can be reused.

  - Test scope for Signal adaptation

In high level, the test cases for L1 measurement requirements, L3 measurement requirements and SCell activation requirements when UE receives DCI 2-9 [no later than] SCell activation MAC-CE shall be introduced.

For L1 measurement, the L1-RSRP measurement serving cell testing, L1-SINR measurement testing and CBD for SCell testing shall be covered.

For L3 measurement, the intra-frequency measurement testing shall be covered.

Proposal 5: For SSB periodicity adaptation, at least introduce following testing

- L1-RSRP measurement serving cell testing

- L1-SINR measurement testing

- CBD for SCell testing

- Intra-frequency measurement testing, different test requirements before and after SSB adaptation

- SCell activation testing when UE receives DCI2-9 [no later than] SCell activation MAC-CE

- Conclusion

Based on the discussion above, the following proposals are concluded.

Proposal 1: At least introduce OD-SSB based deactivated SCell measurement test case to verify the fast measurement requirement and interruption requirement due to FMW.

Proposal 2: For MAC-CE triggered SCell activation, the OD-SSB based activation procedure, delay requirements and interruption requirements shall be tested, both known and unknown case, combination between Case#1, Case#2 and Scenario 2, Scenario 2A shall be covered in the test case. Detail configuration can refer to the table above.

Proposal 3: For Direct SCell activation, the OD-SSB based activation procedure, delay requirements and interruption requirements shall be tested, both known and unknown case shall be covered. For known case, design two test cases that OD-SSB carrier same or different with SSB measured in known condition.

Proposal 4: Introduce cell re-selection test case with UL-WUS for OD-SIB1. For the embedded paging interruption requirements, the legacy TSI-NR = 1280ms can be reused.

Proposal 5: For SSB periodicity adaptation, at least introduce following testing

- L1-RSRP measurement serving cell testing

- L1-SINR measurement testing

- CBD for SCell testing

- Intra-frequency measurement testing, different test requirements before and after SSB adaptation

- SCell activation testing when UE receives DCI2-9 [no later than] SCell activation MAC-CE