3GPP TSG-RAN WG4 Meeting #115			R4-2506433

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.31.5.2

Source: 	CMCC

Title: 			   Discussion on the RRM requirement for NTN phase3 DL CE

Document for:	Discussion

- Introduction

In last meeting, RAN4 achieved some agreements for DL coverage enhancement objective. The WF has been approved in [1]. In this contribution, we further discuss the open issues.

- Downlink coverage enhancements

## Issue 1-0: SMTC configuration and actual SSB transmission period

FFS: Wait for RAN2 progress and discuss the following

- Decide whether the following needs to be captured only in the RAN4 specification as part of the requirement applicability rule, or if the group will send an LS to RAN2 to ensure the following constraint is included in their specifications.

  - If different neighbour cells in the same frequency layer have different SSB periodicities, and if SMTC is not configured to match the largest SSB periodicity among the cells, the periodicity and active patterns of the neighbour cells shall be informed to the UE to assist with UE measurement.

- Further discuss the following if the decision needs to be made in RAN4 RRM:

  - For UEs supporting DL-CE feature in NR Rel-19, it shall be mandatory the support to 4 SMTCs per frequency layer

After introducing the default SSB periodicity larger than 20ms, different neighbour cells in the same frequency layer may have different SSB periodicities and/or different SSB offsets. It may not be feasible for network to configure one SMTC to match all SSB configurations among the neighbour cells. To address such issue, RAN2 introduce SMTC enhancement methodology for DL CE. The LS was sent to RAN4 at [2], we copy the content for information.

For downlink coverage enhancement, RAN2 assumes that it will be possible to have different SSB periodicities among neighbour cells in the same frequency layer, and for the case of intra-frequency neighbouring cells, the SSBs of surrounding neighbour cells may be transmitted at different times. Thus, RAN2 made the following agreements, given that at any time the UE will not use more SMTCs/Gaps in parallel than in previous releases: - RAN2 considers to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements. - RAN2 agrees to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs. RAN2 would like RAN4 to provide feedback on whether it is feasible to support these agreements in Rel-19 timeframe.

We think it is very important to support this feature in Rel-19, and RAN4 requirements shall be defined accordingly.

RAN2 mentioned that UE will not use more SMTCs/Gaps in parallel than in previous releases. In R17 and R18, UE mandatory support 2 parallel SMTCs and optionally support 4 parallel SMTCs based on its capability, and mandatory support 1 MG and optionally support 2 MG based on its capability. Although we think higher UE capability could support this enhancement better, considering only 2 meetings left, we are ok to use legacy UE capability as the starting point of R19 DL CE discussion.

Proposal 1: For the SMTC/MG capability, UE keep the same capability as in previous release.

After examining the current spec, we identify some requirements which may be impacted by SMTC configuration, we give a list below for quick view:

| Cell selection | M1 value |
| --- | --- |
| Cell re-selection requirement | M2/M3/M4 |
| Handover | Trs |
| RRC Re-establishment | TSMTC/TSMTC,i |
| RRC Connection Release with Redirection | Trs |
| Connected intra/inter-frequency measurement | Kmulti_SMTC K_satellite Kp Klayer1_measurement Kgap |

Then we look the related requirements in detail.

In RRC_IDLE and Inactive mode, maximum 6 SMTCs per frequency can be configured, and two different SSB periodicities with different offsets are supported among the configured SMTCs.

Cell selection

For DRX cycle based cell selection requirement, only scaling factor M1 will be impacted by SMTC number and periodicity.

M1 is the scaling factor to guarantee the UE power saving gain in short DRX cycle. In current spec,

-	M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and NSMTC =1, upon one SMTC configured at the UE,

-	M1=2.5 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and 1<NSMTC  4,

-	otherwise M1=1.

With NSMTC>4, the M1 value shall be further relaxed, we propose following:

-	M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and NSMTC =1, upon one SMTC configured at the UE,

-	M1=2.5 if at least one configured SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and 1<NSMTC  4,

-	M1=3 if at least one configured SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and 4<NSMTC  6,

-	otherwise M1=1.

Proposal 2: For cell selection requirement, M1 parameter will be impacted by the SMTC enhancement scheme introduced by RAN2, which should be updated as:

-	M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and NSMTC =1, upon one SMTC configured at the UE,

-	M1=2.5 if at least one configured SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and 1<NSMTC  4,

-	M1=3 if at least one configured SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and 4<NSMTC  6,

-	otherwise M1=1.

Cell re-selection

For DRX cycle based cell re-selection requirement, scaling factor M2/M3/M4 will be impacted by SMTC number and periodicity.

M2/M3/M4 is the scaling factor to guarantee UE power saving gain in longer SMTC period. In current spec,

For basic measurement

-	M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified.

For high-speed measurement

-	When SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2

We can see that the M2 value is related to SMTC periodicity per cell. Therefore, it can be reused with SMTC enhancement scheme. Such per cell level mapping shall be also used for the M2/M3/M4 factor in high-speed measurement.

Proposal 3: Update the Note of M2/M3/M4 in high-speed measurement requirements that if different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified.

Handover & RRC Connection Release with Redirection

For the Handover interruption and RRC Connection Release with Redirection delay requirements, the definition of Trs is as follows:

-	Trs is the SMTC periodicity of the target NR SAN cell if the UE has been provided with an SMTC configuration for the target cell in the handover/redirection command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. (The remaining unrelated parts are skipped)

However, in RRC Connected mode, RAN2 considers to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for neighbour cell measurement, which means when UE has not been provided with an SMTC configuration for target cell, there may be two SMTC periodicities in the MO which having the same SSB frequency and subcarrier spacing.

Therefore, the definition of Trs shall be updated. In the aforementioned case, we think the larger SMTC periodicity shall be used as default for target cell search.

Proposal 4: For handover interruption and RRC Connection Release with Redirection delay requirements, update the definition of Trs as:

-	Trs is the SMTC periodicity of the target NR SAN cell if the UE has been provided with an SMTC configuration for the target cell in the handover/redirection command, otherwise Trs is the maximum SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing. (Remaining unrelated parts are skipped)

RRC Re-establishment

For the RRC Re-establishment delay requirements, the definition of TSMTC/TSMTC,i is as follows:

-	TSMTC: It is the periodicity of the SMTC occasion configured for the intra-frequency carrier. If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2, Tsmtc follows smtc1 or smtc2 according to the physical cell ID of the target cell.

-	TSMTC,i: It is the periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms.

However, in RRC Connected mode, RAN2 considers to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for neighbour cell measurement, which means for intra-frequency carrier and inter-frequency carrier, there may be two SMTC periodicities from the SMTC configuration

Therefore, the definition of TSMTC/TSMTC,i shall be updated. In the aforementioned case, we think the larger SMTC periodicity shall be used as default for conducting measurement.

Proposal 5: For RRC Re-establishment delay requirements, update the definition of TSMTC/TSMTC,i as:

-	TSMTC: It is the maximum periodicity of the SMTC occasion configured for the intra-frequency carrier. If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2, Tsmtc follows smtc1 or smtc2 according to the physical cell ID of the target cell.

-	TSMTC,i: It is the maximum periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms.

Intra/inter frequency measurement

Regarding the scaling factor Kmulti_SMTC/K_satellite for measurement of multiple SMTCs or multiple satellites, currently, the calculation method doesn’t consider the multi SMTC periodicities scenario. For example, 2 SMTC with different periodicity configuration may not always be collided during a single Tidentify or TSSB_measure.

We think same methodology from RRC_IDLE mode can be reused, to add a note that for deriving Kmulti_SMTC /K_satellite for Tidentify and TSSB_measure, two SMTCs are considered as overlapping if they overlap in one or more occasions during a single Tidentify or TSSB_measure.

Proposal 6: For the the scaling factor Kmulti_SMTC/K_satellite for intra/inter frequency measurement, add a note that for deriving Kmulti_SMTC /K_satellite for Tidentify and TSSB_measure, two SMTCs are considered as overlapping if they overlap in one or more occasions during a single Tidentify or TSSB_measure.

Regarding the scaling factor Kp, Klayer1_measurement, and Kgap, we think the definition shall be updated to per target cell level. Existing method as in Table 9.2C.5.2-1 Note 1 can be reused, that add a note to give following further clarification: For calculation of Kp, Klayer1_measurement, and Kgap, if different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified.

Proposal 7: For the the scaling factor Kp, Klayer1_measurement, and Kgap for intra/inter frequency measurement, add a note that for calculation of Kp, Klayer1_measurement, and Kgap, if different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified.

## Issue 1-2-A: RRC Re-establishment (default TSMTC,i)

Agreement (online):

- In RRC re-establishment requirements, for case where the SMTC for the inter-frequency carrier is not provided, the value of TSMTC,i is assumed to be 160ms:

  - Discuss the applicability of the requirement for with the updated value of TSMTC,i to 160ms after the related RAN1/2 agreement.

FFS: Wait for RAN2 progress and discuss the following

- The requirements for intra-frequency and inter-frequency RRC re-establishment are applicable provided that the configured SMTC period is not shorter than the periodicity of the actual SSB transmission of the target cell.

Due to RAN2 has design the SMTC enhancement methodology, we think there is no need to further discuss the FFS above.

Currently, RAN1 and RAN2 hasn’t achieve any UE capability about DL CE feature. RAN4 could either sent a LS to ask the necessity of the UE capability, or postpone the issue and wait for more progress of RAN1/2.

Proposal 8: For the applicability of the requirement with the updated value of TSMTC,i to 160ms, either sent a LS to ask the necessity of the UE capability, or postpone the issue and wait for more progress of RAN1/2.

## Issue 1-5: Paging

Further discussion:

- For the paging interruption requirements for NGSO scenarios when the SSB periods is equal to 160 ms:

  - No update is needed, and K is 5.

To acquire the synchronization from target cell, K*Ttarget_cell_SMTC_period is needed. If the target cell is served by the same satellite as serving or GEO satellite or NR TN network, then K = 2, which is same as legacy. If the target cell is served by the known Non-GEO satellite, then K shall be extended to 5 with the consideration of more samples are needed for acquiring the sync with different doppler assumption. With above analysis, we can see that there is no relationship between K and SMTC period, and SMTC can be configured as 160ms even in earlier release.

However, when SMTC is 160ms, the sync time in paging interruption will reach 900ms, the motivation of further reducing it is reasonable. Therefore, We support to investigate some methods to reduce this paging interruption time, for example, in a new release, whether UE can support smaller K in such scenario based on its capability. Meanwhile, to be noted, such improvement is not necessary to be combined with SMTC periodicity.

Observation 1: The paging interruption time will be longer than 1s when SMTC is configured as 160ms. This observation also valid for earlier release.

Proposal 9: In new release, further investigate whether UE can support smaller K in known Non-GEO target satellite scenario based on its capability.

- Conclusion

Based on the discussion above, the following observations and proposals are concluded.

Proposal 1: For the SMTC/MG capability, UE keep the same capability as in previous release.

Proposal 2: For cell selection requirement, M1 parameter will be impacted by the SMTC enhancement scheme introduced by RAN2, which should be updated as:

-	M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and NSMTC =1, upon one SMTC configured at the UE,

-	M1=2.5 if at least one configured SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and 1<NSMTC  4,

-	M1=3 if at least one configured SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and 4<NSMTC  6,

-	otherwise M1=1.

Proposal 3: Update the Note of M2/M3/M4 in high-speed measurement requirements that if different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified.

Proposal 4: For handover interruption and RRC Connection Release with Redirection delay requirements, update the definition of Trs as:

-	Trs is the SMTC periodicity of the target NR SAN cell if the UE has been provided with an SMTC configuration for the target cell in the handover/redirection command, otherwise Trs is the maximum SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing. (Remaining unrelated parts are skipped)

Proposal 5: For RRC Re-establishment delay requirements, update the definition of TSMTC/TSMTC,i as:

-	TSMTC: It is the maximum periodicity of the SMTC occasion configured for the intra-frequency carrier. If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2, Tsmtc follows smtc1 or smtc2 according to the physical cell ID of the target cell.

-	TSMTC,i: It is the maximum periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms.

Proposal 6: For the the scaling factor Kmulti_SMTC/K_satellite for intra/inter frequency measurement, add a note that for deriving Kmulti_SMTC /K_satellite for Tidentify and TSSB_measure, two SMTCs are considered as overlapping if they overlap in one or more occasions during a single Tidentify or TSSB_measure.

Proposal 7: For the the scaling factor Kp, Klayer1_measurement, and Kgap for intra/inter frequency measurement, add a note that for calculation of Kp, Klayer1_measurement, and Kgap, if different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified.

Proposal 8: For the applicability of the requirement with the updated value of TSMTC,i to 160ms, either sent a LS to ask the necessity of the UE capability, or postpone the issue and wait for more progress of RAN1/2.

Observation 1: The paging interruption time will be longer than 1s when SMTC is configured as 160ms. This observation also valid for earlier release.

Proposal 9: In new release, further investigate whether UE can support smaller K in known Non-GEO target satellite scenario based on its capability.

- Reference

- R4-2504913, WF on RRM requirements for NR_NTN_Ph3_Part2, Moderator (Qualcomm Incorporated)

- R2-2503068, LS on SMTC enhancements, RAN2