3GPP TSG-RAN WG4 Meeting #115                                     	            R4-2506447

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:   	7.33.5

Source:   	LG Electronics

Title: 	Discussion on time/frequency pre-compensation for uplink transmission in NB-IoT NTN TDD mode

Document for:   	Discussion

- Introduction

In this contribution, we will provide our view on the behaviour of a UE performing time/frequency pre-compensation for UL transmission in NB-IoT NTN TDD mode.

- Discussion

The following question was asked for RAN4 to provide an answer in LS [1]:

RAN1 made the following working assumption on pre-compensation for NB-IoT NTN TDD mode: Working assumption For pre-compensation, from RAN1 perspective: - The UE adjusts its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes. - FFS: Whether it is supported to perform segmented pre-compensation within the set of 8 consecutive uplink subframes, and whether in this case a pre-compensation gap is needed. - FFS: whether spec impact is in RAN1, RAN4 or both. RAN1 respectfully asks RAN4 if there are any issues with the above working assumption.

According to the yellow marked part below in TS 36.102, a UE shall update pre-compensation at the beginning of each segment prior to segment transmission. Note that this requirement should be satisfied regardless of the starting time position of the segment.

6.4B Transmit signal quality for category NB1 and NB2 ### 6.4B.1 Frequency error for UE category NB1 and NB2 For UE category NB1 and NB2, the UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift based on received ephemeris information of the SAN in IE EphemerisInfo (TS 36.331 [6]), its own location and UL carrier frequency signalled to the UE by the SAN (according to TS36.300 [8] clause 23.21.2.2). The UE pre-compensated modulated carrier frequency shall be accurate to within the limits in Table 6.4B.1-1, observed over a period of one time slot (0.5 ms for 15 kHz sub-carrier spacing and 2 ms excluding the 2304Ts gap for 3.75 kHz sub-carrier spacing) and averaged over 72/LCtone slots (where LCtone = {1, 3, 6, 12} is the number of sub-carriers used for the transmission), compared to the ideally pre-compensated reference uplink carrier frequency. When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3]. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission. [NOTE: The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift corresponding to the estimated Doppler frequency at the beginning of the transmission.] Table 6.4B.1-1: Frequency error requirement for UE category NB1 and NB2 Carrier frequency [GHz]; Frequency error [ppm]. ≤1; ±0.2. >1; ±0.1.

The RAN1 WA described above provides a level of pre-compensation comparable to the existing operation of TS 36.102, with the additional advantage of reducing the number of UL segment gaps (e.g., when UL transmissions related to different segments are located within a UL burst). In this sense, we believe that there are no problems with the application of this newly introduced pre-compensation technique, especially from a performance perspective. Depending on the scenario, it may be good for the UE to perform pre-compensation multiple times within a UL burst (i.e., set of consecutive 8 UL subframes) in terms of the time/frequency offset estimation accuracy, but it will require a number of UL segment gaps that are not used for actual UL transmissions. To be specific, considering the frequency error variation of 0.27ppm/s and the carrier frequency of 1.6GHz, the additional expected frequency error (i.e., 3.456Hz) that can occur during the time period of the UL burst would be acceptable. Therefore, we do not see a strong technical motivation to support allowing segmented pre-compensation within the UL burst.

Proposal: RAN4 provides an answer to the question on RAN1 LS [1] as follows:

- Regarding the RAN1 WA on pre-compensation for NB-IoT NTN TDD mode, RAN4 has no concerns except that it does not see a strong technical motivation for introducing the behaviour of performing segmented pre-compensation within a set of 8 consecutive UL subframes.

- Conclusion

In this contribution, we discussed how RAN4 provides feedback on RAN1’s question in LS [1]. The following proposal is given.

Proposal: RAN4 provides an answer to the question on RAN1 LS [1] as follows:

- Regarding the RAN1 WA on pre-compensation for NB-IoT NTN TDD mode, RAN4 has no concerns except that it does not see a strong technical motivation for introducing the behaviour of performing segmented pre-compensation within a set of 8 consecutive UL subframes.

- Reference

[1] R4-2505311, “LS on precompensation for NB-IoT NTN TDD mode,” RAN1.