3GPP TSG RAN WG4 Meeting #115	R4-2506458

St Julian’s, Malta, 19th – 23rd May, 2025

Agenda item:			7.29.3.2

Source:	vivo

Title:	On RRM requirements for LTM CSI-RS based L1 measurement

Document for:	Discussion

# Introduction

In the RAN4#114bis meeting, RAN4 discussed RRM requirements for LTM CSI-RS based L1 measurement. Following agreements were made and captured in the WF [1].

#### Issue 4-1: Known cell definition in CSI-RS based L1 measurement Issue 4-1-1: Whether to use detectable or measurable? Agreement: - Confirm the RAN4#114 tentative agreement “Use measurable and specify the side condition for RS being measurable” Issue 4-1-2: Side condition for CSI-RS based L1 measurement for neighbor cell Agreement: - Use legacy side condition of SINR=-3dB. #### Issue 4-2: Measurement procedure #### Issue 4-2-1: Whether to keep or remove SSB based L1 measurement for FR2-1 in step 2 Agreement: - Whether step 2 can be skipped for neighboring cell or not based on new UE capability reporting. - Further discuss whether step 2 can be skipped for serving cell or not. #### Issue 4-2-2: How to interpretate the UE capability (for FR2-1) Agreement: - For a UE not supporting the capability, Step 2 is necessary. - The CSI-RS resources in Step 3 should be at least Type-D QCL’ed with the SSB in Step 2. - UE Rx beam sweeping factor is not included in the CSI-RS based L1-RSRP measurement period assuming the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’ - For a UE supporting the capability, Step 2 can be skipped, and if the Step 2 is skipped: - The CSI-RS resources in Step 3 should be Type-D QCL’ed with the SSB in Step 1. - The CSI-RS resources should be periodic or semi-persistent, and the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’ - UE Rx beam sweeping factor needs to be included in the CSI-RS based L1-RSRP measurement period, and further discuss the detail of beam sweeping factor. #### Issue 4-3: Definition of “inside/outside active BWP” Agreement: - RRM requirements (delay and accuracy) apply under the following condition: - At least 48 RBs of the CSI-RS configured for measurement is within the active BWP. #### Issue 4-5: Measurement on CSI-RS with repetition ON/OFF Agreement for FR2: - Only consider CSI-RS with repetition OFF in RAN4 requirements. #### Issue 4-6: measurement GAP and RTD assumption Agreement: - From RAN4 perspective, - Not to define RAN4 requirements for L1 measurement with gap for CSI-RS based L1 measurement on neighbor cell. - Not to define RAN4 requirements for RTD > CP. #### Issue 4-7: CSI-RS resource type Agreement: - Define RRM requirement for CSI-RS based L1 measurement requirement for periodic CSI-RS and semi-persistent CSI-RS. #### Issue 4-10: Definition of frequency layer Agreements: - No need to introduce definition of frequency layer. #### Issue 4-11: Whether to consider CSI-RS based L1-SINR measurement Agreement: - RAN4 doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement.

In this contribution, we further provide our views on RRM requirements for LTM CSI-RS based L1 measurement.

# Discussion

## 2.1 Measurement definition related

There has been long discussion on definition of intra-frequency/inter-frequency measurement for CSI-RS based L1 measurement. Following options were proposed in the last meeting.

#### Issue 4-4: CSI-RS based intra-frequency/inter-frequency measurement definition Candidate options: - Option 1: Apple, CMCC A measurement is defined as a CSI-RS based intra-frequency measurement provided that: - the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - the center frequency of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the center frequency of any of the CSI-RS resources within DL BWP (if configured). - Option 2: MediaTek, ZTE, Ericsson, Nokia, Qualcomm A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP. - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP. - Option 3: Huawei If L1 CSI-RS RSRP measurement with gap is not supported, either option of intra-frequency CSI-RS based L1 RSRP measurement definition has no big difference. If CSI-RS measurement with gap is supported, to define measurement requirements, the “CSI-RS frequency layer” concept will be touched. Then the intra-frequency definition based on center frequency is more preferred. - Option 4: vivo - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and - the CSI-RS resources of both the neighbour cell and serving cell configured for L1 measurement is included within the active DL BWP. - at least 48 RBs of the CSI-RS resource are confined within the active DL BWP.

When specifying definition of intra-frequency/inter-frequency, some general principles would be taken into account. For intra-frequency measurement, one aspect is that neighbour cell and serving cell can be measured simultaneously, and another aspect is that measurement gap is not supposed to be needed if the measurement resources are in the active BWP. For L3 measurement, the serving cell SSB is used as reference to define intra-frequency and inter-frequency measurement. It is better that similar principle is used for consistency of the definition. It means a reference signal of serving cell for L1 measurement is needed if it is intra-frequency measurement. It depends on the relationship between CSI-RS resource and active BWP whether gap is needed or not. So, it is also an important factor to be considered in the definition of intra-frequency measurement.

With above analysis, we would like to propose option 4 which is based on option 2 with additional changes.

Proposal 1: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:
- the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

- the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

- the CSI-RS resources of both the neighbour cell and serving cell configured for L1 measurement is included within the active DL BWP.

- at least 48 RBs of the CSI-RS resource are confined within the active DL BWP.

## 2.2 Requirements impact

Early CSI acquisition on a candidate cell was introduced for CSI-RS based LTM. There was discussion on whether to define requirements for this procedure.

#### Issue 4-12: Early CSI acquisition on a candidate cell Candidate options: - Option 1: Qualcomm - For the baseline CSI acquisition (DL CSI acquisition after LTM CSC MAC-CE reception), RAN4 to not define requirements. - For the optional CSI acquisition (DL CSI acquisition before LTM CSC MAC-CE reception), RAN4 to discuss whether and what requirements to define after the following aspects for CSI-RS based L1-RSRP measurement from the candidate cell are concluded: - Pre-requisite conditions, maximum RTD value, gap-less vs. gap-based, etc. - Option 2: Nokia - Discuss what is the point in time during the cell switch delay UE starts CSI-acquisition.

The details of early CSI acquisition are as follows based on RAN1 agreements.

For target cell CSI acquisition, - A UE is provided with RRC configurations for periodic CSI-RS resource(s) and CSI report(s) for one or more candidate cell - As baseline, CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE. - Introduce UE capability for CSI-RS measurement can start before reception of LTM CSC MAC CE - UE determines the CSI report configuration based on the CSC - After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration - The latest available measured CSI on target cell resource(s) is conveyed at least by a single report, and the report is sent to the target cell - In addition to periodic CSI-RS resource, semi-persistent CSI-RS resource is supported - Support of semi-persistent CSI-RS resource is subject to separate UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 - A single CSI report configuration is configured - Multiple CSI-RS resources for CMR can be associated with the CSI report configuration - The number of CSI-RS resources for CMR is subject to UE capability For the solution to avoid blind detection for LTM CSI report if valid CSI is not available, - Option 1: CSI is reported when valid CSI is not available Option 1-1: one CQI codepoint is used to indicate that valid CSI is not available Option 1-1-1: Codepoint with the lowest CQI index is reported

LTM cell switch delay is defined as:

DLTM = Tcmd + TLTM-interrupt

TLTM-interrupt = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms,

CSI measurement can be performed after UE obtains downlink synchronization. In legacy LTM cell switch, UE will wait for first RACH occasion for RACH-based cell switch or wait for first uplink transmission occasion for RACH-less cell switch. UE can perform CSI measurement during and after this uncertain waiting time depending on the CSI-RS resource availability.

Since it is different from legacy CSI acquisition after LTM switch, RRM requirements for early CSI acquisition need to be defined. A component of TCSI_reporting can be added to the existing LTM cell switch delay additionally, e.g.,

DLTM = Tcmd + TLTM-interrupt + TCSI_reporting

Where, TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331

For periodic CSI-RS, it should be enough with adding the TCSI_reporting to the cell switch delay. For semi-persistent CSI-RS, UE may need to wait for MAC-CE for activation of the semi-persistent CSI-RS if it is not sent to UE together with cell switch command. To simplify the requirements, it can be assumed two MAC-CE are sent in the same MAC PDU.

Proposal 2: RAN4 to define RRM requirements for target cell early CSI-acquisition.

Proposal 3: A component of TCSI_reporting is added to LTM cell switch delay if the target cell early CSI-acquisition is configured.

Proposal 4: TCSI_reporting is defined as the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331.

## 2.4 Conditions

#### Issue 4-8: Restriction on CSI-RS resource configuration Candidate options: - Option 1: Ericsson - In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction, all CSI-RS resources within a 40 ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5 ms.

It is similar to the restriction of CSI-RS configuration for L3 CSI-RS based mobility. Since the CSI-RS measurement will cause interruptions, it would be better to constrain the interruption to some fixed location. Option 1 is fine in general. In addition, it is better to further clarify the separation of the two windows if configured.

Proposal 5: When CSI-RS based L1 measurements cause scheduling restriction, all CSI-RS resources on one intra-frequency layer are configured within up to two separate windows where each window is up to 5 ms and the two windows, if configured, are separated by 20ms.

# Summary

In this contribution, we further provided our views on RRM requirements for LTM CSI-RS based L1 measurement. Following proposals and observations are present.

Proposal 1: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:
- the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

- the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

- the CSI-RS resources of both the neighbour cell and serving cell configured for L1 measurement is included within the active DL BWP.

- at least 48 RBs of the CSI-RS resource are confined within the active DL BWP.

Proposal 2: RAN4 to define RRM requirements for target cell early CSI-acquisition.

Proposal 3: A component of TCSI_reporting is added to LTM cell switch delay if the target cell early CSI-acquisition is configured.

Proposal 4: TCSI_reporting is defined as the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331.

Proposal 5: When CSI-RS based L1 measurements cause scheduling restriction, all CSI-RS resources on one intra-frequency layer are configured within up to two separate windows where each window is up to 5 ms and the two windows, if configured, are separated by 20ms.

- References

- R4-2502601	WF on RRM requirements for NR_Mob_Ph4_Part1, Apple