3GPP TSG RAN WG4 Meeting#115                                    R4- 2506466

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.21.3

Source:	vivo

Title:	Discussion on CSI reporting requirement and testing framework for CSI prediction

Document for:	Discussion

- Introduction

In R18 SI, we have studied on general aspects, specific issues related to use cases, and interoperability and testability aspects. The agreements have been collected in TR [1]. In RAN#105 meeting, CSI prediction has been agreed for normative work and then the requirement of CSI prediction needs to be defined in this release [2].

- CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

In previous meetings, we have the following agreements for CSI prediction.

Issue 1-1: Codebook to be used in the tests Agreement: - In Rel-19 only PMI requirements based on AI/ML CSI prediction will be defined - use Rel-18 eType II Doppler codebook for AI/ML based CSI predictions Issue 1-2: Requirement baseline Agreement: - Requirement to be introduced for Dopler domain basis N4=1 - N4>1 can be further discussed in the future if needed Issue 1-4: Test metric Requirements to be defined based on the ratio (same metric as for the current PMI reporting requirements in 38.101-4) - tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/ML [a given SNR] using the precoders configured according to the UE reports, - trnd is the throughput measured at SNRue_AI/ML [the same SNR] with random precoding. - trnd is obtained with Rel-15 type I single panel codebook - Note: SNRue_AI/ML is calculated in the same way as SNRue and SNRfollow1,follow2 in section 6.3 of 38.101-4

In this contribution, we further provide our views on CSI prediction.

- Discussion on CSI prediction

In the previous meetings, RAN4 have agreed to use relative throughput as requirement metric for CSI prediction, and companies are invited to bring contribution on baseline throughput.

In last meeting, the following simulation assumptions have been agreed for further study.

Table 1: Updated simulation assumptions for AIML based CSI prediction Parameter; Value. Duplex, Waveform; FDD, TDD, OFDM. Carrier frequency; 2GHz for FDD 4GHz for TDD. Subcarrier spacing; 15kHz for FDD, 30kHz for TDD. Bandwidth; 10MHz for FDD, 40MHz for TDD. Symbol; 1 symbol. Number of subbands; 13 subbands, 4RB as a bundle for FDD 14 subbands, 8RB as a bundle for TDD. gNB TX antennas; Option1:32(N1,N2)=(4,4) as defined in Rel-15 type1 Option2:16(N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements. UE RX antennas; 4. Channel model; TDLA30 with XP medium correlation as baseline. Doppler spread; 20Hz, 50Hz and 100Hz. Channel estimation for CSI prediction; Step-1: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Step-2: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., to check SGCS after CSI feedback). Step-3: - Practical DL channel estimation, use a common CSI-RS configuration as in demodulation test (may need to define/choose a CSI-RS configuration). CSI-RS configuration; FFS: following CSI-RS configuration: - For 32Tx Number of CSI-RS ports (X); 32. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0, l1); (5, 12). - For 16Tx Number of CSI-RS ports (X); 16. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0); (5).. Rank per UE; Step-1: Rank 1 and 2 to do quick check Step-2 and Step-3: Rank 2 as baseline. CSI feedback assumption; Step1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account. Step-2 and Step-3: take CSI feedback into account - Use Rel-18 eTypeII-Doppler, N4 = 1， N4>1 can be further discussed in the future if needed. Test setup(timeline); Only for step-3 FFS: following test setup (timeline): For FDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 4 slots Note: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4) - Test setup for FDD test setup with Periodic CSI-RS configuration is captured in Table2 For TDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 6 slots Note : If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). - Test setup for TDD test setup with Periodic CSI-RS configuration is captured in Table3 FFS test setup for aperiodic CSI-RS configuration. SNR assumption; Only for step-3 [FFS, e.g., Option1: claimed by companies for step-2 first round simulation(TP as KPI) Option2: Example: [-6:2:10] dB Option3: other suggestions ]. CSI feedback overhead; PC7 as baseline. Baseline; FFS: - Random PMI with Rel-15 Type I single panel codebook if needed (e.g., for Step-2 or performance requirements). KPI; Step-1: check SGCS before CSI feedback (between the raw channel matrix and ground-truth at UE) - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), - comparing the SVD of model output with SVD of the ground truth Raw channel Step-2: - check SGCS after CSI feedback Step-3: - check TPs after CSI feedback. Model input type; Raw channel matrix. Model out type; Raw channel matrix. Observation window; Observation window (number/distance): 5/5ms as baseline Optional for 10/5ms. Prediction window; Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms. Training data and test data; Use following combinations: Training dataset (TDLA30); Test on (TDLA30). 20Hz Doppler spreads; 20Hz Doppler spreads. 50Hz Doppler spreads; 50Hz Doppler spreads. 100Hz Doppler spreads; 100Hz Doppler spreads. Mixed Doppler spreads (20Hz, 50Hz, 100Hz); 20Hz Doppler spreads. 50Hz Doppler spreads. 100Hz Doppler spreads..

Based on the above simulation assumptions, the updated evaluation results for FDD and 32gNB Tx are provided in the following table. Note that for model trained by 20Hz, 50Hz and 100Hz Doppler spread, all samples of different Doppler spreads are used. Then, it is seen that mixed training may improve the SGCS performance in some cases.

Table 2-1. Initial evaluation results for CSI prediction AI/ML model, for FDD and 32gNB Tx.

| FDD, 32 gNB Tx | Test on 20Hz Doppler spread | Test on 50Hz Doppler spread | Test on 100Hz Doppler spread |
| --- | --- | --- | --- |
| Model trained by 20Hz Doppler spread | 0.993 | / | / |
| Model trained by 50Hz Doppler spread | / | 0.902 | / |
| Model trained by 100Hz Doppler spread | / | / | 0.298 |
| Model trained by 20Hz, 50Hz and 100Hz Doppler spread | 0.994 | 0.907 | 0.296 |

Observation 1: Initial evaluation results for CSI prediction AI/ML model is shown in Table 2-1, for FDD and 32gNB Tx.

During the simulation, there would be so many simulation assumptions which are not aligned but have large impact on the performance, such as the input RB number, input Rx number of the AI model. Also, different model structures and model parameters would have large impact on the performance. Then, if it is difficult to align the results corresponding to test metric, reference AI model for CSI prediction would be needed.

Proposal 1: Reference AI model for CSI prediction would be needed for test metric, if it is difficult to align the simulation results.

In the study of R18 and R19, AI CSI prediction is supposed to achieve some performance gain compared to non-AI CSI prediction, and then it may be not suitable for AI CSI prediction to directly reuse the the test setup and simulation assumptions of non-AI CSI prediction. RAN4 could work on the test setup and simulation assumptions of AI CSI prediction, by the future evaluations. For this one-sided use case, reference model should be defined to align results from companies, and then the requirements could be derived based on the aligned results.

Proposal 2: AI prediction and non-AI prediction could follow the similar test procedure.

In 2024 Q1 to Q3, it is SI of AI prediction and RAN1 focused on the simulation of AI prediction, to obtain acceptable performance gain compared to non-AI prediction. AI prediction has been agreed for WI in 2024 Q3. In 2024 Q4, RAN1 has been limited to the study on consistency of training/inference. Then, in fact, RAN1 has just started the work of spec impact of AI prediction. Some RAN1 LCM agreements have been achieved.

Agreement For CSI prediction using UE-side model, for performance monitoring, support UE assisted performance monitoring subject to an additional UE capability, and UE assisted performance monitoring is based on Type 3 performance monitoring Agreement For CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric. Agreement For the definition of SGCS, Note: How to handle layer mapping mismatch, if any, is up to UE implementation.

Currently, RAN1 has agreed that UE assisted performance monitoring is based on Type 3 performance monitoring. RAN4 to discuss Type 3 performance monitoring for UE assisted performance monitoring of CSI prediction. Since LCM aspects are still under discussion in RAN1, other LCM requirements should wait for RAN1 further progress. If there is any spec impact, RAN4 to discuss whether to capture it in the RRM spec.

Proposal 3: RAN4 to discuss Type 3 performance monitoring for UE assisted performance monitoring of CSI prediction. Other LCM requirements could wait for RAN1 further progress. If there is any spec impact, RAN4 to discuss whether to capture it in the RRM spec.

- Summary

Based on above analysis, following proposals and observations are present.

Observation 1: Initial evaluation results for CSI prediction AI/ML model is shown in Table 2-1, for FDD and 32gNB Tx.

Table 2-1. Initial evaluation results for CSI prediction AI/ML model, for FDD and 32gNB Tx.

| FDD, 32 gNB Tx | Test on 20Hz Doppler spread | Test on 50Hz Doppler spread | Test on 100Hz Doppler spread |
| --- | --- | --- | --- |
| Model trained by 20Hz Doppler spread | 0.993 | / | / |
| Model trained by 50Hz Doppler spread | / | 0.902 | / |
| Model trained by 100Hz Doppler spread | / | / | 0.298 |
| Model trained by 20Hz, 50Hz and 100Hz Doppler spread | 0.994 | 0.907 | 0.296 |

Proposal 1: Reference AI model for CSI prediction would be needed for test metric, if it is difficult to align the simulation results.

Proposal 2: AI prediction and non-AI prediction could follow the similar test procedure.

Proposal 3: RAN4 to discuss Type 3 performance monitoring for UE assisted performance monitoring of CSI prediction. Other LCM requirements could wait for RAN1 further progress. If there is any spec impact, RAN4 to discuss whether to capture it in the RRM spec.

- References

- 3GPP TR38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”, V18.0.0, 2023-12.

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#105, Sep 9-12, 2024.